3GPP TSG RAN WG4 Meeting #104-e	R4-2211752

# Electronic Meeting, 15 - 26 Aug, 2022

Agenda item:	11.11.2 (NonCol_intraB_ENDC_NR_CA)

Source:	SoftBank Corp.,

Title:	Discussion on the power imbalance requirement for intra-band non-contiguous CA with non-collocated scenario

Document for:	Approval

1.  Introduction

In the RAN#95-e meeting, the new WI for intra-band non-collocated EN-DC/NR-CA deployment was approved [1] and the revised WID for clarifying the objective was approved in RAN#96 [2]. In this contribution, we discuss the power imbalance requirements for intra-band non-contiguous NR-CA considering those for EN-DC already specified.

2.  Discussion

For reference, the power imbalance requirements for DC_42_n77/78 is exerted as follows [3]:

#### 7.6B.2.6 Power imbalance for type 2 UE of inter-band EN-DC with overlapping DL bands Power imbalance requirement only for band combination of type 2 UE which is indicated by capability interBandMRDC-WithOverlapDL-Bands-r16 is a measure of a receiver’s ability to receive LTE and NR signals with 25dB power imbalance. For these test parameters in table 7.5B.6-1, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). Table 7.6B.2.6-1: Test parameters for FDD-FDD or TDD-TDD inter-band EN-DC operation with overlapping or partially overlapping DL bands Test configurations; Carriers; Rx Power in transmission bandwidth configuration (dBm); channel bandwidth; Frequency relationship (Center of BWanother Relative to edge of BWwanted). 1; Wanted carrier; REFSENS + 1; BWwanted ≤ BWanother; < max (5/2* BWanother, 50MHz). Another carrier with overlapping DL bands; Power of wanted carrier + 25. 2; Wanted carrier; REFSENS + 1; BWwanted > BWanother. Another carrier with overlapping DL bands; Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother). 3; Wanted carrier; REFSENS + 1; NA; ≥ max (5/2* BWanother, 50MHz). Another carrier with overlapping DL bands; Power of wanted carrier + 25. NOTE 1: For NR carrier, the transmitter shall be set to 24dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 [2] with PCMAX_L,f,c,NR as defined in clause 6.2B.4. NOTE 2: For E-UTRA carrier, the transmitter shall be set to 24 dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2 [4] with PCMAX_L_E-UTRA,c as defined in clause 6.2B.4 for single carrier. NOTE 3: BWwanted is the channel bandwidth of wanted carrier. BWanother is the channel bandwidth of another carrier with overlapping DL bands. The applicability of these test configurations is shown below for type 2 UE which is indicated by capability interBandMRDC-WithOverlapDL-Bands-r16: When the capability interBandContiguousMRDC is indicated, test configuration 1 or 2 can be used to verify the RX power imbalance requirement for type 2 UE. When the capability interBandContiguousMRDC is absent, test configuration 1, 2 or 3 can be used to verify the RX power imbalance requirement for type 2 UE. It’s allowed to use one of test configurations to verify the RX power imbalance requirement for type 2 UE. For type 2 UEs indicating interBandMRDC-WithOverlapDL-Bands-r16 for the following EN-DC band combinations in Table 7.6B.2.6-2, the Rx requirements for four Rx ports do not apply for each band. Table 7.6B.2.6-2: EN-DC combinations EN-DC combination. DC_42_n77. DC_42_n78.

In Rel-15 discussion, we have already concluded the possible UE architecture for DC_42_n77/78 would be that for intra-band non-contiguous CA with common LNA and PA due to the frequency range of n77/78 contains that of B42 [4]. And the shared filtering for both B42 and n77/78, such as intra-band non-contiguous CA, was assumed when the power imbalance requirements for DC_42_n77/78 was discussed [5].

Observation: The possible UE architecture for DC_42_n77/78 would be that for n77/78 intra-band non-contiguous CA.

Considering the above, the power imbalance requirements for n77/78 intra-band non-contiguous NR-CA should be the same of that for DC_42_n77/78. One thing is that the possible UE architecture for those band combinations might be updated from Rel-15 discussion. Our proposal for the power imbalance requirements for n77/78 intra-band non-contiguous NR-CA are as follows.

Proposal 1: The power imbalance requirements specified for DC_42_n77/78 are the baseline of those for n77/78 intra-band non-contiguous CA.

Proposal 2: If there are some updates of possible UE architecture, it should be clarified, and their impacts on the power imbalance requirements should be reflected.

3.  Conclusion

In this contribution, we share our thoughts for the power imbalance requirements for intra-band non-contiguous NR-CA considering those for EN-DC already specified as follows.

Observation: The possible UE architecture for DC_42_n77/78 would be that for n77/78 intra-band non-contiguous CA.

Proposal 1: The power imbalance requirements specified for DC_42_n77/78 are the baseline of those for n77/78 intra-band non-contiguous CA.

Proposal 2: If there are some updates of possible UE architecture, it should be clarified, and their impacts on the power imbalance requirements should be reflected.

Reference

RP-221012		New WI: Support of intra-band non-collocated EN-DC/NR-CA deployment, KDDI

RP-221809		Revised WI: Support of intra-band non-collocated EN-DC/NR-CA deployment, KDDI

- [3]	TS38.101-3 v17.6.0

- [4]	R4-1709390		Feasibility of DC_42A-n77A and 42A-n78A, NTT DOCOMO, INC.

- [5]	R4-2111461		Imbalance Requirement for Type 2 UE RX, Qualcomm Incorporated