3GPP TSG-RAN WG4 Meeting # 104-e		R4-2211782

# Electronic Meeting, August 15 – August 26, 2022

Source:	China Telecom

Title:	On BS PUSCH demodulation requirements for NR coverage enhancements

Agenda Item:	9.16.2.1

Document for:	Discussion

- Introduction

In the RAN4 #103-e meeting, the BS PUSCH demodulation requirements for NR coverage enhancements were discussed, and agreements on most of the test parameters were made in the WF in [1].

In this paper, we provide our views on the remaining issues for TBoMS and PUSCH JCE.

- Discussion

Test requirement discussion scope for FR2

Status in the WF [1]:

  - Option 1: Limit the discussion scope of Rel-17 NR coverage enhancement demodulation to FR1 and FR2-1

  - Option 2: Do not limit the scope in this WI

  - Option 3: Following the decision in the RF core part

It has been agreed in the RF core part that the test scope for DMRS bundling requirements is limited to FR1 and FR2-1 in the RAN4 LS to RAN1 in RF session (in R4-2211225):

4) Non-support for FR2-2 RAN4 has also agreed not to extend the DMRS bundling requirements to FR2-2 in Rel-17.

To be aligned with the RF core part requirements, it is proposed that the BS demodulation requirements for Rel-17 coverage enhancements are limited to FR1 and FR2-1.

Proposal 1: The BS demodulation requirements for Rel-17 coverage enhancements are limited to FR1 and FR2-1, to be aligned with the RF core part requirements.

TDD UL-DL pattern and test applicability for BS requirements for PUSCH TBoMS

Status in the WF [1]:

  - For 15/60/120kHz SCS, use 3D1S1U, S=10D:2G:2U.

  - Keep the existing test applicability rule for different TDD UL-DL patterns as baseline and interested companies can bring TBoMS simulation results between different TDD patterns.

  - FFS whether Manufacturer declaration can be introduced for TBoMS

We are supportive to define manufacturer declaration for TBoMS since it is optional for UE and BS to support. At the same time, once supported, TBoMS can be performed regardless of the supported TDD pattern, thus the supportive of TBoMS should be independent on the supported TDD pattern for each SCS. Therefore, we propose the manufacturer declaration should be simply whether TBoMS is supported like below:

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| D.yyy | TB over Multi-slots (TBoMS) | Declaration of TB over Multi-slots support |

Observation 1: Once supported, TBoMS can be performed regardless of the supported TDD pattern, thus the supportive of TBoMS should be independent on the supported TDD pattern for each SCS.

Proposal 2: Define manufacturer declaration for TBoMS like below:

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| D.yyy | TB over Multi-slots (TBoMS) | Declaration of TB over Multi-slots support |

TDD UL-DL pattern for BS PUSCH demod requirements with JCE

Status in the WF [1]:

- Candidate options for new TDD patterns:

  - For FR1 15KHz SCS

    - Option 1A: DSUUU

    - Option 1B: 7D1S2U, S=6D:4G:4U

    - Option 1C: DDSUU, S=10G:2G:2U

  - For FR2 60/120 kHz SCS:

    - Option 1A: DSUUU

    - Option 1B: 5D1S4U

    - Option 1C: DDSUU, S=10G:2G:2U

- Proposals for manufacture declaration for support of JCE:

  - Proposal 1:

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| D.yyy | SCS for PUSCH JCE and PUCCH JCE | Declaration of supported SCS for PUSCH JCE and PUCCH JCE, i.e. {15kHz, 30kHz, 60kHz 120kHz} |

- Proposals on JCE test applicability rule for different TDD patterns for each SCS:

  - Proposal 1: the requirement defined for each SCS can be applicable for other TDD patterns with same number of physical consecutive slots (aTDW length)

  - Proposal 2: The same requirements are applicable to TDD with different UL-DL patterns, as long as those patterns allow for the mandated aTDW

- Agreement in the second round:

  - Make decision on the exact TDD pattern for 15/60/120kHz in the next meeting.

  - Use the below TDD patterns for simulation purpose only, and the same requirement can be reused if other patterns with same number of UL consecutive slot number is agreed:

    - 7D1S2U, S=6D:4G:4U for 15kHz

    - DDSUU, S=10G:2G:2U for 60/120kHz

  - Further discuss the test applicability rule for different TDD patterns for each SCS.

  - Further discuss the details for manufacture declaration for PUSCH JCE.

For the TDD pattern for PUSCH JCE for 15/60/120kHz, we propose to use 7D1S2U, S=6D:4G:4U for 15kHz and DDSUU, S=10G:2G:2U for 60/120kHz for requirement definition to avoid duplicate simulation work.

Proposal 3: Use 7D1S2U, S=6D:4G:4U for 15kHz and DDSUU, S=10G:2G:2U for 60/120kHz for requirement definition.

Since the cTDW and the aTDW lengths of 2 consecutive slots have been agreed for TDD, the JCE requirements for 7D1S2U or DDSUU should also be applicable for other TDD patterns with even number of UL consecutive slots, e.g., 5D1S4U, since JCE is still performed per 2 UL slots in that case. As for TDD patterns with odd number of UL consecutive slots, e.g., DSUUU, the requirements for 7D1S2U or DDSUU can be still applicable by disabling the UL transmission on the first or the last UL slot.

Observation 2: JCE requirements for 7D1S2U or DDSUU should also be applicable for other TDD patterns with even number of UL consecutive slots since JCE is still performed per 2 UL slots.

Observation 3: As for TDD patterns with odd number of UL consecutive slots, the requirements for 7D1S2U or DDSUU can be still applicable by disabling the UL transmission on the first or the last UL slot.

Considering other TDD patterns may be supported by the BS, we propose to define applicability rules for different TDD patterns for each SCS to have better coverage of the JCE requirements as below:

Proposal 4: Define test applicability rule for PUSCH JCE:

  - For each supported SCS, the requirements are also applicable for other TDD patterns with more than 1 physical UL consecutive slots.

  - Note that for the TDD pattern with odd number of UL consecutive slots, UL transmission on the first or the last UL slot is disabled.

We support to define manufacture declaration for PUSCH JCE. Since it is depending on the supported TDD pattern for each SCS ono whether PUSCH JCE can be supported, we agree that the manufacture declaration for PUSCH JCE to be defined for each SCS.

Proposal 5: Define following manufacture declaration for PUSCH JCE:

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| D.yyy | SCS for PUSCH JCE and PUCCH JCE | Declaration of supported SCS for PUSCH JCE and PUCCH JCE, i.e. {15kHz, 30kHz, 60kHz 120kHz} |

PRB number for BS PUSCH demod requirements with JCE

Status in the WF [1]:

  - Cover the minimum bandwidth for each SCS

  - FFS on whether to cover the largest CHBW for each SCS and interested companies can bring simulation results for the largest CHBW

We already have had the test applicability rule for different CHBW that the same requirements can be reused for other CHBW. In this contribution we provide simulation results on whether the same requirements can be reused between the smallest and the largest CHBW.

Table I. Ideal simulation results for PUSCH JCE for 10MHz and 100MHz CHBW (PUSCH Mapping type A, TDD 30kHz SCS)

| Antenna Configuration | CHBW | DMRS | Ideal results |
| --- | --- | --- | --- |
| 1T4R | 10MHz | DMRS 1+0 | -7.9 |
|  |  | DMRS 1+1 | -8.7 |
|  | 100MHz | DMRS 1+0 | -8.1 |
|  |  | DMRS 1+1 | -8.8 |
| 1T8R | 10MHz | DMRS 1+0 | -11 |
|  |  | DMRS 1+1 | -11.8 |
|  | 100MHz | DMRS 1+0 | -10.9 |
|  |  | DMRS 1+1 | -11.8 |

It is observed that similar SNR points can be achieved for PUSCH JCE under the smallest and the largest CHBW. It is expected that the same SNR requirements can also be applicable for other CHBW. Therefore, taking the simulation results and also the test coverage point of view, we support to cover the smallest and the largest CHBW for each SCS for PUSCH JCE requirement definition.

Proposal 6: Cover the smallest and the largest CHBW for each SCS for PUSCH JCE requirement definition.

Additional DM-RS position for BS PUSCH demod requirements with JCE (for FR1)

Status in the WF [1]:

- Candidate options:

  - Option 1: DMRS 1+1

  - Option 2: Use both DMRS 1+1 and DMRS 1+0

  - Option 3: DMRS 1+0

- Agreement in the second round:

  - Make decision on this issue in the next meeting.

  - Encourage companies to bring simulation results for both DMRS1+1 and DMRS1+0 for the next meeting

We provide simulation results on PUSCH JCE as well as the baseline PUSCH repetition under different DMRS configurations for both TDD and FDD in Table II and Table III.

It can be observed that for TDD with aTDW length of 2, larger JCE gain can be achieved with DMRS 1+1.

As for FDD with aTDW length of 8, the PUSCH demodulation with JCE has already achieved the optimal channel estimation performance regardless of the DMRS symbol number. Larger JCE gain can be achieved for DMRS1+0 because the baseline PUSCH repetition performacne is poorer.

For the purpose of better verification of PUSCH JCE, we propose to use DMRS 1+1 for TDD and DMRS 1+0 for FDD.

Observation 4: For TDD with aTDW length of 2, larger JCE gain can be achieved with DMRS 1+1.

Observation 5: For FDD with aTDW length of 8, the PUSCH demodulation with JCE has already achieved the optimal channel estimation performance regardless of the DMRS symbol number. Larger JCE gain can be achieved for DMRS 1+0 because the baseline PUSCH repetition performance is poorer.

Proposal 7: Use DMRS 1+1 for TDD and DMRS 1+0 for FDD.

Table II. Ideal simulation results for PUSCH JCE for different DMRS configurations (PUSCH Mapping type B, TDD 30kHz SCS 10MHz with aTDW length = 2)

| Antenna Configuration | DMRS | With JCE | w/o JCE | JCE Gain |
| --- | --- | --- | --- | --- |
| 1T2R | DMRS 1+0 | -4.0 | -3.7 | 0.3 |
|  | DMRS 1+1 | -4.7 | -3.9 | 0.8 |
| 1T4R | DMRS 1+0 | -7.9 | -7.2 | 0.7 |
|  | DMRS 1+1 | -8.7 | -7.5 | 1.2 |
| 1T8R | DMRS 1+0 | -11 | -10.0 | 1.0 |
|  | DMRS 1+1 | -11.8 | -10.3 | 1.5 |

Table III. Ideal simulation results for PUSCH JCE for different DMRS configurations (PUSCH Mapping type B, FDD 15kHz SCS 10MHz with aTDW length = 8)

| Antenna Configuration | DMRS | With JCE | w/o JCE | JCE Gain |
| --- | --- | --- | --- | --- |
| 1T2R | DMRS 1+0 | -9.6 | -7.5 | 2.1 |
|  | DMRS 1+1 | -9.5 | -8.3 | 1.2 |
| 1T4R | DMRS 1+0 | -13.5 | -11.0 | 2.5 |
|  | DMRS 1+1 | -13.5 | -11.2 | 2.3 |
| 1T8R | DMRS 1+0 | -16.7 | -13.6 | 3.1 |
|  | DMRS 1+1 | -16.7 | -14.0 | 2.7 |

Phase offset modelling and CFO for BS PUSCH demod requirements with JCE

Status in the WF [1]:

  - Use ideal phase modelling for the PUSCH JCE test and choose one of the following

    - Option 1: companies can consider the phase offset in the impairment results

    - Option 2: Phase offset model will be covered by TE side in the test uncertainty

  - Do not consider CFO modelling for the PUSCH JCE test and choose one of the following

    - Option 1: companies can consider the CFO impact in the impairment results.

    - Option 2: CFO impact will be covered by TE side in the test uncertainty

Since we have failed to receive TE vendors’ feedback on the phase offset modeling, we propose companies to consider the phase offset and CFO impact in the impairment results. In this way, we can reuse the MU and TT defined for normal PUSCH demodulation requirements.

Proposal 8: Companies to consider the phase offset and CFO impact in the impairment results.

Proposal 9: Reuse the MU and TT defined for normal PUSCH demodulation requirements.

- Conclusion

In this paper, China Telecom’s views on the remain issues for PUSCH coverage enhancements are given.

Proposal 1: The BS demodulation requirements for Rel-17 coverage enhancements are limited to FR1 and FR2-1, to be aligned with the RF core part requirements.

Observation 1: Once supported, TBoMS can be performed regardless of the supported TDD pattern, thus the supportive of TBoMS should be independent on the supported TDD pattern for each SCS.

Proposal 2: Define manufacturer declaration for TBoMS like below:

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| D.yyy | TB over Multi-slots (TBoMS) | Declaration of TB over Multi-slots support |

Proposal 3: Use 7D1S2U, S=6D:4G:4U for 15kHz and DDSUU, S=10G:2G:2U for 60/120kHz for requirement definition.

Observation 2: JCE requirements for 7D1S2U or DDSUU should also be applicable for other TDD patterns with even number of UL consecutive slots since JCE is still performed per 2 UL slots.

Observation 3: As for TDD patterns with odd number of UL consecutive slots, the requirements for 7D1S2U or DDSUU can be still applicable by disabling the UL transmission on the first or the last UL slot.

Proposal 4: Define test applicability rule for PUSCH JCE:

  - For each supported SCS, the requirements are also applicable for other TDD patterns with more than 1 physical UL consecutive slots.

  - Note that for the TDD pattern with odd number of UL consecutive slots, UL transmission on the first or the last UL slot is disabled.

Proposal 5: Define following manufacture declaration for PUSCH JCE:

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| D.yyy | SCS for PUSCH JCE and PUCCH JCE | Declaration of supported SCS for PUSCH JCE and PUCCH JCE, i.e. {15kHz, 30kHz, 60kHz 120kHz} |

Proposal 6: Cover the smallest and the largest CHBW for each SCS for PUSCH JCE requirement definition.

Observation 4: For TDD with aTDW length of 2, larger JCE gain can be achieved with DMRS 1+1.

Observation 5: For FDD with aTDW length of 8, the PUSCH demodulation with JCE has already achieved the optimal channel estimation performance regardless of the DMRS symbol number. Larger JCE gain can be achieved for DMRS 1+0 because the baseline PUSCH repetition performance is poorer.

Proposal 7: Use DMRS 1+1 for TDD and DMRS 1+0 for FDD.

Proposal 8: Companies to consider the phase offset and CFO impact in the impairment results.

Proposal 9: Reuse the MU and TT defined for normal PUSCH demodulation requirements.

- References

R4-2210666, WF on PUSCH demodulation performance of Rel-17 NR coverage enhancement, China Telecom, RAN4 #103-e, May 2022.