3GPP TSG-RAN WG4 Meeting # 104-e		R4-2211783

# Electronic Meeting, August 15 – August 26, 2022

Source:	China Telecom

Title:	Simulation results on BS PUSCH demodulation requirements for NR coverage enhancements

Agenda Item:	9.16.2.1

Document for:	Discussion

- Introduction

In this paper, we provide ideal and impairment simulation results on BS PUSCH demodulation requirements for NR coverage enhancements for TBoMS and PUSCH JCE. Test parameters are based on the agreed WF in [1].

- Simulation results for TBoMS

Table I. PUSCH TBoMS Results

| TBoMS over | Antenna | Duplex Mode and SCS | Channel Model | PUSCH Mappin Type | Ideal results | Impairment Results |
| --- | --- | --- | --- | --- | --- | --- |
| 4 available slots | 1T2R | FDD 15kHz SCS | TDLB100-400 Low | PUSCH Mapping type A | -7.1 | -5.1 |
|  |  |  |  | PUSCH Mapping type B | -7.2 | -5.2 |
| 2 available slots |  | TDD 15kHz SCS |  | PUSCH Mapping type A | -5.6 | -3.6 |
|  |  |  |  | PUSCH Mapping type B | -5.7 | -3.7 |
| 4 available slots |  | FDD 30kHz SCS |  | PUSCH Mapping type A | -6.8 | -4.8 |
|  |  |  |  | PUSCH Mapping type B | -7.1 | -5.1 |
| 2 available slots |  | TDD 30kHz SCS |  | PUSCH Mapping type A | -5.4 | -3.4 |
|  |  |  |  | PUSCH Mapping type B | -5.5 | -3.5 |
| 2 available slots |  | TDD 60kHz SCS | TDLA30-300 Low | PUSCH Mapping type B | -4.9 | -2.9 |
| 2 available slots |  | TDD 120kHz SCS |  | PUSCH Mapping type B | -4.9 | -2.9 |

- Simulation results for PUSCH JCE

Table II. PUSCH JCE Results for 1T2R

| JCE over | Duplex Mode and SCS | Channel Model | DMRS Configuration | PUSCH Mappin Type | Ideal Results | Impairment Results |
| --- | --- | --- | --- | --- | --- | --- |
| 8 slots | FDD 15kHz SCS 5MHz | TDLA30-10 Low | DMRS 1+0 | PUSCH Mapping type A | -9.5 | -7.5 |
|  |  |  |  | PUSCH Mapping type B | -9.6 | -7.6 |
|  |  |  | DMRS 1+1 | PUSCH Mapping type A | -9.5 | -7.5 |
|  |  |  |  | PUSCH Mapping type B | -9.5 | -7.5 |
| 2 slots | TDD 15kHz SCS 5MHz |  | DMRS 1+0 | PUSCH Mapping type A | -3.8 | -1.8 |
|  |  |  |  | PUSCH Mapping type B | -3.9 | -1.9 |
|  |  |  | DMRS 1+1 | PUSCH Mapping type A | -4.4 | -2.4 |
|  |  |  |  | PUSCH Mapping type B | -4.5 | -2.5 |
| 8 slots | FDD 30kHz SCS 10MHz |  | DMRS 1+0 | PUSCH Mapping type A | -9.9 | -7.9 |
|  |  |  |  | PUSCH Mapping type B | -9.9 | -7.9 |
|  |  |  | DMRS 1+1 | PUSCH Mapping type A | -10.0 | -8.0 |
|  |  |  |  | PUSCH Mapping type B | -10.0 | -8.0 |
| 2 slots | TDD 30kHz SCS 10MHz |  | DMRS 1+0 | PUSCH Mapping type A | -4.0 | -2.0 |
|  |  |  |  | PUSCH Mapping type B | -4.0 | -2.0 |
|  |  |  | DMRS 1+1 | PUSCH Mapping type A | -4.5 | -2.5 |
|  |  |  |  | PUSCH Mapping type B | -4.7 | -2.7 |
| 2 slots | TDD 60kHz SCS 50MHz | TDLA30-75 Low | DMRS 1+1 | PUSCH Mapping type B | -5.3 | -3.3 |
|  | TDD 120kHz SCS 50MHz |  | DMRS 1+1 | PUSCH Mapping type B | -5.7 | -3.7 |

Table III. PUSCH JCE Results for 1T4R

| JCE over | Duplex Mode and SCS | Channel Model | DMRS Configuration | PUSCH Mapping Type | Ideal Results | Impairment Results |
| --- | --- | --- | --- | --- | --- | --- |
| 8 slots | FDD 15kHz SCS 5MHz | TDLA30-10 Low | DMRS 1+0 | PUSCH Mapping type A | -13.5 | -11.5 |
|  |  |  |  | PUSCH Mapping type B | -13.5 | -11.5 |
|  |  |  | DMRS 1+1 | PUSCH Mapping type A | -13.5 | -11.5 |
|  |  |  |  | PUSCH Mapping type B | -13.5 | -11.5 |
| 2 slots | TDD 15kHz SCS 5MHz |  | DMRS 1+0 | PUSCH Mapping type A | -7.7 | -5.7 |
|  |  |  |  | PUSCH Mapping type B | -7.9 | -5.9 |
|  |  |  | DMRS 1+1 | PUSCH Mapping type A | -8.5 | -6.5 |
|  |  |  |  | PUSCH Mapping type B | -8.6 | -6.6 |
| 8 slots | FDD 30kHz SCS 10MHz |  | DMRS 1+0 | PUSCH Mapping type A | -13.9 | -11.9 |
|  |  |  |  | PUSCH Mapping type B | -13.8 | -11.8 |
|  |  |  | DMRS 1+1 | PUSCH Mapping type A | -13.8 | -11.8 |
|  |  |  |  | PUSCH Mapping type B | -13.7 | -11.7 |
| 2 slots | TDD 30kHz SCS 10MHz |  | DMRS 1+0 | PUSCH Mapping type A | -7.9 | -5.9 |
|  |  |  |  | PUSCH Mapping type B | -7.9 | -5.9 |
|  |  |  | DMRS 1+1 | PUSCH Mapping type A | -8.7 | -6.7 |
|  |  |  |  | PUSCH Mapping type B | -8.7 | -6.7 |
|  | [TDD 30kHz SCS 100MHz] |  | DMRS 1+0 | PUSCH Mapping type A | -8.1 | -6.1 |
|  |  |  | DMRS 1+1 | PUSCH Mapping type A | -8.8 | -6.8 |

Table Ⅳ. PUSCH JCE Results for 1T8R

| JCE over | Duplex Mode and SCS | Channel Model | DMRS Configuration | PUSCH Mapping Type | Ideal Results | Impairment Results |
| --- | --- | --- | --- | --- | --- | --- |
| 8 slots | FDD 15kHz SCS 5MHz | TDLA30-10 Low | DMRS 1+0 | PUSCH Mapping type A | -16.6 | -14.6 |
|  |  |  |  | PUSCH Mapping type B | -16.7 | -14.7 |
|  |  |  | DMRS 1+1 | PUSCH Mapping type A | -16.6 | -14.6 |
|  |  |  |  | PUSCH Mapping type B | -16.7 | -14.7 |
| 2 slots | TDD 15kHz SCS 5MHz |  | DMRS 1+0 | PUSCH Mapping type A | -10.9 | -8.9 |
|  |  |  |  | PUSCH Mapping type B | -10.9 | -8.9 |
|  |  |  | DMRS 1+1 | PUSCH Mapping type A | -11.9 | -9.9 |
|  |  |  |  | PUSCH Mapping type B | -11.9 | -9.9 |
| 8 slots | FDD 30kHz SCS 10MHz |  | DMRS 1+0 | PUSCH Mapping type A | -16.9 | -14.9 |
|  |  |  |  | PUSCH Mapping type B | -16.9 | -14.9 |
|  |  |  | DMRS 1+1 | PUSCH Mapping type A | -16.9 | -14.9 |
|  |  |  |  | PUSCH Mapping type B | -16.9 | -14.9 |
| 2 slots | TDD 30kHz SCS 10MHz |  | DMRS 1+0 | PUSCH Mapping type A | -11.0 | -9.0 |
|  |  |  |  | PUSCH Mapping type B | -11.0 | -9.0 |
|  |  |  | DMRS 1+1 | PUSCH Mapping type A | -11.8 | -9.8 |
|  |  |  |  | PUSCH Mapping type B | -11.9 | -9.9 |
|  | [TDD 30kHz SCS 100MHz] |  | DMRS 1+0 | PUSCH Mapping type A | -10.9 | -8.9 |
|  |  |  | DMRS 1+1 | PUSCH Mapping type A | -11.8 | -9.8 |

- References

R4-2210666, WF on PUSCH demodulation performance of Rel-17 NR coverage enhancement, China Telecom, RAN4 #103-e, May 2022.