3GPP TSG-RAN WG4 Meeting # 104-e		R4-2211784

# Electronic Meeting, August 15 – August 26, 2022

Source:	China Telecom

Title:	Discussion on the CRS-IM test with only inter-RAT MO configured for scenario 2

Agenda Item:	9.10.1.3

Document for:	Discussion

- Introduction

In the RAN4#103-e meeting, all issues for CRS-IM under DSS scenario (scenario 1) were agreed, and there still remain open issues for the CRS-IM test with only inter-RAT MO configured for non-DSS scenario (scenario 2). The WF was approved in [1] that captures all the agreements and remain issues in the last meeting.

In this paper, we give our views on the remaining open issues for CRS-IM under scenario 2.

- Discussion

Whether the same CRS-IM test requirements can be applied in the two sets of test setup in scenario 2 for 15kHz SCS

Status in the WF in [1]:

  - RAN4 target to specify single set of requirements for two sets with NWA and only inter-RAT MO configured pending on the performance with power detection method.

For UEs supporting CRS-IM for scenario 2 without NWA signaling on the CHBW, UE can obtain the LTE bandwidth information either by power detection and/or neighbour cell PBCH decoding. Since the system bandwidth for LTE is limited, we expect reliable power detection accuracy under the agreed INR values, i.e., no LTE CHBW misdetection can be assumed. Therefore, we propose to use same requirement for the two sets of test setup in scenario 2 for 15kHz SCS.

Proposal 1: Use same requirement for the two sets of test setup in scenario 2 for 15kHz SCS.

Test applicability

Status in the WF in [1]:

  - If a UE supports both without NWA and NWA based CRS-IM for 15kHz SCS (i.e., UE Capability #2 and #3),

    - Option 1: UE is only required to pass performance requirements without NWA signalling based test setup, i.e. UE capability#2.

    - Option 2: UE is only required to pass performance requirements with NWA signalling based test setup, i.e. UE capability#3.

  - If a UE supports both without NWA and NWA based CRS-IM for 30kHz SCS (i.e., UE Capability #4 and #5),

    - Option 1: UE is only required to pass performance requirements without NWA signalling based test setup, i.e. UE capability#4.

    - Option 2: UE is only required to pass performance requirements with NWA signalling based test setup, i.e. UE capability#5.

Compared with capability#3/5, the UE capability of obtaining the correct LTE CHBW information by PBCH and/or power detection (capability#2/4) is more complicated for the UE implementation. Moreover, as have been discussed in the previous meetings, the mis-detection of LTE CHBW information would have big performance decrease for CRS-IM, which is not likely happened with NWA configured.

Thus we think the UE capability for CRS-IM without NWA is more important to be verified as long as it is supported.

Proposal 2: For UEs declare to support CRS-IM both with and without NWA on a certain each SCS, at least the performance requirements without NWA signaling based test setup should be passed.

Test case structure in TS38.101-4

In the last meeting, it was remained open on whether the CRS-IM requirements for TDD scenario 2 with 15 and 30kHz SCS should be captured within one clause [2]:

  - Whether to define the CRS-IM requirements for TDD scenario 2 with 15kHz SCS and 30kHz SCS in the same or different sections can be further discussed in the next meeting.

Based on the up-to-date agreement, most of the test configurations and all the necessary test procedures for TDD scenario 2 for 15kHz SCS and 30kHz SCS are the same. Moreover, in our understanding, we usually do not keep test requirements for the same feature under the same scenario into separate clauses just because they are for different SCS.

Considering the above, we propose to keep the CRS-IM requirements for TDD scenario 2 with 15kHz SCS and 30kHz SCS in the same clause in TS38.101-4.

Proposal 3: Keep the CRS-IM requirements for TDD scenario 2 with 15kHz SCS and 30kHz SCS in the same clause in TS38.101-4.

SNR value requirement derivation for CRS-IM

In the endorsed draft big CR in the last meeting, all SNR values are ‘TBD’ since it was the first meeting we submitted the CRS-IM simulation results. Considering we are going to close this WI in this meeting and the SNR value deriving rule is also being discussed in the MMSE-IRC topic, we propose to reuse the same rule to and the SPAN should be controlled within 2.0 dB, which is same with the NR Rel-15 SNR requirement derivation.

Proposal 4: For this meeting, make decision on the SNR requirement values for CRS-IM with the following rule:

  - RAN4 does not consider the farthest result(s) from the ideal AVERAGE value, until the span becomes 2.0 dB or less. The final requirements are derived from AVERAGE impairment results with the corresponding ideal results whose span is within 2.0 dB.

- Conclusions

In this paper, we give our views on the remaining open issues.

Proposal 1: Use same requirement for the two sets of test setup in scenario 2 for 15kHz SCS.

Proposal 2: For UEs declare to support CRS-IM both with and without NWA on a certain each SCS, at least the performance requirements without NWA signaling based test setup should be passed.

Proposal 3: Keep the CRS-IM requirements for TDD scenario 2 with 15kHz SCS and 30kHz SCS in the same clause in TS38.101-4.

Proposal 4: For this meeting, make decision on the SNR requirement values for CRS-IM with the following rule:

  - RAN4 does not consider the farthest result(s) from the ideal AVERAGE value, until the span becomes 2.0 dB or less. The final requirements are derived from AVERAGE impairment results with the corresponding ideal results whose span is within 2.0 dB.

- References

R4-2210659, WF on the test with only inter-RAT MO configured for scenario 2, Huawei, HiSilicon, RAN4 #103-e, May 2022.

R4-2210524, Email discussion summary for [103-e][321] NR_perf_enh2_Demod_Part1, China Telecom, RAN4 #103-e, May 2022.