3GPP TSG-RAN4 Meeting #104e	R4-2211787

# Electronic Meeting, August 15 – August 26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-1 | CR | - | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on PUSCH with DMRS bundling BS performance test for FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_cov_enh-Perf |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 has agreed to define PUSCH with DMRS bundling BS performance test for FR1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced sections for PUSCH with DMRS bundling BS performance test for FR1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance part of NR_cov_enh can not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.13 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

8.2.13	Performance requirements for PUSCH with DMRS bundling

8.2.13.1	Definition and applicability

The performance requirement of PUSCH with DMRS bundling is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ re-transmissions.

Which specific test(s) are applicable to BS is based on the test applicability rules defined in clause [TBA].

8.2.13.2	Minimum Requirement

The minimum requirement is in TS 38.104 [2] clause [TBA].

8.2.13.3	Test Purpose

The test shall verify the receiver's ability to achieve throughput under multipath fading propagation conditions for a given SNR with DMRS bundling.

8.2.13.4	Method of test

8.2.13.4.1	Initial Conditions

Test environment:	Normal, see annex B.2.

RF channels to be tested for single carrier:	M; see clause 4.9.1.

RF channels to be tested for carrier aggregation: MBW Channel CA; see clause 4.9.1.

8.2.13.4.2	Procedure

1)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to all BS antenna connectors for diversity reception via a combining network as shown in annex D.5 and D.6 for BS type 1-C and type 1-H respectively.

2)	Adjust the AWGN generator, according to the channel bandwidth, defined in table 8.2.13.4.2-1.

Table 8.2.13.4.2-1: AWGN power level at the BS input

| Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- |
| 15kHz | 5 | -86.5 dBm / 4.5MHz |
|  | [50] | TBD |
| 30kHz | 10 | -83.6 dBm / 8.64MHz |
|  | [100] | TBD |
| NOTE: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |  |

3)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A and the test parameters in table 8.2.13.4.2-2.

Table 8.2.13.4.2-2: Test parameters for testing PUSCH with DMRS bundling

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence (Note 2) | 0, 3, 0, 3 for TDD 0, 0, 0, 0 for FDD |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | TBD |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| Configured time domain window (cTDW) length |  | 2 for TDD 8 for FDD |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to TBA. NOTE 2: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |

4)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex G.

5)	Adjust the equipment so that required SNR specified in table 8.2.13.5-1 to table 8.2.13.5-16 is achieved at the BS input.

6)	For each of the reference channels in table 8.2.13.5-1 to table 8.2.13.5-16 applicable for the base station, measure the throughput.

8.2.1.5	Test Requirement

The throughput measured according to clause 8.2.13.4.2 shall not be below the limits for the SNR levels specified in table 8.2.13.5-1 to table 8.2.13.5-16.

Table 8.2.13.5-1: Test requirements for PUSCH with 70% of maximum throughput, Type A, 5 MHz channel bandwidth, FDD 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-10 Low | 70 % | TBA | TBA | TBA |
|  | 4 | Normal | TDLA30-10 Low | 70 % |  |  |  |
|  | 8 | Normal | TDLA30-10 Low | 70 % |  |  |  |

Table 8.2.13.5-2: Test requirements for PUSCH with 70% of maximum throughput, Type B, 5 MHz channel bandwidth, FDD 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-10 Low | 70 % | TBA | TBA | TBA |
|  | 4 | Normal | TDLA30-10 Low | 70 % |  |  |  |
|  | 8 | Normal | TDLA30-10 Low | 70 % |  |  |  |

[Table 8.2.13.5-3: Test requirements for PUSCH with 70% of maximum throughput, Type A, 50 MHz channel bandwidth, FDD 15 kHz SCS]

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-10 Low | 70 % | TBA | TBA | TBA |
|  | 4 | Normal | TDLA30-10 Low | 70 % |  |  |  |
|  | 8 | Normal | TDLA30-10 Low | 70 % |  |  |  |

[Table 8.2.13.5-4: Test requirements for PUSCH with 70% of maximum throughput, Type B, 50 MHz channel bandwidth, FDD 15 kHz SCS]

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-10 Low | 70 % | TBA | TBA | TBA |
|  | 4 | Normal | TDLA30-10 Low | 70 % |  |  |  |
|  | 8 | Normal | TDLA30-10 Low | 70 % |  |  |  |

Table 8.2.13.5-5: Test requirements for PUSCH with 70% of maximum throughput, Type A, 5 MHz channel bandwidth, TDD 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-10 Low | 70 % | TBA | TBA | TBA |
|  | 4 | Normal | TDLA30-10 Low | 70 % |  |  |  |
|  | 8 | Normal | TDLA30-10 Low | 70 % |  |  |  |

Table 8.2.13.5-6: Test requirements for PUSCH with 70% of maximum throughput, Type B, 5 MHz channel bandwidth, TDD 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-10 Low | 70 % | TBA | TBA | TBA |
|  | 4 | Normal | TDLA30-10 Low | 70 % |  |  |  |
|  | 8 | Normal | TDLA30-10 Low | 70 % |  |  |  |

[Table 8.2.13.5-7: Test requirements for PUSCH with 70% of maximum throughput, Type A, 50 MHz channel bandwidth, TDD 15 kHz SCS]

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-10 Low | 70 % | TBA | TBA | TBA |
|  | 4 | Normal | TDLA30-10 Low | 70 % |  |  |  |
|  | 8 | Normal | TDLA30-10 Low | 70 % |  |  |  |

[Table 8.2.13.5-8: Test requirements for PUSCH with 70% of maximum throughput, Type B, 50 MHz channel bandwidth, TDD 15 kHz SCS]

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-10 Low | 70 % | TBA | TBA | TBA |
|  | 4 | Normal | TDLA30-10 Low | 70 % |  |  |  |
|  | 8 | Normal | TDLA30-10 Low | 70 % |  |  |  |

Table 8.2.13.5-9: Test requirements for PUSCH with 70% of maximum throughput, Type A, 10 MHz channel bandwidth, FDD 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-10 Low | 70 % | TBA | TBA | TBA |
|  | 4 | Normal | TDLA30-10 Low | 70 % |  |  |  |
|  | 8 | Normal | TDLA30-10 Low | 70 % |  |  |  |

Table 8.2.13.5-10: Test requirements for PUSCH with 70% of maximum throughput, Type B, 10 MHz channel bandwidth, FDD 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-10 Low | 70 % | TBA | TBA | TBA |
|  | 4 | Normal | TDLA30-10 Low | 70 % |  |  |  |
|  | 8 | Normal | TDLA30-10 Low | 70 % |  |  |  |

[Table 8.2.13.5-11: Test requirements for PUSCH with 70% of maximum throughput, Type A, 100 MHz channel bandwidth, FDD 30 kHz SCS]

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-10 Low | 70 % | TBA | TBA | TBA |
|  | 4 | Normal | TDLA30-10 Low | 70 % |  |  |  |
|  | 8 | Normal | TDLA30-10 Low | 70 % |  |  |  |

[Table 8.2.13.5-12: Test requirements for PUSCH with 70% of maximum throughput, Type B, 100 MHz channel bandwidth, FDD 30 kHz SCS]

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-10 Low | 70 % | TBA | TBA | TBA |
|  | 4 | Normal | TDLA30-10 Low | 70 % |  |  |  |
|  | 8 | Normal | TDLA30-10 Low | 70 % |  |  |  |

Table 8.2.13.5-13: Test requirements for PUSCH with 70% of maximum throughput, Type A, 10 MHz channel bandwidth, TDD 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-10 Low | 70 % | TBA | TBA | TBA |
|  | 4 | Normal | TDLA30-10 Low | 70 % |  |  |  |
|  | 8 | Normal | TDLA30-10 Low | 70 % |  |  |  |

Table 8.2.13.5-14: Test requirements for PUSCH with 70% of maximum throughput, Type B, 10 MHz channel bandwidth, TDD 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-10 Low | 70 % | TBA | TBA | TBA |
|  | 4 | Normal | TDLA30-10 Low | 70 % |  |  |  |
|  | 8 | Normal | TDLA30-10 Low | 70 % |  |  |  |

[Table 8.2.13.5-15: Test requirements for PUSCH with 70% of maximum throughput, Type A, 100 MHz channel bandwidth, TDD 30 kHz SCS]

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-10 Low | 70 % | TBA | TBA | TBA |
|  | 4 | Normal | TDLA30-10 Low | 70 % |  |  |  |
|  | 8 | Normal | TDLA30-10 Low | 70 % |  |  |  |

[Table 8.2.13.5-16: Test requirements for PUSCH with 70% of maximum throughput, Type B, 100 MHz channel bandwidth, TDD 30 kHz SCS]

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-10 Low | 70 % | TBA | TBA | TBA |
|  | 4 | Normal | TDLA30-10 Low | 70 % |  |  |  |
|  | 8 | Normal | TDLA30-10 Low | 70 % |  |  |  |

<End of change>