3GPP TSG-RAN WG4 Meeting # 104-e	R4-2211832

# Electronic Meeting, August 15 – August 26, 2022

Agenda item:	9.18.5.1.2

Source:	Apple Inc.

Title:	On PDCCH/PBCH Demodulation Requirements for Reduced Capability Devices in NR

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#103-e RedCap UE demodulation and CSI reporting requirements were discussed and way forward [1] was agreed. In this contribution we present the requested simulation results for PDCCH and PBCH demodulation requirements.

# 2. Discussion

In [1] the simulation assumptions for PDCCH PBCH demodulation requirements for RedCap UE were agreed. No additional open issues remain; hence the requested simulation results are presented.

Simulation Results

For the agreed simulation parameters stated in [1], we contribute with our performance results for PDCCH and PBCH demodulation. For clarity of exposition, we present the full table requested in [1] but omitting the results of those test cases that are reused from Rel.15.

FR1 FDD 15kHz

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | SNR[dB] / Reuse | Impair Res.[dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10 | 48 | 1 | 8 | TDLA30-10 | 2x1 low | 1% | 5.3.2.1.2-1 Test 3 (R.PDCCH.1-1.3 FDD) | 1.6 | 3.5 |
| 2Rx UE | 10 | 48 | 2 | 4 | TDLA30-10 | 1x2 low | 1% | 5.3.2.1.1-1 Test 3 (R.PDCCH.1-2.4 FDD) | Reuse | Reuse |
| 2Rx UE | 10 | 48 | 1 | 8 | TDLA30-10 | 2x2 low | 1% | 5.3.2.1.2-1 Test 3 (R.PDCCH.1-1.3 FDD) | Reuse | Reuse |

FR1 TDD 30kHz

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | SNR[dB] / Reuse | Impair Res.[dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20 | 48 | 1 | 4 | TDLC300-100 | 1x1 | 1% | 5.3.2.2.1-1 Test 2 (R.PDCCH.2-1.2 TDD) | 7.1 | 9.2 |
| 2Rx UE | 20 | 48 | 1 | 4 | TDLC300-100 | 1x2 low | 1% | 5.3.2.2.1-1 Test 2 (R.PDCCH.2-1.2 TDD) | 2.2 | 4.0 |
| 2Rx UE | 20 | 48 | 1 | 8 | TDLC300-100 | 2x2 low | 1% | 5.3.2.2.2-1 Test 1 (R.PDCCH.2-1.3 TDD) | -2.2 | -0.4 |

FR2 TDD 120kHz

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | New / Reuse | Impair Res.[dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx UE | 100 | 60 | 1 | 4 | TDLA30-300 | 1x2 low | 1% | 7.3.2.2.1-1 Test 1-2 (R.PDCCH. 5-1.2 TDD) | Reuse | Reuse |
| 2Rx UE | 100 | 60 | 1 | 8 | TDLA30-75 | 2x2 low | 1% | 7.3.2.2.2-1 Test 2-1 (R.PDCCH.5-1.3 TDD) | Reuse | Reuse |

# 3. Conclusion

In this contribution we provide the requested simulation results for PDCCH and PBCH demodulation requirements for RedCap UE.

# Reference

- R4-2210672, “WF on RedCap demodulation and CQI reporting requirements”, Ericsson.