3GPP TSG-RAN WG4 Meeting # 104-e	R4-2211844

# Electronic Meeting, August 15 – August 26, 2022

Agenda item:	9.8.1.3

Source:	Apple

Title:	On PUCCH SCell activation requirement and testing

Document for:	Discussion

# Introduction

In RAN4 #103e meeting, the WF[1] for PUCCH SCell activation was agreed, but there are still some open issues for testing configuration as followings,

### Issue 1-1-2: Whether the PL-RS will introduce extra delay time when the known condition is met in FR2 (the value of [X] in 8.3.12)? Candidate options: - Option 1: - When PL-RS of target PUCCH SCell is known, the X=5 sample measurement time is always considered and no need to consider condition of ‘maintain’ or ‘not maintain’. - Option 2: - If PUCCH Scell is known in FR2, the reported L3 measurement results can be reused for pathloss estimation and additional PL-RS measurement is not needed during PUCCH Scell activation. - If PUCCH Scell is unknown in FR2, [X] = 0 if PL-RS is maintained and [X] = 5 if PL-RS is not maintained. - Option 3: - X=4 when the PUCCH Scell activated is unknown at the reception of PUCCH Scell activation command and the RS used for L1-RSRP is same as PL-RS. - For all other cases where PL-RS not maintained is X=5. ### Issue 2-1-5: Whether the test cases for the following band combinations are needed? Agreements: - Define test cases for: - FR1 PCell/PSCell + FR1 PUCCH SCell - FR2 PCell/PSCell + FR2 PUCCH SCell(inter-band) - FFS whether to define test cases for: - FR1 PCell/PSCell + FR2 PUCCH SCell

In this contribution, we discuss the above remaining issues from last RAN4 meeting.

# Discussion

### Issue 1-1-2: Whether the PL-RS will introduce extra delay time when the known condition is met in FR2 (the value of [X] in 8.3.12)

For known PUCCH SCell, if PL-RS is known that means L3 measurement has been performed and reported on PL-RS or the DL-RS which is QCLed with PL-RS, but it doesn’t mean the PL-RS is maintained at UE since this PUCCH SCell is an being activated SCell rather than a real active serving cell (the L1-RSRP for beam management is not active on the being-activated SCell). Thus, the 5 samples time are always needed and no need to consider condition of ‘maintain’ or ‘not maintain’.

For unknown PUCCH SCell, the PL-RS is known because of L1-RSRP report before the PL-RS activation, but UE never did any actual periodical PL-RS measurement for pathloss estimation, e.g., the L1-RSRP measurement before uplink spatial relation and PL-RS activation is mainly used to determine the beam information rather than pathloss estimation. Thus, the 5 samples time are always needed and no need to consider condition of ‘maintain’ or ‘not maintain’.

Regarding issue 1-1-2, we think when PL-RS of target PUCCH SCell is known, the X=5 sample measurement time is considered and no need to consider condition of ‘maintain’ or ‘not maintain’ for known PL-RS.

However, in last meeting, companies also mentioned that, for unknown SCell case, if the RS used for L1-RSRP is same as PL-RS, one sample measured from L1-RSRP can also be used for the following RL-RS measurement filtering, and therefore the sample number needed for the PL-RS measurement after L1-RSRP measurement can be 4. Although that will make UE to differently implement a processing method for PL-RS measurement filtering, i.e., store one PHY sample from previous L1-RSRP measurement during unknown PUCCH SCell activation, we can compromise to it for this special case to move forward.

Proposal 1: When PL-RS of target PUCCH SCell is known, the X=5 sample measurement time is always considered and no need to consider condition of ‘maintain’ or ‘not maintain’.

To move forward, we can also compromise to option 3:

  - X=4 when the PUCCH Scell activated is unknown at the reception of PUCCH Scell activation command and the RS used for L1-RSRP is same as PL-RS.

  - For all other cases where PL-RS not maintained is X=5.

### Issue 2-1-5: Whether the test cases for the following band combinations are needed?

In previous RAN4 meeting, the WF on Rel-15 NR RRM test case related issues was agreed in [2], and main conclusion for FR1+FR2 testability is as following:

## Criteria for selecting FR1/LTE+FR2 test with OTA testability problem - FR1/LTE+FR2 test has OTA testability problem if at least one of the following criteria is met: - Tests where any requirement is tested for FR1/LTE, - Tests where UE receives any DL message (e.g. RRC/DCI/MAC-CE configuration message/command etc) on FR1/LTE between the starting point and ending point of the test, and - Tests where UE transmits any UL signal (e.g. measurement report, ACK/NACK, CSI etc) b on FR1/LTE between the starting point and ending point of the test.

In the following test cases, the criteria for identifying testability problem is met:

- FR1 PCell/PSCell + FR2 PUCCH SCell: MAC-CE for activation received on FR1 and in some cases cross-PUCCH group CSI reporting (L1-RSRP for beam information)

Thus, we think FR1 PCell/PSCell + FR2 PUCCH SCell test cases for PUCCH SCell activation requirement shall be delayed until testability issues are solved.

Proposal 2: FR1 PCell/PSCell + FR2 PUCCH SCell test cases for PUCCH SCell activation requirement shall be delayed until testability issues are solved

# Conclusion

In this contribution, we discuss the above remaining issues for PUCCH SCell activation from last RAN4 meeting.

Proposal 1: When PL-RS of target PUCCH SCell is known, the X=5 sample measurement time is always considered and no need to consider condition of ‘maintain’ or ‘not maintain’.

Proposal 2: FR1 PCell/PSCell + FR2 PUCCH SCell test cases for PUCCH SCell activation requirement shall be delayed until testability issues are solved

# References

[1] R4-2211208, WF on further RRM enhancement for NR and MR-DC - PUCCH SCell activation/deactivation requirements, CATT, RAN4 #103e.

[2] R4-2115240, WF on Rel-15 NR RRM test case related issues, Ericsson, RAN4 #100e