3GPP TSG-RAN4 Meeting #104-e	R4-2211873

# Electronic Meeting, Aug. 15 – 26, 2022

Agenda item:	9.14.2

Source:	Apple

Title:   	Remaining issues on system parameters for NR operation in 52.6GHz - 71GHz

Document for:	Discussion

# Introduction

In RAN4 meeting#103-e, most of the work related to system parameters was completed, including channel raster and sync. raster. However, there are still some remaining issues that would require further discussion. In this contribution, we discuss two issues, one is the note for band operating bands table, i.e., Table 5.2-1 in 38.101-2, and the other is channel spacing for CA.

# Discussion

The note for unlicensed operation

The current note in Table 5.2-1 is put in brackets, as shown below

During the discussion, there were mainly two technical points. The first point is to recognize this band n263 is an unlicensed band, and needs to be clearly designated so. The second point is while it is unlicensed band, shared spectrum channel access such as LBT is not mandated in all the applicable regions or countries.

While the current note captures the essence of what this unlicensed band is, it can benefit from some further wording refinement. As discussed at the last meeting, the following two alternatives are acceptable to us:

Alt. 1: NOTE 1: This band is for unlicensed operation.

Alt. 2: NOTE 1: This band is for unlicensed operation and subject to regional and/or country specific regulations.

Alt. 1 is concise while Alt. 2 provides a bit more info, although such additional info is helpful, but not essential.

Proposal 1: RAN4 to agree on either Alt. 1 or Alt. 2 for Table 5.2-1.

Channel spacing for CA

With the fixed channel raster, with the minimum spacing of 100.8MHz between 100MHz channels and 403.2MHz for other channel bandwidth including 400MHz/800MHz/1600MHz/2000MHz. In addition, there are some additional 2000MHz channels specified to maximize the spectrum utilization.

To enable continuous CA, we need to update the following definition of channel spacing for CA.

If we don’t consider CA of 2000MHz with another CBW, it can be seen that the channel spacing between two center frequencies of two channels are multiple of 100.8MHz or multiple of 100.8MHz plus 50.4MHz. Therefore, we propose the following:

Proposal 2: For NR operating bands in FR2-2, nominal channel spacing is when the center frequencies of two closest channels are multiple of 100.8MHz or multiple of 100.8MHz plus 50.4MHz and the two channels do not overlap. Note CA of 2000MHz CBW with another CBW is not considered.

# Conclusion

In this contribution, we discussed the remaining issues on system parameters. Specifically, we have the following proposals:

Proposal 1: RAN4 to agree on either Alt. 1 or Alt. 2 for Table 5.2-1.

Proposal 2: For NR operating bands in FR2-2, nominal channel spacing is when the center frequencies of two closest channels are multiple of 100.8MHz or multiple of 100.8MHz plus 50.4MHz and the two channels do not overlap. Note CA of 2000MHz CBW with another CBW is not considered.

# References

- RP-202925, “Revised WID:  Extending current NR operation to 71 GHz”