3GPP TSG RAN WG4 Meeting #104-e	R4-2211886

# E-meeting, 15 May 2022 – 26 May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft CR | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on UL gaps for BPS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh2-Core |  |  |  |  |  | Date: |  |  | 2022-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Confirm X value in prioritization of valid CSI reporting during RRM procedures over overlapping UL gaps. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Start of change 1 >

### 9.1.11	UL gap for Tx power management

The UL gap pattern for TX power management are listed in Table 9.1.11-1 if UE supports the UL gap for Tx power management, the UE shall support at least one of UL MGP#1 and UL MGP#3. All other UL MGPs are optional.

Table 9.1.11-1: UL Gap Pattern Configurations

|  | UL Gap Length (UGL) [ms] | UL gap repetition periodicity (UGRP) [ms] |
| --- | --- | --- |
| UL MGP #0 | 1.0 | 20 |
| UL MGP #1 | 1.0 | 40 |
| UL MGP #2 | 0.5 | 160 |
| UL MGP #3 | 0.125 when SCS of active UL BWP =120kHz 0.25 when SCS of active UL BWP =60kHz | 5 |

An uplink gap consists of consecutive static UL slot(s) in one or more TDD-UL-DL-Pattern duration, starting from the first static UL slot of an UL gap repetition period. UGL is the aggregated length of consecutive UL slots used as the UL gap within an UL gap repetition period. That means, there can be a DL slot and/or special slot but no static UL slot between the two consecutive static UL slots within the UL gap length.

When an UL gap overlaps with an uplink transmission in NR serving cells in FR2 single CC or FR2 intra-band CA, then the UE is not required to conduct any transmission during the UL gap on the NR serving cells other than those listed below:

-	UL signals belonging to random access procedure according to TS 38.321 [7].

-	the PUSCH transmissions due to configured grant (CG-PUSCH (type 1 and 2)) [2].

-	the valid CSI report and/or valid L1-RSRP report during SCell activation procedure, where the valid CSI report is valid CQI with non-zero CQI index defined in clause 5.2.2.1, TS 38.214 [26] and the valid L1-RSRP report is non lowest L1-RSRP defined in clause 10.1.6.

-	The UE need not apply UL gap prioritization rules specified above for SCell activation procedure if the time period between UL gap colliding with CSI report of non-zero CQI or L1-RSRP and the slot where the SCell activation MAC CE or CSI report activation command is received is less than 10ms.

-	the PUCCH allocations for scheduling request (SR) and link recovery request (LRR) defined in clause 8.5.

For inter-band FR2-FR2 CA/DC, UE may or may not be required to conduct transmission to the corresponding NR serving cells based on UE capability whether UL transmission within a gap is feasible.

< End of change 1>