3GPP TSG-RAN4 Meeting #104-e	R4-2211894

# Electronic Meeting, Aug 15 – Aug 26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draft | rev |  | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Intra-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MG in FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh-Perf |  |  |  |  |  | Date: |  |  | 2022-07-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As agreed in WF R4-2210586, a test case for intra-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MG in FR2 needs to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a test case for intra-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MG in FR2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The corresponding test case would still be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) A7.6.X1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

#### A7.6.X1	Intra-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MG in FR2

##### A7.6.X1.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event with autonomous activation/deactivation of Pre-MG. This test will partly verify the TDD intra-frequency cell search requirements in clause 9.2.5.1 and 9.2.5.2. Supported test configurations are shown in Table A7.6.X1.1-1.

Table A7.6.X1.1-1: supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

There are two cells in the test, PCell (Cell 1) and a FR2 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for the Cell 1 and Cell 2 are given in Table A7.6.X1.1-2, A7.6.X1.1-3 and A7.6.X1.1-4 below.

In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A4 is used.

Before the test starts,

UE is connected to Cell 1 (PCell) on radio channel 1 (PCC).

UE is configured with 2 different UE-specific bandwidth parts for Cell 1 (PCell), BWP-1 and BWP-2, before starting the test.

BWP-1 includes bandwidth of the initial DL BWP and SSB.

BWP-2 does not include bandwidth of the initial DL BWP and SSB.

UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 in PCell.

The TE schedules ontinuous DL data on PCell throughout the test.

The test consists of 4 successive time periods, with durations of T1, T2, T3 and T4, respectively.

The time period T1 starts when a DCI format 1_1 command for PCell DL BWP switch, sent from the test equipment to the UE, is received at the UE side in PCell’s slot # denoted i. The UE shall switch its bandwidth part from BWP-1 to BWP-2.

The time period T3 starts when a DCI format 1_1 command for PCell DL BWP switch, sent from the test equipment to the UE, is received at the UE side in PCell’s slot # denoted j. The UE shall switch its bandwidth part from BWP-2 to BWP-1.

Table A7.6.X1.1-2: General test parameters for intra-frequency event triggered reporting with network-controlled activation/deactivation of Pre-MG

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | PCell (Cell 1) |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1: Cell 1 and Cell 2 | One TDD carrier frequency is used for the NR cells. |
| SMTC configuration |  | SMTC.1 |  |
| A4-Offset | dB | -120 |  |
| CP length |  | Normal |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| Gap Pattern Id |  | 13 |  |
| Measurement gap offset | ms | 39 |  |
| Time offset between Cell 1 and Cell 2 |  | 3 s | Synchronous cells |
| T1 | s | 0.1 |  |
| T2 | s | 5 |  |
| T3 | s | 0.1 |  |
| T4 | s | 5 |  |

Table A7.6.X1.1-3: NR Cell specific test parameters for intra-frequency event triggered reporting with network-controlled activation/deactivation of Pre-MG

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| TDD configuration |  | TDDConf.3.1 |  |  |  | TDDConf.3.1 |  |  |  |
| BWchannel | MHz | 100: NRB,c = 66 |  |  |  | 100: NRB,c = 66 |  |  |  |
| Data RBs allocated |  | 24 |  |  |  | 24 |  |  |  |
| Intial BWP configuration |  | DLBWP.0.1 ULBWP.0.1 |  |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |  |
| BWP-1 Configuration |  | DLBWP.1.6 ULBWP.1.6 |  |  |  | N/A |  |  |  |
| BWP-2 Configuration |  | DLBWP.1.5 ULBWP.1.5 |  |  |  | N/A |  |  |  |
| RLM-RS |  | CSI-RS |  |  |  | N/A |  |  |  |
| PDSCH RMC configuration |  | SR.3.2 TDD |  |  |  | N/A |  |  |  |
| RMSI CORESET RMC configuration |  | CR.3.1 TDD |  |  |  | N/A |  |  |  |
| Dedicated CORESET RMC configuration |  | CCR.3.1 TDD |  |  |  | N/A |  |  |  |
| TRS configuration |  | TRS.2.1 TDD |  |  |  | N/A |  |  |  |
| PDSCH/PDCCH TCI states |  | TCI.State.2 |  |  |  | N/A |  |  |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 |  |  |  | 120 |  |  |  |
| OCNG Patterns |  | OP.5 |  |  |  | N/A |  |  |  |
| cellIndividualOffset | dB | N/A |  |  |  | 16 |  |  |  |
| SSB |  | SSB.1 FR2 |  |  |  | SSB.7 FR2 |  |  |  |
| Propagation Condition |  | AWGN |  |  |  | AWGN |  |  |  |

Table A7.6.X1.1-4: NR OTA Cell specific test parameters for intra-frequency event triggered reporting with network-controlled activation/deactivation of Pre-MG

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| AoA setup |  | Setup 3 defined in A.3.15.3 |  |  |  |  |  |  |  |
|  |  | AoA1 |  |  |  | AoA2 |  |  |  |
| Beam assumptionNote 3 |  | Rough |  |  |  | Rough |  |  |  |
| Es | dBm/SCS | -89 | -89 | -89 | -89 | -infinity | -89 | -infinity | -89 |
| BB Note 4 | dB | -0.12 | -0.12 | -0.12 | -0.12 | -Infinity | -0.12 | -Infinity | -0.12 |
| SSB_RP | dBm/SCS | -89 | -89 | -89 | -89 | -infinity | -89 | -infinity | -89 |
|  | dBm/95.04MHz | -64.41 | -64.41 | -64.41 | -64.41 | -Infinity | -64.41 | -Infinity | -64.41 |
| Time multiplexing of the downlink transmissions from each AoA |  | Defined in Figure A.7.6.1.1.1-1 |  |  |  |  |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 4: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |  |  |

##### A7.6.X1.2	Test Requirements

During T1 and T2, the UE shall be able to receive PDSCH and report corresponding valid ACK/NACK for those PDSCHs scheduled in the slots overlapped with the Pre-MG occasions, starting from the 1st complete Pre-MG occasion after the beginning of PCell’s DL slot (i+TBWPswitchDelay) + 5ms as defined in clause 8.19.2.

During T3 and T4, the UE shall not report corresponding valid ACK/NACK for those PDSCHs scheduled in the slots overlapped with the Pre-MG occasions, starting from the 1st complete Pre-MG occasion after the beginning of PCell’s DL slot (j+TBWPswitchDelay) + 5ms as defined in clause 8.19.2.

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

-	1.6s for a UE supporting power class 1 and 5,

-	0.96s for a UE supporting power class 2, 3 and 4

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Y ms from the beginning of time period T4, where Y is

-	3.2s for a UE supporting power class 1 and 5,

-	1.92s for a UE supporting power class 2, 3 and 4

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

End of Change 1