3GPP TSG-RAN WG4 Meeting # 104-e                                                        R4-2211926			R4-22xxxxx

# Electronic Meeting, August 15 – August 26, 2022

Source:	China Unicom

Title:	Views on lower MSD for Inter-band CA

Agenda Item:	11.6.4

Document for:	Approval

- 1	Introduction

- In RAN #96 meeting,  new WID [1] was approved for UE FR1 requirement focus on evolution for potential RAN4 enhancements for NR frequency range 1, in which the main objectives are:

- Enable 4Tx on a single carrier for CPE/FWA/vehicle/industrial devices

- Enable 8Rx and for CPE/FWA/vehicle/industrial device

- Investigate the feasibility of lower MSD for inter-band CA/EN-DC/DC combinations

The detailed objective for the scope of lower MSD is quoted below:

Investigate the feasibility of lower MSD for inter-band CA/EN-DC/DC combinations [RAN4]

- Select a limited set of band combinations (2-4 combinations) to cover all types of MSD (harmonic, harmonic mixing, IMD and cross band isolation)

- Study how the MSD performance can be improved for the example band combinations

- Study of MSD improvement with different MSD sources (harmonics, IMD2/3/4/5, cross band isolation and harmonic mixing)

- Study the feasibility of and options for allowing a UE to signal improved lower MSD performance capability for combinations where MSD is allowed

- Aim to conclude the study phase by RAN#99, and further discuss in RAN#99 how to handle the objective based on the study progress.

In this paper, we share our initial considerations on lower MSD for Interband-CA combinations.

- 2 Discussion

- Band Combinations

- Many NR networks have been launched using mid-band spectrum, as it serves a combination of coverage and capacity.

- For FDD bands,  there are 38 operators that have been investing in 5G at 2100 MHz and 37 of those are known to be deploying 5G in band n1. It has been recorded that 21 operators invested in 5G in band n3, including 18 that are actively deploying 5G in band n3, with many operators planning to refarm this spectrum for 5G deployment. For TDD bands, there had been 150 operators identified as actively deploying or having launched 5G networks using bands n77 or n78 [2].

- From the specification TS38.101-1, it is found that the combinations of n1+n78 and n3+n78 are facing the following reference sensitivity exceptions due to UL harmonic interference and intermodulation interference:

Table 7.3A.4-1: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC3 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n3 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n78 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| NOTE 2: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

Table 7.3A.5-1: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n78 | n1 | 1950 | 5 | 25 | 2140 | 8.0 | FDD | IMD4 |
|  |  |  |  |  |  | 10.75 |  |  |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
| CA_n3-n78 | n3 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD24 |
|  |  |  |  |  |  | 28.75 |  |  |
|  | n78 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD44 |
|  |  |  |  |  |  | 10.75 |  |  |
|  | n78 | 3435 | 10 | 25 | 3435 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1877.5 | 2.2 | FDD | IMD7 |
|  | n78 | 3305 3780 | 10 10 | 1 (RBstart=3) 1 (RBstart=0) | 3305 3780 | N/A | TDD | N/A |

Table 7.3A.5-1a: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n78 | n1 | 1950 | 5 | 25 | 2140 | [17.8] | FDD | IMD4 |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
| CA_n3-n78 | n3 | 1740 | 5 | 25 | 1835 | 31.9 | FDD | IMD2 |
|  | n78 | 3575 | 10 | 50 | 3575 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 18.5 | FDD | IMD4 |
|  | n78 | 3435 | 10 | 50 | 3435 | N/A | TDD | N/A |

- It is also observed that MSD comes from IMD is higher for PC2 than for PC3.

- Proposal 1: Considering the commercial deployment scale and MSD types, it is proposed to have CA_n1A-n78A and CA_n3A-n78A as example band combinations for the lower MSD investigation.

- Proposal 2: Consider both PC3 and PC2 for investigation of lower MSD based on intermodulation.

- UE signalling & feasibility

- Once the improved MSD capabilities are completed, signalling would be needed to inform the gNB about the situation. In order to minimize the impacts to legacy UE and network scheduling, the reported improved MSD capability should be optional. In addition, it needs to be clarified what behaviors are expected from network side after the signalling is received.

- Proposal 3: The improved MSD capability signalling should be an optional feature.

- Proposal 4: It needs to be clarified what are the expected behaviour for gNB after receiving the improved MSD information from UE.

- 3	Conclusion

In this paper, we provided our initial considerations on the example band combinations and feasibility of UE signalling report. The following proposals are made:

- Proposal 1: Considering the commercial deployment scale and MSD types, it is proposed to have CA_n1A-n78A and CA_n3A-n78A as example band combinations for the lower MSD investigation.

- Proposal 2: Consider both PC3 and PC2 for investigation of lower MSD based on intermodulation.

- Proposal 3: The improved MSD capability signalling should be an optional feature.

- Proposal 4: It needs to be clarified what are the expected behaviour for gNB after receiving the improved MSD information from UE.

- 4	Reference

- [1] RP-220987 New WID: Further RF requirements enhancement for NR frequency range 1 (FR1)	Moderator (Huawei, HiSilicon)

- [2] Mid-Band-Spectrum June 2022 Summary-Report		https://gsacom.com/paper/mid-band-spectrum-june-2022-summary-report/		GSA 2022.