3GPP TSG-RAN WG4 Meeting # 104-e                                                        		R4-2212004

Electronic Meeting, 15th April – 26th August, 2022

Source:	Vodafone

Title:	TP for TR 36.718-06-02 to add CA_1A-3A-8A-41A-41A

Agenda item:	12.1.3.2

Document for:	Approval

# Background

This contribution provides text proposal for the LTE CA configuration CA_1A-3A-8A-41A-41A with UL configurations CA_1A-3A, CA_1A-8A, CA_1A-41A, CA_3A-8A, CA_3A-41A and CA_8A-41A as proposed in [1].

# Reference

[1] RP-221831 Rel-18 LTE-A inter-band CA for x bands (x<= 6) DL with y bands (y=1, 2) UL

# Text Proposal

##### ---Start of changes---

### 5.3.x	CA_1A-3A-8A-41A-41A

#### 5.3.x.1	Channel bandwidths per operating band for CA

Table 5.3.x.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-8A-41A-41A | CA_1A-3A CA_1A-8A CA_1A-41A CA_3A-8A CA_3A-41A CA_8A-41A | 1 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 3 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 41 | See CA_41A-41A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |

#### 5.3.x.2	Co-existence studies

Coexistence requirements for CA_1-3, CA_1-8, CA_1-41, CA_3-8, CA_3-41 and CA_8-41 already exist in TS 36101.

#### 5.3.x.3	∆TIB and ∆RIB values

The following ∆TIB and ∆RIB values are drawn from CA_1-3-8-41 in TS 36101:

Table 6.X.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-3-8-41-41 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 8 | 0.3 |
|  | 41 | 0.35 |
|  |  | 0.86 |
| NOTE 5: The requirement is specified for the frequency range of 2545-2690MHz. NOTE 6: The requirement is specified for the frequency range of 2496-2545MHz. |  |  |

Table 6.X.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1-3-8-41-41 | 1 | 0 |
|  | 3 | 0 |
|  | 8 | 0 |
|  | 41 | 05 |
|  |  | 0.56 |
| NOTE 5: The requirement is specified for the frequency range of 2545-2690MHz. NOTE 6: The requirement is specified for the frequency range of 2496-2545MHz. |  |  |

#### 5.3.x.4	REFSENS requirements

No additional requirements necessary.

##### ---End of changes---