3GPP TSG-RAN WG4 Meeting # 104-e	R4-2212037

# Electronic Meeting, August 15 – August 26, 2022

Title: 	Discussion on the remaining issues of R17 NR redcap

Agenda Item:	9.18.3.1.1

Source: 	OPPO

Document for:	Approval

Introduction

In RAN4#103-e meeting, a WF was approved for R17 NR redcap UE [1]. In this document, we further discuss the remaining issues.

Discussion

Following highlighted text to capture in the IDLE mode CR was discussed but not agreed. Interested companies may provide their view at next meeting: “If the UE is not configured with eDRX_IDLE cycle and the UE has evaluated according to Table 4.2B.2.2-1 for 1 Rx RedCap or Table 4.2.2.2-1 for 2 Rx RedCap in Nserv_RedCap consecutive DRX cycles that the serving cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting UE measurement activities. In this case the UE shall not relax measurements on any of the neighbor cells even if the UE is configured with any relaxed measurement criterion and has fulfilled that criterion.”

In our view, the requirements for power saving when the UE is not configured for eDRX should also apply. Whether UE is configured with eDRX_IDLE cycle is decoupled with whether UE shall meet relaxed measurement requirements on neighbour cells. But it depends on network configurations of relaxed measurement criterion and whether UE has fulfilled corresponding criterion. Therefore, we think it is not needed to add such restrictions for the case if the UE is not configured with eDRX_IDLE cycle.

Observation 1: The requirements for power saving when the UE is not configured for eDRX should also apply.

Proposal 1: No need to add restrictions of relaxed measurements for the case if the UE is not configured with eDRX_IDLE cycle.

Conclusion

Observation 1: The requirements for power saving when the UE is not configured for eDRX should also apply.

Proposal 1: No need to add restrictions of relaxed measurements for the case if the UE is not configured with eDRX_IDLE cycle.

Reference

[1] R4-2210592, WF on RedCap RRM requirements, Ericsson