3GPP TSG-RAN WG4 Meeting # 104-e			                                   R4-2212047

Electronic Meeting, August 15 – August 26, 2022

Agenda item:	9.19.2.2.3

Source: 	OPPO

Title: 	Simulation results for PRS-RSRPP

Document for:	Discussion

Introduction

In the RAN4 #103-e meeting, it was agreed to define accuracy requirements for PRS-RSRPP measurements and the interested companies can provide simulation results based on the following assumptions [1][2]. This contribution will give our simulation results.

Issue 4-4-2: Conditions/scenarios for PRS-RSRPP measurement accuracy Agreements: - RAN4 to conduct link level simulations to evaluate the baseband performance of PRS-RSRPP measurement. - The parameters for evaluation - Channel model: further down-select the following two options in this meeting - Option 1: Modified TDL-D - Option 2: Two-tap channel - Use realistic TOA estimation in the simulation for PRS-RSRPP. - Use the following Es/Iot and PRS BW conditions at least for the simulation of PRS-RSRPP accuracy requirements. - Set 1: Es/Iot >= -13dB, BW >= 24/52/104 RBs, 4 sample - Set 2: Es/Iot >= -6dB, BW >= 52/104 RBs, 1 sample - FFS: ideal PRS-RSRPP definition. Issue 4-4-3: Link simulation assumptions for PRS-RSRPP measurement accuracy Agreements: - Revise the simulation assumption in R4-2209234 based on the agreements in issue 4-4-3.

Discussion

During the last meeting discussion, there are two leftover issues for RSRPP simulation assumptions. In our simulation, the ideal RSRPP is derived by the following formula based on the perfect channel acknowledge and perfect TOA of first path. Modified TDL-D channel is considered, where delay spread DS=30ns, K-factor Kdesired=0dB, and the path power and delay are adjusted following the procedure described in clause 7.7.6 of TR 38.901[3]. Our simulation results are shown in the Table below.

RSRPPp∝kHkexpj2πDpkNIFFT2

Table 1. simulation results for PRS-RSRPP with modified TDL-D channel

| SCS (kHz) | BW (RB) | PRS_Lenth PerSlot | 1-sample |  | 4-sample |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SINR=-6dB | SINR=-13dB | SINR=-6dB | SINR=-13dB |
| 15 | 24 | 1 | [-9.49, 1.01] | [-5.12, 2.62] | [-7.25, 0.64] | [-4.05, 1.89] |
|  |  | 2 | [-4.76, 0.84] | [-4.87, 2.21] | [-4.20, 0.56] | [-4.0, 1.72] |
|  | 52 | 1 | [-3.21, 0.91] | [-5.55, 2.68] | [-2.88, 0.53] | [-4.38, 1.95] |
|  |  | 2 | [-1.12, 0.73] | [-4.15, 2.18] | [-2.15, 0.45] | [-3.28, 1.72] |
|  | 104 | 1 | [-1.75, 0.56] | [-6.16, 1.59] | [-2.62, 0.33] | [-5.42, 1.02] |
|  |  | 2 | [-4.71, 0.45] | [-3.37, 1.19] | [-3.10, 0.29] | [-2.72, 1.03] |
|  | 268 | 1 | [-6.05, 0.51] | [-5.29, 0.98] | [-5.25, 0.28] | [-4.88, 0.69] |
|  |  | 2 | [-5.15, 0.29] | [-4.63, 1.04] | [-4.73, 0.19] | [-4.21, 0.68] |
| 30 | 24 | 1 | [-9.95, 1.01] | [-4.60, 2.67] | [-7.39, 0.63] | [-3.81, 1.87] |
|  |  | 2 | [-3.78, 0.88] | [-4.92, 2.27] | [-3.27, 0.56] | [-3.93, 1.72] |
|  | 48 | 1 | [-3.89, 0.89] | [-4.75, 2.57] | [-3.21, 0.62] | [-3.58, 1.88] |
|  |  | 2 | [-3.19, 0.77] | [-4.05, 2.49] | [-3.11, 0.56] | [-3.06, 2.22] |
|  | 132 | 1 | [-5.08, 0.60] | [-5.49, 1.54] | [-4.79, 0.36] | [-4.98, 0.98] |
|  |  | 2 | [-5.05, 0.47] | [-5.26, 1.35] | [-4.98, 0.33] | [-4.24, 1.05] |
|  | 272 | 1 | [-7.22, 0.39] | [-6.55, 0.96] | [-6.80, 0.21] | [-5.90, 0.65] |
|  |  | 2 | [-7.18, 0.28] | [-6.20, 0.75] | [-7.25, 0.17] | [-5.59, 0.52] |
| 120 | 32 | 1 | [-9.58, 0.91] | [-5.59, 2.23] | [-7.74, 0.52] | [-4.88, 1.50] |
|  |  | 2 | [-8.88, 0.71] | [-5.85, 1.84] | [-6.91, 0.45] | [-4.63, 1.41] |
|  | 64 | 1 | [-9.79, 0.62] | [-8.12, 1.45] | [-6.88, 0.35] | [-7.47, 0.96] |
|  |  | 2 | [-0.54, 0.44] | [-7.98, 1.27] | [-3.60, 0.29] | [-6.89, 0.88] |
|  | 128 | 1 | [-12.88, 0.40] | [-10.56, 0.96] | [-8.63, 0.22] | [-9.18, 0.59] |
|  |  | 2 | [-0.34, 0.31] | [-10.81, 0.78] | [-3.72, 0.18] | [-8.94, 0.53] |
| Note1: PRS_LenthPerSlot = (DL_PRS_NumSymbols x DL_PRS_ResourceRepetitionFactor) /DL_PRS_CombSizeN |  |  |  |  |  |  |

Conclusion

In this contribution, our simulation results on PRS-RSRPP measurement accuracy are given.

- References

- R4-2210600, WF on NR Positioning Enhancements (Part 1), Ericsson

- R4-2209234, Discussion on accuracy requirements and TCs for PRS-RSRPP, Huawei, HiSilicon

- TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz