3GPP TSG-RAN WG4 Meeting # 104-e	R4-2212050

Electronic Meeting, August 15 – August 26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev |  | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to Test sets 4-9: PRS-RSRPP accuracy test case with reduced number of samples in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Perf |  |  |  |  |  |  | Date: |  | 2022-08-10 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | PRS-RSRPP measurement requirements were defined and the test cases for PRS-RSRPP accuracy should be introduced according to the work split in R4-2210485. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the following test case for PRS-RSRPP accuracy requirements. Test set 4-9: PRS-RSRPP accuracy test case with reduced number of samples in FR1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The accuracy requirements cannot be verified in the realistic environment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) A.6.7.X.Y1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.533 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# < Start of change 1>

A.6.7.X.Y1	SA: measurement accuracy with reduced PRS samples in FR1

A.6.7.X.Y1.1	Test Purpose and Environment

The purpose of this test is to verify that the PRS-RSRPP measurement accuracy with reduced PRS samples is within the specified limits. This test will verify the requirements in clauses [10.1.24.Z.1] and [10.1.24.Z.2].

The UE under test should support [M-sample measurements], and the TE indicates the UE to perform positioning measurements with reduced number of samples. Since the PRS bandwidth is not within the active BWP, Nsample= 2 is assumed.

A.6.7.X.Y1.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Supported test configurations are shown in table A.6.7.X.Y1.2-1. Both absolute and relative accuracy of PRS-RSRPP measurements are tested by using the parameters in A.6.7.X.Y1.2-2. In all test cases, Cell 1 is the PCell.

Table A.6.7.X.Y1.2-1: PRS-RSRPP supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table A.6.7.X.Y1.2-2: PRS-RSRPP test parameters

| Parameter |  |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 |  | Cell 2 |
| Cell ID |  |  |  | 489 |  | 0 |
| SSB ARFCN |  |  |  | freq1 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |
|  |  | Config 2,3 |  | TDD |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |
| BWP BW |  | Config 1 |  | 10: NRB,c = 52 |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  | NA |
|  |  | Config 2 |  | TRS.1.1 TDD |  | NA |
|  |  | Config 3 |  | TRS.1.2 TDD |  | NA |
| DRX Cycle |  |  | ms | Not Applicable |  |  |
| Measurement gap |  |  |  | GP#24 or GP#0 Note 7 |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  | - |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  | - |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |
| Control channel RMC |  | Config 1 |  | CCR.1.1 FDD |  | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  |  |
|  |  | Config 3 |  | CCR2.1 TDD |  |  |
| PRS configuration |  | Config 1 |  | PRS.1.4 FR1 |  | PRS.1.4 FR1 |
|  |  | Config 2 |  | PRS.1.4 FR1 |  | PRS.1.4 FR1 |
|  |  | Config 3 |  | PRS.2.4 FR1 |  | PRS.2.4 FR1 |
| PRS Resource slot offset (slot) |  | Config 1,2,3 | slot | 0 |  | 4 |
| SSB configuration |  | Config 1 |  | SSB.1 FR1 |  | SSB.1 FR1 |
|  |  | Config 2 |  | SSB.1 FR1 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.2 FR1 |  | SSB.2 FR1 |
| Time offset with Cell 1 |  | Config 1 | ms | - |  | 3 |
|  |  | Config 2,3 | s | - |  | 3 |
| SMTC configuration |  | Config 1 |  | SMTC.2 |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | dBm/15KhZ | -106 |  |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H |  | Not applicableNote 5 |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -106 |  |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H |  | Not applicableNote 5 |  |  |
|  |  |  | dB | 2.46 | -5.97 |  |
|  |  |  | dB | 6 | 1 |  |
| PRS-RSRP Note3 | Config 1, 2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | dBm/SCS | -100 |  | -105 |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H |  | Not applicable Note 5 |  | Not applicable Note 5 |
| IoNote3 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | dBm/9.36MHz | -70.09 |  |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | dBm/38.16MHz | Not applicable Note 5 |  |  |
| Propagation condition |  |  |  | [Modified TDL-D or Two-tap channel] |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Subtest 1 is not used when testing with 30kHz SSB SCS. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification Note 7: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |  |  |

A.6.7.X.Y1.3	Test Requirements

In each test, the absolute PRS-RSRPP measurement for each cell shall fulfil the absolute accuracy requirement in clause [10.1.24.Z.1]. The relative PRS-RSRPP measurement between the two PRS resources within the same cell shall fulfil the relative accuracy requirement in clause [10.1.24.Z.2].

# < End of change 1>