3GPP TSG-WG RAN4 Meeting #104-e	R4-2212068

# Online, 15th – 26th Auguest, 2022

Source: 				Nokia, Nokia Shanghai Bell

Title: 				TP on System parameters for APT600

Agenda Item: 	10.19.2

Document for:	Approval

# Introduction

A new WI to introduce a 600 MHz band for APT region was approved in RAN#96 [1]. The feasibility study of the band has been already completed as reported in [2]. The band plan to specify is based on option B1 with a single FDD duplexer 2x40 MHz for 663-703 MHz uplink and 612-652 MHz.

In this contribution band definition and system parameters are discussed.

# Discussion

Band number 105 has been already reserved for LTE_TDD_1670_1675MHz. n106 is the next available band number for a new NR band. However, there are unused band numbers reserved for E-UTRA TDD bands below 64, which may be used instead of 105 for LTE_TDD_1670_1675MHz.

It is noted that there is also a WI LTE_terr_bcast_bands_part2, which needs a new band number assignment. It is proposed this WI NR_600MHz_APT to take the next available band number (n105 or n106) as LTE_terr_bcast_bands_part2 may need more discussion how many bands are specified. The band definition is proposed in Table 1.

Table 1: NR operating band for APT600 band

| Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n105/n106 | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

Proposal 1: The operating band number for APT600 is n105 or n106 depending on the decision on WI LTE_terr_bcast_bands_part2.

In the WID scope [1], the channel bandwidth and subcarrier spacing are according to the following.

Table 2: Channel bandwidth and SCS parameters for APT600 band

| SCS (kHz) | channel bandwidth (MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 5 | 10 | 15 | 20 | 25 | 30 | 35 |  |
| 30 |  | 10 | 15 | 20 | 25 | 30 | 35 |  |

The system parameter should be aligned with n71 and other low frequency bands <2.2GHz. Specifically, channel raster is proposed to be based on 100 kHz. ARFCN range is defined according to the following table.

Table 3: Applicable NR-ARFCN for APT600 band

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n105/n106 | 100 | 132600 – <20> – 140600 | 122400 – <20> – 130400 |

Proposal 2: The channel raster is 100 kHz based and NR-ARFCN is 132600 – <20> – 140600 for uplink and 122400 – <20> – 130400 for downlink.

Then, for sync raster, SSB SCS is proposed to be 15kHz and pattern A, as presented in Table 4. The GSCN range is 1535-<1>-1624.

Table 4: Applicable SS raster entries for APT600 band

| NR operating band | SS Block SCS | SS Block pattern (NOTE 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n105/n106 | 15 kHz | Case A | 1535 – <1> – 1624 |

Proposal 3: SSB SCS is 15 kHz with pattern A. Range of GSCN is 1535 – <1> – 1624.

# Conclusion

Proposal 1: The operating band number for APT600 is n105 or n106 depending on the decision on WI LTE_terr_bcast_bands_part2.

Proposal 2: The channel raster is 100 kHz based and NR-ARFCN is 132600 – <20> – 140600 for uplink and 122400 – <20> – 130400 for downlink.

Proposal 3: SSB SCS is 15 kHz with pattern A. Range of GSCN is 1535 – <1> – 1624.

A text proposal to the TR is attached in Annex.

# Reference

[1] RP-221778 New WID on APT 600 MHz NR band

[2] TR 38.860 Study on extended 600MHz NR band

ANNEX: Text proposal to TR 38.XXX

# 8	Channel numbering for NR

## 8.1	System Parameters for APT600 MHz

NR band APT600 is designed to operate in the operating band defined in Table 8.1.

Table 8.1: NR operating band for APT600 band

| Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n105/n106 | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

The requirements in the TR apply to the combination of channel bandwidths, SCS and operating bands shown in Table 8.2.

Table 8.2: Channel bandwidth and SCS parameters for APT600 band

| SCS (kHz) | channel bandwidth (MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 5 | 10 | 15 | 20 | 25 | 30 | 35 |  |
| 30 |  | 10 | 15 | 20 | 25 | 30 | 35 |  |

The channel raster of APT600 band is 100 kHz and the applicable entries are defined in Table 8.3

Table 8.3: Applicable NR-ARFCN for APT600 band

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n105/n106 | 100 | 132600 – <20> – 140600 | 122400 – <20> – 130400 |

The synchronization raster for APT600 band is give in Table 8.4.

Table 8.4: Applicable SS raster entries for APT600 band

| NR operating band | SS Block SCS | SS Block pattern (NOTE 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n105/n106 | 15 kHz | Case A | 1535 – <1> – 1624 |