3GPP TSG-RAN4 Meeting #104-e	R4-2212099

Aug 2022

Title:	BS requirements for 5G terrestrial broadcast

Source:	TDF

Agenda item:	12.4.1

Release:	Rel-18

WI: 	LTE_terr_bcast_bands_part2-Core

Lead WG: 	RAN1

Document for:	Approval

# 1.	Introduction

New RAN4 lead WI [1] was approved with the BS RF objective as follows:

- For MBMS-dedicated cells:

  - Specify band(s), within the portion of UHF spectrum allocated to broadcast systems (~470 - ~694/698 MHz, depending on the region), that support the channelization available to broadcasting operators [RAN4].

    - RAN4 to define the exact band plan suitable for broadcasting operators’ spectrum allocation

    - RAN4 to define REFSENS for this band(s)

  - For BS the requirements for signal quality, occupied bandwidth, ACLR, unwanted emissions shall be applied as provided by the corresponding regulatory bodies in the different regions for 6/7/8 MHz, as applicable. The specifications shall contain corresponding references [RAN4].

  - …

NOTE 1:	This work item shall not affect any functionality of unicast LTE, it targets only MBMS-			dedicated cells.

NOTE 2: 	RAN4 shall assume that coexistence among different systems in the portion of the UHF band 		allocated to broadcast (~470 - ~694/698 MHz) is ensured through coordination, in line with 		regional and national regulation.

WID [1] has other objectives for UE RF requirements, but this document only addresses band definition and BS requirements.

# 2. 	Discussion

TDF [2] is a French Broadcast network operator and is member of Broadcast Network Europe (BNE [3]).  BNE represents Europe’s terrestrial network operators in Europe and internationally. Terrestrial broadcast network operators are responsible for managing, operating and maintaining the infrastructure that brings TV, radio and other wireless and over-the-air services to homes, cars, and other users.

TDF and BNE have been contributing to the specification of LTE based 5G Terrestrial Broadcasting (in brief 5G Broadcast) from the very first days of the EnTV study item in 2015.

5G Broadcast has the potential to become a global solution enabling the delivery of linear media services to mobile devices and connected vehicles. The UHF band spectrum can be used to operate 5G Broadcast, and the networks deployment can leverage existing HPHT broadcast terrestrial network infrastructure.

The 5G Broadcast system enables the deployment of a downlink-only 3GPP network, independently from any uplink capability.  A new band type needs to be defined in 3GPP to address this new concept, beyond e.g. SDL.

Proposal 1:	RAN4 to define a new band type for independent downlink-only transmissions.

At ITU-R all three ITU Regions allocate the band 470-694/698 MHz to the broadcasting service, on a primary basis. Regional variations further apply to this band :

- Spectrum usage is subject to general ITU-R rules applicable for the broadcasting service and corresponding regulatory frameworks for the different Regions – such as the GE06 Agreement for Region 1 [4].

- 6, 7 or 8 MHz carrier bandwidth is used depending on the ITU Region

- The upper limit of the UHF band allocated to the broadcast service depends on used carrier bandwidth: the band 470 – 694 MHz can accommodate respectively 32 and 28 channels for 7 and 8 MHz bandwidth, while the band 470 – 698 MHz can accommodate 38 channels of 6 MHz bandwidth.

Proposal 2:	RAN4 to define a band for LTE based 5G Terrestrial Broadcast covering the spectrum range 		470 – 698 MHz to be used in terms of 6, 7 or 8 MHz carrier bandwidth.

Operating stations of the broadcasting services is governed by the respective rules and sharing / compatibility criteria of the regional regulatory arrangements, and coordination between affected administrations.

5G Broadcast networks will be deployed alongside existing DTT transmissions. Compatibility from a technical perspective can be achieved by employing the parameters defined in corresponding documents on which regulatory frameworks and negotiations are based in different ITU Regions.

- In ITU Region 1 operation of transmitters of the broadcasting service are governed by the Technical Annexes of the GE06 Agreement [4] and ETSI specification ETSI EN 302 296 [5].

- In ITU Region 2, the relevant documents are offered by corresponding national regulators such as, for example FCC in the US [6] and Anatel in Brazil [7].

- In ITU Region 3 also national regulation is applied in coordination and negotiation between affected administrations, such as [8] in China.

Proposal 3:	RAN4 to re-use BS requirements as provided in existing regulatory agreements 				and documentation provided by the ITU and national regulators for the broadcasting service.

# Summary

Based on the discussion the following is proposed to achieve the objectives of the WI:

Proposal 1:	RAN4 to define a new band type for independent downlink-only transmissions.

Proposal 2:	RAN4 to define a band for LTE based 5G Terrestrial Broadcast covering the spectrum range 		470 – 698 MHz to be used in terms of 6, 7 or 8 MHz carrier bandwidth.

Proposal 3:	RAN4 to re-use BS requirements as provided in existing regulatory agreements 				and documentation provided by the ITU and national regulators for the broadcasting service.

# References

[1]	RP-220518, “Revised WID: New bands and bandwidth allocation for LTE based 5G terrestrial broadcast - part 2”, EBU, Qualcomm., 3GPP TSG RAN Meeting #95e, Electronic Meeting, March 17 - 23, 2022

[2] www.tdf.fr

[3] https://broadcast-networks.eu

[4] https://www.itu.int/pub/R-ACT-RRC.14-2006/en

[5] https://www.etsi.org/deliver/etsi_en/302200_302299/302296/02.02.00_20/en_302296v020200a.pdf

[6] Title 47 CFR 73.622, Digital television table of allotments, FCC, United States

[7] ABNT 15601, NORMA BRASILEÑA, Televisión digital terrestre — Sistema de transmisión ISDB-Tb, Anatel, Brazil

[8] GB20600-2006 [8], Framing structure, channel coding and modulation for digital television terrestrial broadcasting system, National Radio and Television Standardization Technical Committee, People’s Republic of China