3GPP TSG-RAN WG4 Meeting #104-e 	R4-2212101

Electronic Meeting, 15-26 Aug., 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation Results on PDSCH demodulation and CQI requirements for inter-cell interference MMSE-IRC

Agenda item:	9.10.1.1

Document for:	Information

Introduction

In this contribution simulation results on CQI reporting requirements in inter-cell interference using MMSE-IRC receiver are presented. The simulation assumptions are as follows:

Simulation Results for CQI reporting requirements

Simulation Parameters

Table 1 Simulation parameters for CQI reporting requirements

| Parameter |  | Unit | Serving cell FDD | Serving Cell TDD |  | Interferer cell |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 | 40 |  | Same as serving cell |
| Subcarrier spacing |  | kHz | 15 | 30 |  | Same as serving cell |
| Duplex Mode |  |  | FDD | TDD |  | Same as serving cell |
| Propagation channel |  |  | TDLA30-5 |  |  | Static |
| Antenna configuration |  |  | 2×2, 2x4 |  |  | 1x2, 1x4 |
| Correlation configuration |  |  | ULA low |  |  |  |
| Beamforming Model |  |  | As specified in Annex B.4.1 |  |  | NA |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |  | Not configured |
|  | CDM Type |  | FD-CDM2 |  |  | Not configured |
|  | Density (ρ) |  | 1 |  |  | Not configured |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,4 |  |  | Not configured |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 |  |  | Not configured |
|  | CSI-RS periodicity and offset | slot | 5/1 |  | 10/1 | Not configured |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 2 |  |  | 1 |
|  | CDM Type |  | FD-CDM2 |  |  | No-cdm |
|  | Density (ρ) |  | 1 |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 3,(6) |  |  | Row 3,(6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 |  |  | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 5/1 | 10/1 |  | Same as serving cell |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic |  |  | Not configured |
|  | CSI-IM RE pattern |  | 0 |  |  | Not configured |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4, 9) |  |  | Not configured |
|  | CSI-IM timeConfig periodicity and offset | slot | 5/1 | 10/1 |  | Not configured |
| ReportConfigType |  |  | Periodic |  |  | Not configured |
| CQI-table |  |  | Table 2 |  |  | Not configured |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |  | Not configured |
| timeRestrictionForChannelMeasurements |  |  | Not configured |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |  |  | Not configured |
| pmi-FormatIndicator |  |  | Wideband |  |  | Not configured |
| Sub-band Size |  | RB | 8 | 16 |  | Not configured |
| Csi-ReportingBand |  |  | 1111111 |  |  | Not configured |
| CSI-Report periodicity and offset |  | slot | 5/0 |  |  | Not configured |
| aperiodicTriggeringOffset |  |  | Not configured |  |  | Not configured |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |  |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |  |  | 1 |
|  | (CodebookConfig-N1, CodebookConfig-N2) |  | Not configured |  |  | Not Configured |
|  | CodebookSubsetRestriction |  | 010000 |  |  | Not configured |
|  | RI Restriction |  | N/A |  |  | Not configured |
| Physical channel for CSI report |  |  | PUCCH |  |  | Not configured |
| CQI/RI/PMI delay |  | ms | 8 | 9.5 |  | Not configured |
| Maximum number of HARQ transmission |  |  | 1 |  |  | 1 |
| Measurement channel |  |  | As specified in Table A.4-2, TBS.2-1 |  |  |  |
| INR |  | dB | N/A |  |  | 10.04 |

Simulation Results for CQI reporting requirements

Throughput ratios obtained using simulations at -2dB SINR are presented below

Table 2 Throughput ratio at -2 dB SINR

| Duplex method | Throughput ratio 2Tx 2Rx | Throughput ratio 2Tx 4Rx |
| --- | --- | --- |
| TDD | 2.15 | 2.05 |
| FDD | 3.05 | 2.87 |

- References

- R4-2210681 - WF on PDSCH demodulation and CSI requirements for inter-cell interference MMSE-IRC