3GPP TSG-RAN WG4 Meeting # 104-e	R4-2212109

Electronic Meeting, August 15-26, 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	PDCCH_PBCH simulation results for the extension to 71 GHz

Agenda item: 	9.14.8.2.2

Document for: 	Discussion

Introduction

This paper presents PDCCH and PBCH simulation results for the definition of demodulation requirements related to the operation between 52.6 GHz and 71 GHz.

PDCCH Simulation Results

Table 1 Results for PDCCH

| CBW/ SCS | CORESET duration | CORESET RB | Interleaver size | REG bundle size | Aggregation level | Propagation Condition | DCI format | Information Bit | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Pm dsg (%) | SNRBB (dB) |
| 100/120 | 1 | 60 | 3 | 2 | 2 | TDLA10-200 | 1_0 | 40 | 1x2 Low | 1 | 5.80 |
| 100/120 | 1 | 60 | 2 | 6 | 4 | TDLA10-200 | 1_1 | 56 | 1x2 Low | 1 | -0.56 |
| 100/120 | 1 | 60 | 3 | 2 | 8 | TDLA10-200 | 1_1 | 56 | 2x2 Low | 1 | -2.72 |
| 100/120 | 2 | 60 | 3 | 2 | 16 | TDLA10-200 | 1_0 | 40 | 2x2 Low | 1 | -7.27 |
| 400/480 | 1 | 60 | 2 | 6 | 4 | TDLA10-200 | 1_1 | 56 | 1x2 Low | 1 | 1.05 |
| 400/480 | 2 | 60 | 3 | 2 | 16 | TDLA10-200 | 1_0 | 40 | 2x2 Low | 1 | -5.78 |

PBCH Simulation Results

Table 2 Test parameters for PBCH

| Parameter | Unit | Single antenna port |
| --- | --- | --- |
| Physical Cell ID |  | 0 |
| Cyclic prefix |  | Normal |
| Number of SS/PBCH blocks within an SS burst set periodicity |  | 1 |
| SS/PBCH block index Note1 |  | 0 |
| SS/PBCH block periodicity | ms | 20 |
| Note 1: as specified in clause 4.1 of TS 38.213 |  |  |

Table 3 Minimum performance PBCH in case SS/PBCH block index is not known

| Test number | Bandwidth (MHz) / Subcarrier spacing (kHz) | Reference channel | Propagation condition | Antenna configuration and correlation matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNRBB (dB) |
|  | 100 / 120 |  | TDLA10-200 | 1 x 2 Low | 1 | 1.88 |
|  | 400 / 480 |  | TDLA10-200 | 1 x 2 Low | 1 | 0.22 |
|  | 400 / 960 |  | TDLA10-200 | 1 x 2 Low | 1 | 2.38 |

Table 4 Minimum performance PBCH in case SS/PBCH block index is known

| Test number | Bandwidth (MHz) / Subcarrier spacing (kHz) | Reference channel | Propagation condition | Antenna configuration and correlation matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | PBCH SNR (dB) |
|  | 100 / 120 |  | TDLA10-200 | 1 x 2 Low | 1 | 1.94 |
|  | 400 / 480 |  | TDLA10-200 | 1 x 2 Low | 1 | 0.31 |
|  | 400 / 960 |  | TDLA10-200 | 1 x 2 Low | 1 | 2.44 |

References

- R4-2210665 - WF on UE demodulation performance requirements definition for 52.6 - 71 GHz

- R4-2210332 - Email discussion summary for [103-e][326] NR_exto71GHz_Demod_Part2

- draft_R4-2210332_Summary_326_2ndRound.docx

- TS 38.101-4, NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements