3GPP TSG-RAN WG4 Meeting #104-e	Draft_R4-2212110

# Electronic,  , 15th - 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | draftCR | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 38.101-4: PDCCH requiremeents for FR2-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Perf |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of the structure of the PDCCH requirements for FR2-2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposal for scheleton of PDCCH requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No PDCCH requirements for FR2-2 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3, 7.3.2.2, 7.3.2.2.1, 7.3.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is an updated version of the postponed draft CR R4-2208264 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Start of change 1>>

## 7.3	PDCCH demodulation requirements

The receiver characteristics of the PDCCH are determined by the probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg).

The parameters specified in Table 7.3-1 are valid for all PDCCH FR2-1 tests unless otherwise stated.

The parameters specified in Table 7.3-2 are valid for all PDCCH FR2-2 tests unless otherwise stated.

Table 7.3-1: Common test Parameters for FR2-1

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 1) |  |  | 0 |
| DL BWP configuration #1 | Cyclic prefix |  |  | Normal |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |
|  | SSB position in burst |  |  | First SSB in Slot #0 |
|  | SSB periodicity |  | ms | 20 |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | Each slot |
|  | Number of PDCCH candidates |  |  | 1 |
|  | Frequency domain resource allocation for CORESET |  |  | Start from RB = 0 with contiguous RB allocation |
|  | TCI state |  |  | TCI state #1 |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | 0 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | CSI-RS resource 1: 4 CSI-RS resource 2: 8 CSI-RS resource 3: 4 CSI-RS resource 4: 8 |
|  | Number of CSI-RS ports (X) |  |  | 1 |
|  | CDM Type |  |  | No CDM |
|  | Density (ρ) |  |  | 3 |
|  | CSI-RS periodicity |  | Slots | 160 |
|  | CSI-RS offset |  | Slots | 80 for CSI-RS resource 1 and 2 81 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |
|  | QCL info |  |  | TCI state #0 |
| NZP CSI-RS for beam refinement | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | 0 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | CSI-RS resource 1: 8 CSI-RS resource 2: 9 |
|  | Number of CSI-RS ports (X) |  |  | 1 |
|  | CDM Type |  |  | No CDM |
|  | Density (ρ) |  |  | 3 |
|  | CSI-RS periodicity |  | Slots | 120 kHz SCS: 160 for CSI-RS resource 1,2 |
|  | CSI-RS offset |  | Slots | 0 for CSI-RS resource 1,2 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4) *4 |
|  | Repetition |  |  | ON |
|  | QCL info |  |  | TCI state #1 |
| PDCCH & PDCCH DMRS Precoding configuration |  |  |  | Single Panel Type I, Random per slot with equal probability of each applicable i1, i2 combination, and with REG bundling granularity for number of Tx larger than 1 |
| TCI state #0 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type D |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type D |
| Symbols for all unused REs |  |  |  | OP.1 FDD as defined in Annex A.5.1.1 OP.1 TDD as defined in Annex A.5.2.1 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.3. |
| Note 1: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 [6] for tested channel bandwidth and subcarrier spacing. Note 2: The high layer parameter precoderGranularity equals to sameAsREG-bundle as defined in clause 7.4.1.3 of TS 38.211 [9] |  |  |  |  |

Table 7.3-2: Common test Parameters for FR 2-2

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 1) |  |  | 0 |
| DL BWP configuration #1 | Cyclic prefix |  |  | Normal |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |
|  | SSB position in burst |  |  | First SSB in Slot #0 |
|  | SSB periodicity |  | ms | 20 |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | See respective test parameter section. |
|  | Number of PDCCH candidates |  |  | 1 |
|  | Frequency domain resource allocation for CORESET |  |  | Start from RB = 0 with contiguous RB allocation |
|  | TCI state |  |  | TCI state #1 |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | 0 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | CSI-RS resource 1: 4 CSI-RS resource 2: 8 CSI-RS resource 3: 4 CSI-RS resource 4: 8 |
|  | Number of CSI-RS ports (X) |  |  | 1 |
|  | CDM Type |  |  | No CDM |
|  | Density (ρ) |  |  | 3 |
|  | CSI-RS periodicity |  | Slots | See respective test parameter section. |
|  | CSI-RS offset |  | Slots | See respective test parameter section. |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |
|  | QCL info |  |  | TCI state #0 |
| NZP CSI-RS for beam refinement | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | 0 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | CSI-RS resource 1: 8 CSI-RS resource 2: 9 |
|  | Number of CSI-RS ports (X) |  |  | 1 |
|  | CDM Type |  |  | No CDM |
|  | Density (ρ) |  |  | 3 |
|  | CSI-RS periodicity |  | Slots | See respective test parameter section. |
|  | CSI-RS offset |  | Slots | 0 for CSI-RS resource 1,2 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4) *4 |
|  | Repetition |  |  | ON |
|  | QCL info |  |  | TCI state #1 |
| PDCCH & PDCCH DMRS Precoding configuration |  |  |  | Single Panel Type I, Random per slot with equal probability of each applicable i1, i2 combination, and with REG bundling granularity for number of Tx larger than 1 |
| TCI state #0 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type D |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type D |
| Symbols for all unused REs |  |  |  | OP.1 FDD as defined in Annex A.5.1.1 OP.1 TDD as defined in Annex A.5.2.1 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.3. |
| Note 1: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 [6] for tested channel bandwidth and subcarrier spacing. Note 2: The high layer parameter precoderGranularity equals to sameAsREG-bundle as defined in clause 7.4.1.3 of TS 38.211 [9] |  |  |  |  |

### 7.3.1	1RX requirements

(Void)

### 7.3.2	2RX requirements

#### 7.3.2.1	FDD

(Void)

#### 7.3.2.2	TDD

The parameters specified in Table 7.3.2.2-1, 7.3.2.2-X1, 7.3.2.2-X2, 7.3.2.2-X3 are valid for all TDD tests unless otherwise stated.

Table 7.3.2.2-1: Test Parameters for FR2-1

| Parameter | Unit | 1 Tx Antenna | 2 Tx Antenna |
| --- | --- | --- | --- |
| TDD UL-DL pattern |  | FR2.120-1 |  |
| CCE to REG mapping type |  | Interleaved |  |
| REG bundle size |  | 2 for test 1-1 6 for test 1-2 | 2 |
| Interleaver size |  | 3 for test 1-1 2 for test 1-2 | 3 |
| Shift index |  | 0 |  |

Table 7.3.2.2-X1: Test Parameters with 120kHz for FR2-2

| Parameter |  | Unit | 1 Tx Antenna | 2 Tx Antenna |
| --- | --- | --- | --- | --- |
| TDD UL-DL pattern |  |  | FR2.120-1 |  |
| CCE to REG mapping type |  |  | Interleaved |  |
| REG bundle size |  |  | [2 for test 1-1 6 for test 1-2] | [2] |
| Interleaver size |  |  | [3 for test 1-1 2 for test 2-1] | [3] |
| Shift index |  |  | [0] |  |
| PDCCH configuration | Slots for PDCCH monitoring |  | Each slot |  |
|  | CSI Periodicity | Slots | [160] |  |
| CSI-RS for tracking | CSI-RS offset | Slots | [80 for CSI-RS resource 1 and 2 81 for CSI-RS resource 3 and 4] |  |
| NZP CSI-RS for beam refinement | CSI-RS periodicity | Slots | [160 for CSI-RS resource 1,2] |  |

Table 7.3.2.2-X2: Test Parameters with 480kHz for FR2-2

| Parameter |  | Unit | 1 Tx Antenna | 2 Tx Antenna |
| --- | --- | --- | --- | --- |
| TDD UL-DL pattern |  |  | [FR2.480-1] |  |
| CCE to REG mapping type |  |  | Interleaved |  |
| REG bundle size |  |  | [6] | [2] |
| Interleaver size |  |  | [2] | [3] |
| Shift index |  |  | [0] |  |
| PDCCH configuration | Slots for PDCCH monitoring |  | Every 4th slot |  |
|  | CSI Periodicity | Slots | [640] |  |
| CSI-RS for tracking | CSI-RS offset | Slots | [320 for CSI-RS resource 1 and 2 321 for CSI-RS resource 3 and 4] |  |
| NZP CSI-RS for beam refinement | CSI-RS periodicity | Slots | [640 for CSI-RS resource 1,2] |  |

Table 7.3.2.2-X3: Test Parameters with 960kHz for FR2-2

| Parameter |  | Unit | 1 Tx Antenna | 2 Tx Antenna |
| --- | --- | --- | --- | --- |
| TDD UL-DL pattern |  |  | [FR2.960-1] |  |
| CCE to REG mapping type |  |  | Interleaved |  |
| REG bundle size |  |  | TBD | TBD |
| Interleaver size |  |  | TBD | TBD |
| Shift index |  |  | TBD |  |
| PDCCH configuration | Slots for PDCCH monitoring |  | Every 8th slot |  |
|  | CSI Periodicity | Slots | [1280] |  |
| CSI-RS for tracking | CSI-RS offset | Slots | [640 for CSI-RS resource 1 and 2 641 for CSI-RS resource 3 and 4] |  |
| NZP CSI-RS for beam refinement | CSI-RS periodicity | Slots | [1280 for CSI-RS resource 1,2] |  |

##### 7.3.2.2.1	1 Tx Antenna performances

For the parameters specified in Table 7.3.2.2-1, 7.3.2.2-X1, 7.3.2.2-X2[, 7.3.2.2-X3], the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 7.3.2.2.1-1, 7.3.2.2.1-X1, 7.3.2.2.1-X2[, 7.3.2.2.1-X3]. The downlink physical setup is in accordance with Annex C.5.1.

Table 7.3.2.2.1-1: Minimum performance requirements with 120 kHz SCS for FR2-1

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNRBB (dB) |
| 1-1 | 100 | 60 | 1 | 2 | R.PDCCH. 5-1.1 TDD | TDLA30-75 | 1x2 Low | 1 | 6.4 |
| 1-2 | 100 | 60 | 1 | 4 | R.PDCCH. 5-1.2 TDD | TDLA30-300 | 1x2 Low | 1 | 3.0 |

Table 7.3.2.2.1-X1: Minimum performance requirements with 120 kHz SCS for FR2-2

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNRBB (dB) |
| [1-1] | 100 | 60 | 1 | 2 | [R.PDCCH. X1-1.Y1 TDD] | TDLA10-200 | 1x2 Low | 1 | TBD |
| [1-2] | 100 | 60 | 1 | 4 | [R.PDCCH. X1-1.Y2 TDD] | TDLA10-200 | 1x2 Low | 1 | TBD |

Table 7.3.2.2.1-X2: Minimum performance requirements with 480 kHz SCS for FR2-2

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNRBB (dB) |
| 1-1 | 400 | 60 | 1 | 4 | [R.PDCCH. X2-1.Y1 TDD] | TDLA10-200 | 1x2 Low | 1 | TBD |
| TBD | [1600] | TBD | TBD | TBD | TBD | TBD | 1x2 Low | TBD | TBD |

Table 7.3.2.2.1-X3: Minimum performance requirements with 960 kHz SCS for FR2-2

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNRBB (dB) |
| TBD | [400] | TBD | TBD | TBD | TBD | TBD | 1x2 Low | TBD | TBD |
| TBD | [2000] | TBD | TBD | TBD | TBD | TBD | 1x2 Low | TBD | TBD |

##### 7.3.2.2.2	2 Tx Antenna performances

For the parameters specified in Table 7.3.2.2-1, 7.3.2.2.-X1, 7.3.2.2.-X2[, 7.3.2.2.-X3], the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 7.3.2.2.2-1, 7.3.2.2.2-X1, 7.3.2.2.2-X2[, 7.3.2.2.2-X3]. The downlink physical setup is in accordance with Annex C.5.1.

Table 7.3.2.2.2-1: Minimum performance requirements with 120 kHz SCS for FR2-1

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNRBB (dB) |
| 2-1 | 100 | 60 | 1 | 8 | R.PDCCH. 5-1.3 TDD | TDLA30-75 | 2x2 Low | 1 | 0.1 |
| 2-2 | 100 | 60 | 2 | 16 | R.PDCCH. 5-2.1 TDD | TDLA30-75 | 2x2 Low | 1 | -3.0 |

Table 7.3.2.2.2-X1: Minimum performance requirements with 120 kHz SCS for FR2-2

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNRBB (dB) |
| 2-1 | 100 | 60 | 1 | 8 | [R.PDCCH. X1-1.Y1 TDD] | TDLA10-200 | 2x2 Low | 1 | TBD |
| 2-2 | 100 | 60 | 2 | 16 | [R.PDCCH. X1-2.Y1 TDD] | TDLA10-200 | 2x2 Low | 1 | TBD |

Table 7.3.2.2.2-X2: Minimum performance requirements with 480 kHz SCS for FR2-2

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNRBB (dB) |
| 2-1 | 400 | 60 | 2 | 16 | [R.PDCCH. X2-2.Y1 TDD] | TDLA10-200 | 2x2 Low | 1 | TBD |
| TBD | [1600] | TBD | TBD | TBD | TBD | TBD | 2x2 Low | TBD | TBD |

Table 7.3.2.2.2-X3: Minimum performance requirements with 960 kHz SCS for FR2-2

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNRBB (dB) |
| TBD | [400] | TBD | TBD | TBD | TBD | TBD | 2x2 Low | TBD | TBD |
| TBD | [2000] | TBD | TBD | TBD | TBD | TBD | 2x2 Low | TBD | TBD |

##### 7.3.2.2.3	Minimum requirements for power saving

During the test the UE shall monitor the DCI format 2_6 PDCCH in DRX off state and decide whether to receive the following PDCCH in DRX on period.

## << End of change 1 >>