3GPP TSG-RAN4 Meeting #104-e	R4-2212123

# Electronic meeting, 15th-26th, Aug., 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR to TS 38.133 TRP specific Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Perf |  |  |  |  |  | Date: |  |  | 2022-8-2 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In Rel-17, TRP specific Beam Failure Detection and Link Recovery test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode needs to be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add TRP specific Beam Failure Detection and Link Recovery Test case for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - TRP specific Beam Failure Detection and Link Recovery requirement will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | - A.7.5.x1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X | X | Test specifications |  |  |  | TS/TR 38.533 CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==========================Start of first change =============================

#### A.7.5.x1.1	TRP specific Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode

##### A.7.5.x1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects TRP specific SSB-based beam failure and link recovery in the sets q0,0 and q0,1 for TRP1 when no DRX is used for an FR2 serving cell requirements in clause 8.18.

The test parameters are given in Tables A.7.5.x1.1.1-1, A.7.5.x1.1.1-2, A.7.5.x1.1.1-3 and A.7.5.x1.1.1-4 below. There is one active serving cell configured with two TRPs in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.5.x1.1.1-1 shows the variation of the downlink SNR of the SSB in set q0,0 and q1,0 for TRP1 and TRP2 respectively. Figure A.7.5.x1.1.1-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q0,1  for TPR1.

Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is not enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms) in test 1.

Table A.7.5.x1.1.1-1: Supported test configurations for FR2 PCell

| Configuration | Description |
| --- | --- |
| 1 | TDD duplex mode, 120 kHz SSB SCS, 100 MHz bandwidth |
| 2 | TDD duplex mode, 240 kHz SSB SCS, 100 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR2 |  |

Table A.7.5.x1.1.1-2: General test parameters for FR2 PCell for SSB-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  | Test Config. | Unit | TRP1 | TRP2 | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| Active PCell |  | 1-2 |  | Cell 1 | Cell 1 |  |
| RF Channel Number |  | 1-2 |  | 1 | 1 |  |
| Duplex mode |  | 1-2 |  | TDD | TDD |  |
| TDD Configuration |  | 1-2 |  | TDDConf.3.1 | TDDConf.3.1 |  |
| BWchannel |  | 1-2 |  | 100: NRB,c = 66 | 100: NRB,c = 66 |  |
| Data RBs allocated |  | 1-2 |  | 66 | 66 |  |
| PDSCH/PDCCH subcarrier spacing |  | 1-2 | kHz | 120 | 120 |  |
| DL initial BWP configuration |  | 1-2 |  | DLBWP.0.1 | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  | 1-2 |  | DLBWP.1.1 | DLBWP.1.1 |  |
| UL initial BWP configuration |  | 1-2 |  | ULBWP.0.1 | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  | 1-2 |  | ULBWP.1.1 | ULBWP.1.1 |  |
| PDSCH Reference Channel |  | 1 |  | SR.3.2 TDD | SR.3.2 TDD |  |
|  |  | 2 |  | SR.3.3 TDD | SR.3.3 TDD |  |
| RMSI CORESET Reference Channel |  | 1 |  | CR.3.1 TDD | CR.3.1 TDD |  |
|  |  | 2 |  | CR.3.2 TDD | CR.3.2 TDD |  |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.3.1 TDD | CCR.3.1 TDD |  |
|  |  | 2 |  | CCR.3.7 TDD | CCR.3.7 TDD |  |
| OCNG parameters |  | 1-2 |  | OP.1 | OP.1 |  |
| CP length |  | 1-2 |  | Normal | Normal |  |
| PDSCH/PDCCH TCI state |  | 1-2 |  | TCI.State.0 | TCI.State.0 |  |
| CSI-RS for tracking |  | 1-2 |  | TRS.2.1 TDD | TRS.2.1 TDD |  |
| SSB Configuration |  | 1 |  | SSB.1 FR2 | SSB.1 FR2 |  |
|  |  | 2 |  | SSB.2 FR2 | SSB.2 FR2 |  |
| SMTC Configuration |  | 1-2 |  | SMTC.3 | SMTC.3 |  |
| PRACH Configuration |  | 1-2 |  | FR2 PRACH configuration 2 | FR2 PRACH configuration 2 | A.3.8.3.2 |
| DRX configuration |  | 1-2 |  | OFF | OFF |  |
| SSB index assigned as BFD RS |  | 1-2 |  | 0 | 0 |  |
| SSB index assigned as CBD RS |  | 1-2 |  | 1 | 1 |  |
| SSB index assigned as RLM RS |  | 1-2 |  | 0,1 | 0,1 |  |
| Beam failure detection transmission parameters | DCI format | 1-2 |  | 1-0 | 1-0 |  |
|  | Number of Control OFDM symbols | 1-2 |  | 2 | 2 |  |
|  | Aggregation level | 1-2 | CCE | 8 | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 1-2 | dB | 0 | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 1-2 | dB | 0 | 0 |  |
|  | DMRS precoder granularity | 1-2 |  | REG bundle size | REG bundle size |  |
|  | REG bundle size | 1-2 |  | 6 | 6 |  |
| Gap pattern ID |  | 1-2 |  | gp0 | gp0 |  |
| gapOffset |  | 1-2 | ms | 0 | 0 |  |
| rlmInSyncOutOfSyncThreshold |  | 1-2 |  | absent | absent | Value 0 is applied. (Table 8.1.1-1). |
| rsrp-ThresholdSSB |  | 1 | dBm/SCS | -95 | -95 | Threshold used for Qin_LR_SSB |
|  |  | 2 |  | -92 | -92 |  |
| powerControlOffsetSS |  | 1-2 |  | db0 | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  | 1-2 |  | n1 | n1 | see TS 38.321 [7], clause 5.17 |
| beamFailureDetectionTimer |  | 1-2 |  | pbfd4 | pbfd4 | see TS 38.321 [7], clause 5.17 |
| CSI-RS configuration for CSI reporting |  | 1-2 |  | CSI-RS.3.1 TDD | CSI-RS.3.1 TDD |  |
| reportConfigType |  | 1-2 |  | periodic | periodic |  |
| reportQuantity |  | 1-2 |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI |  |
| CSI reporting periodicity |  | 1-2 | slot | 40 | 40 |  |
| CSI reporting offset |  | 1-2 | slot | 4 | 4 |  |
| T310 |  | 1-2 | ms | 1000 | 1000 |  |
| N310 |  | 1-2 |  | 2 | 2 |  |
| T1 |  | 1-2 | s | 1 | 1 | The UE shall be fully synchronized to cell 1 during T1 |
| T2 |  | 1-2 | s | [5.17] | [5.17] |  |
| T3 |  | 1-2 | s | [3.24] | [3.24] |  |
| T4 |  | 1-2 | s | 0 | 0 |  |
| T5 |  | 1-2 | s | [1.97] | [1.97] |  |
| D1 |  | 1-2 | s | [1.93] | [1.93] |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |

Table A.7.5.x1.1.1-3: Cell specific test parameters for FR2 PCell for SSB-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  |  | Setup 3 as specified in clause A.3.15 Note 12 |  |  |  |  |
| Assumption for UE beams Note 10 |  |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_SSB of set q0,0 | Config 1-2 | dB | 5Note 11 | -3Note 11 | -12 | -12 | -12 |
| SNR_SSB of set q1,0 | Config 1-2 | dB | 5Note 11 | 5Note 11 | 5Note 11 | 5Note 11 | 5Note 11 |
| SNR_SSB of set q0,1 | Config 1-2 | dB | 0.2 | 0.2 | 20.2 | 20.2 | 20.2 |
|  | Config 1 | dBm/ SCS | -104.5 | -104.5 | -84.5 | -84.5 | -84.5 |
| SSB_RP of set q0,1 |  |  |  |  |  |  |  |
|  | Config 2 |  | -101.5 | -101.5 | -81.5 | -81.5 | -81.5 |
|  | Config 1,2 | dBm/120 KHz | -104.7 |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.5.5.1.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. Note 10: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 11: This value allows up to 1dB degradation from applied SNR to UE baseband Note 12: AoA1 for TRP1 and AoA2 for TRP2 |  |  |  |  |  |  |  |

Table A.7.5.x1.1.1-4: Void

Figure A.7.5.x1.1.1-1: SNR and L1-RSRP variation SSB for SSB-based beam failure detection and link recovery testing in non-DRX mode

##### A.7.5.x1.1.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1 for TRP 1 and TRP2.

During the period from time point A to time point B the UE shall transmit uplink signal for TRP 1 and TRP2 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3, T4, T5, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission for TRP 2.

During T3 the UE shall detect beam failure and initiate link recovery for TRP 1. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q0,1.

No later than time point F occurring no later than D1 = [1920]+10 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q0,1 for TRP1. The UE shall not transmit preamble on a beam associated with the candidate beam set q0,1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

==========================End of first change =============================