3GPP TSG-RAN WG4 Meeting # 104-e	R4-2212127

# Electronic meeting, 15th-26th, Aug., 2022

Agenda Item:	9.17.3.4

Source:	Intel Corporation

Title:	Discussion on test case design for TRP specific Beam Failure Detection and Link Recovery

Document for:	Discussion

# Introduction

In last meeting, WF regarding to TRP specific Beam Failure Detection and Link Recovery are as follows:

TRP specific BFD and LR - Define the TRP specific BFR and LR test case for both FR1 and FR2 and both EN-DC and NR SA. - Scenarios and configurations for defining TRP specific BFD and LR test case - RAN4 aims to agree a reasonable test case list to minimize test effort - The table below can be taking starting point of scenarios and configurations of the test for further discussion 1; EN-DC; FR1; PSCell; SSB based; Non-DRX. 2; EN-DC; FR2; PSCell; CSI-RS based; DRX. 3; EN-DC; FR1; SCell; CSI-RS-based BFD and SSB-based LR; Non-DRX. 4; NR SA; FR1; PCell; CSI-RS based; DRX. 5; NR SA; FR2; PCell; SSB based; Non-DRX. 6; NR SA; FR2; SCell; CSI-RS based; DRX. - Other options not precluded - DRX configuration for the TRP specific BFD and LR test cases - For each test case, UE is configured either with DRX or without DRX

In this meeting, we discuss the test case design for TRP specific Beam Failure Detection and Link Recovery for FR2

and the corresponding CRs[2,3] are also provided.

# Discussion

BFD-RS configuration

Different from legacy beam failure detection and link recovery testcase, two TRPs will be configured. If BFD-RS (or CBD-RS) resource in one TRP is different from BFD-RS (or CBD-RS) resource in another TRP. UE can perform independent beam failure detection and link recovery for the two TRPs respectively. From the test perspective, there is no difference from legacy.

Observation 1: if BFD-RS (or CBD-RS) resource in one TRP is different from BFD-RS (or CBD-RS) resource in another TRP, there is no difference from legacy from the test perspective.

In last meeting, it’s agreed that scaling factor will be introduced when BFD-RS collides:

Issue 3-1-1 The conditions on applying PTRP = 2 in FR2 - Agreements: - When BFD-RS (or CBD-RS) resource in one BFD (or CBD) set are overlapped with the RS resource from the other BFD (or CBD) set.

If scaling factor is introduced, extra measurement delay will be needed, which will have impact on 5 during times, e.g. T1~T5. Therefore, we suggest to consider the configuration that BFD-RS of two TRPs are overlapped.

Proposal 1: For TRP specific beam failure detection testcase, suggest to consider the configuration that BFD-RS of two TRPs are overlapped.

SNR and L1-RSRP variation

Another issue is whether one TRP or both TRPs will detect beam failure and perform link recovery. From our point of view, there is no much difference. Therefore, for test case, we suggest that TRP1 will detect beam failure and the beam quality of TRP2 will not degrade. The SNR and L1-RSRP variation are shown below:

Figure 1  SNR and L1-RSRP variation for beam failure detection and link recovery testing

Different from legacy SNR variation figure, a new purple line is added, which is corresponding to SNR level of SSB/CSI-RS in q1,0 for TRP2. The SNR always keep at high level for TRP2 and no beam failure will happen.

Proposal 2: For TRP specific beam failure detection testcase, only one TRP will detect beam failure and perform link recovery. SNR level of TRP 2 will always keep at high level.

Time duration for T1~T5

The rule for determining legacy time duration of T1~T5  is as follows:

  - T2 = BFD evlaution period + CBD evaluation+ 50ms margin

  - T3 = BFD evlaution period + 40ms margin

  - T4=0 ms

  - T5= CBD evaluation period + 50ms margin

  - D1 = CBD evaluation period + 10ms margin

Therefore, the legacy T1~T5 are calculated in the following table:

Table 1: legacy T1~T5 duration time for BFD and link recovery test

| FR | RS type | DRX Config | T1 | T2 (BFD time +CBD time +margin) | T3 (BFD time +margin) | T4 | T5 (CBD time +margin) | D1 (CBD time +margin) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR2 | SSB | Non-DRX | 1s | 2.61s = 1.6s+0.96s+0.05s | 1.64s = 1.6s+0.04s | 0 | 1.01s =0.96s+0.05s | 0.97s =0.96s+0.01s |
| FR2 | CSI-RS | DRX | 1s | 5.43s = 5.12s+0.26s+0.05s | 5.16s = 5.12s+0.04s | 0s | 0.31s 0.26s+0.05s | 0.27s = 0.26+0.01s |

Since BFD-RS is overlapped for two TRPs and the measurement will be scaled by factor 2, the T2~T5 time will be extended, the updated value will be:

Table 2:  T1~T5 duration time for TRP specific BFD and link recovery test

| FR | RS type | DRX Config | T1(s) | T2(s) (BFD time +CBD time +margin) | T3(s) (BFD time +margin) | T4(s) | T5(s) (CBD time +margin) | D1(s) (CBD time +margin) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR2 | SSB | Non-DRX | 1 | 2.61 = 2*1.6+2*0.96+0.05 | 3.24 = 2*1.6+0.04 | 0 | 1.97 =2*0.96+0.05 | 1.93 =2*0.96+0.01 |
| FR2 | CSI-RS | DRX | 1 | 10.81 =2*5.12+2*0.26+0.05 | 10.28 = 2*5.12+0.04 | 0 | 0.57 =2*0.26+0.05 | 0.53 =2*0.26+0.01 |

Proposal 3: For TRP specific beam failure detection and link recovery testcase, T2~T5 duration time will be extended due to scaling factor.

# Conclusion

In this contribution, we provide our views regarding inter-cell beam management in FeMIMO:

Observation 1: if BFD-RS (or CBD-RS) resource in one TRP is different from BFD-RS (or CBD-RS) resource in another TRP, there is no difference from legacy from the test perspective.

Proposal 1: For TRP specific beam failure detection testcase, suggest to consider the configuration that BFD-RS of two TRPs are overlapped.

Proposal 2: For TRP specific beam failure detection testcase, only one TRP will detect beam failure and perform link recovery. SNR level of TRP 2 will always keep at high level.

Proposal 3: For TRP specific beam failure detection and link recovery testcase, T2~T5 duration time will be extended due to scaling factor.

# Reference

[1] R4-2210617, WF on FeMIMO RRM performance requirements

[2] R4-2212123, Draft CR about TRP specific Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode.

[3] R4-2212124, Draft CR about TRP specific Beam Failure Detection and Link Recovery Test for FR2 SCell configured withCSI-based BFD and LR in DRX mode