3GPP TSG-RAN WG4 Meeting #104-e		R4-2212141

Electronic Meeting, August 15 – August 26, 2022

Agenda item:	9.18.3.1.1

Source:	Intel Corporation

Title:	Discussion on the RSRP-related THLDs of 1 Rx RedCap UEs for SDT procedure

Document for:	Discussion

- Introduction

There were discussions on impacts of 1 Rx. RedCap UEs for SDT procedure in RAN4 103-e [1]. In the WF [2], the following open issues were captured as below.

RSRP margin for SDT absolute RSRP thresholds - Option 1 (Intel, E///, Apple, Nokia): For FR1, RedCap UE determines the above RSRP related thresholds for SDT procedure as follows: • UE using 2 Rx branches determines any of the above threshold (H1) based on existing signalling and RSRP range defined in TS 38.133. • For absolute RSRP threshold, UE using 1 Rx branch determines any of the above threshold (H2) as H2 = H1 + offsetabsolute - Absolute RSRP threshold for SDT procedure includes sdt-RSRP-Threshold and cg-SDT-RSRP-ThresholdSSB - Option 1b (MTK, Nokia): - Option 1 is agreed provided offset (offsetabsolute) is a constant number. - Option 2 (vivo): 1 Rx UE does not need to apply RSRP offset based on the 2 Rx threshold for CG-SDT. RSRP margin for SDT RSRP change threshold - Option 1 (Intel): For RSRP change threshold, UE using 1 Rx branch determines any of the above threshold (H2) by - Option 1a (Intel, Nokia, Apple, E///): H2 = H1 + offsetRSRPChange RSRP change threshold for SDT procedure includes cg-SDT-RSRP-ChangeThreshold. - Option 1b: Separate threshold H2 used - Option 1c (MTK, Nokia): - Option 1 is agreed provided offset (offsetRSRPChange) is a constant number. - Option 2 (vivo): 1 Rx UE does not need to apply RSRP offset based on the 2 Rx threshold for CG-SDT.

- This paper updates the issue on RSRP Threshold offset issues and provides our views.

- Discussion

## Review on SDT procedure and related RSRP thresholds

In MAC specification for SDT procedure, RRC configures the following parameters [3]:

-	sdt-DataVolumeThreshold: data volume threshold for the UE to determine whether to perform SDT procedure

-	sdt-RSRP-Threshold: RSRP threshold for UE to determine whether to perform SDT procedure

-	cg-SDT-RSRP-ThresholdSSB: an RSRP threshold configured for SSB selection for CG-SDT.

Observation 1: For SDT procedure, RRC configures absolute RSRP threshold of sdt-RSRP-Threshold to define the reliable channel conditions. For cg-SDT procedure, additional cg-SDT-RSRP-ThresholdSSB is configured by RRC.

In addition to these absolute RSRP related threshold values, RSRP-change threshold is defined for TA validation of cg-SDT procedure as below [4]. The RSRP-change is the difference between the first RSRP (RSRP1) measurement and the second RSRP (RSRP2) measurements i.e. RSRP-change = | RSRP(T1’) - RSRP(T2’)|.

### 5.x.3 TA validation requirements When cg-SDT-RSRP-ChangeThreshold [TS 38.331] is configured for TA validation based on the RSRP change criterion according to clause 5.8.2.x in [TS 38.321], the UE is allowed to transmit using CG-SDT using the timing derived using the latest available NTA value as specified in subclause 7.1 provided that - the first RSRP (RSRP1) measurement and the second RSRP (RSRP2) measurements used in the TA validation are valid measurements and, - timing alignment validation for transmission using CG-SDT is valid according to the validation criteria in clause 5.8.2.x in [TS 38.321]. RSRP1 and RSRP2 are considered valid provided that the conditions in Table 5.x.3-1 and Table 5.x.3-2 are met for FR1 and FR2-1. Table 5.x.3-1 Valid measurement for FR1 Measurement; FR1. RSRP1; (T1 – min(640ms, M1*TDRX)) ≤ T1’ ≤ (T1 + min(640ms, M1*TDRX)). RSRP2; (T2 – min(640ms, M1*TDRX)) ≤ T2’ ≤ T2. Table 5.x.3-2 Valid measurement for FR2-1 Measurement; FR2-1. RSRP1; (T1 – [X1]) ≤ T1’ ≤ (T1 + [X1]). RSRP2; (T2 – [X1]) ≤ T2’ ≤ T2. If at least one of RSRP1 and RSRP2 is considered to be invalid based on the above conditions, then the UE shall not validate the CG-SDT using RSRP1 and RSRP2 and shall not transmit using CG-SDT. Additionally, the UE shall not transmit in an CG-SDT occasion that occurs more than [Z ms] after T2. Where: - T1 is the time when the latest NTAwas obtained by the UE via Timing Advance Command MAC control element. - T1’ is the time when the UE has completed RSRP1. - T2 is the time when the UE performs TA validation as defined in clause 5.8.2.x in [TS 38.321] for transmission using CG-SDT. - T2’ is the time when the UE has completed RSRP2. - TDRX is the DRX cycle length in ms. - M1 the scaling factor as defined in clause 4.2.2.2.

The RRC IE for cg-SDT-RSRP-ChangeThreshold is cg-SDT-RSRP-ChangeThreshold-r17 and it has format of  ENUMERATED { dB2, dB4, dB6, dB8, dB10, dB14, dB18, dB22, dB26, dB30, dB34, spare5, spare4, spare3, spare2, spare1}[5].

Observation 2: For cg-SDT procedure, RRC additionally configures cg-SDT-RSRP-ChangeThreshold for TA validation.

## On the necessity of configurable 1 Rx. RSRP margin for SDT procedure

For 1 Rx RedCap UE in SDT procedure, sdt-RSRP-Threshold, cg-SDT-RSRP-ThresholdSSB and cg-SDT-RSRP-ChangeThreshold need to be adjusted for reliable SDT transmission. Recently RAN2 also recognized that 1 Rx. configuration margin also needs to be applied to sdt-RSRP-Threshold [7][8][9] in addition to RA related RSRP parameters mentioned in the LS to RAN2 [6].

For absolute RSRP threshold, UE using 1 Rx branch determines the threshold (H2) as H2 = H1 + offsetabsolute

  - Absolute RSRP threshold for SDT procedure includes sdt-RSRP-Threshold and cg-SDT-RSRP-ThresholdSSB

  - This offsetabsolute is expected to be configured  as a non-negative value since it is for the compensation of 1 Rx. RSRP measurement accuracy degradation i.e.  offsetabsolute >= 0 dB.

Observation 3: Recently RAN2 also recognized that 1 Rx. configuration margin also needs to be applied to sdt-RSRP-Threshold.

Proposal 1: Reflect the RAN2 consideration of sdt-RSRP-Threshold in RAN4 RRM requirements as below.

For FR1, RedCap UE determines the absolute RSRP related thresholds for SDT procedure as follows: • UE using 2 Rx branches determines any of the above threshold (H1) based on existing signalling and RSRP range defined in TS 38.133. • For absolute RSRP threshold, UE using 1 Rx branch determines any of the above threshold (H2) as H2 = H1 + offsetabsolute - - Absolute RSRP threshold for SDT procedure includes sdt-RSRP-Threshold and cg-SDT-RSRP-ThresholdSSB - - The offsetabsolute is to be inherited from RRC parameter of 1 Rx. RSRP absolute configuration margin which is - to be introduced for absolute RSRP THLDs for RA-related procedures.

Proposal 1a: Send an LS to RAN2 capturing the Proposal 1 above to include cg-SDT-RSRP-ThresholdSSB among the candidate of 1 Rx. RSRP absolute configuration margin.

## On the necessity of configurable 1 Rx. RSRP change margin for SDT procedure

For RSRP change threshold for cg-SDT procedure, UE using 1 Rx branch needs to determine the RSRP change threshold (H2) by H2 = H1 + offsetRSRPChange,cg-SDT. since offsetRSRPChange, cg-SDT is expected to be configured as a non-positive value since it is for the compensation of 1 Rx. RSRP change measurement degradation i.e.  offsetRSRPChange,cg-SDT <= 0 dB.

Observation 4: For TA validation of cg-SDT procedure, separate offset of offsetRSRPChange,cg-SDT is required since it is a different measure and shows different value range compared with offsetabsolute.

Proposal 2: Introduce separate offset of offsetRSRPChange, cg-SDT for TA validation of cg-SDT procedure as below.

For FR1, RedCap UE determines the RSRP change related threshold for SDT procedure as follows: • UE using 2 Rx branches determines any of the above threshold (H1) based on existing signalling and RSRP range defined in TS 38.133. • For RSRP change threshold for TA validation of cg-SDT procedure, UE using 1 Rx branch determines any of the above threshold (H2) as H2 = H1 + offsetRSRPChange, cg-SDT.

Proposal 2a: Send an LS to RAN2 capturing the Proposal 2 above.

# Conclusion

The following are the observations and proposals based on the analysis provided in this paper:

Observation 1: For SDT procedure, RRC configures absolute RSRP threshold of sdt-RSRP-Threshold to define the reliable channel conditions. For cg-SDT procedure, additional cg-SDT-RSRP-ThresholdSSB is configured by RRC.

Observation 2: For cg-SDT procedure, RRC additionally configures cg-SDT-RSRP-ChangeThreshold for TA validation.

Observation 3: Recently RAN2 also recognized that 1 Rx. configuration margin also needs to be applied to sdt-RSRP-Threshold.

Proposal 1: Reflect the RAN2 consideration of sdt-RSRP-Threshold in RAN4 RRM requirements as below.

For FR1, RedCap UE determines the absolute RSRP related thresholds for SDT procedure as follows: • UE using 2 Rx branches determines any of the above threshold (H1) based on existing signalling and RSRP range defined in TS 38.133. • For absolute RSRP threshold, UE using 1 Rx branch determines any of the above threshold (H2) as H2 = H1 + offsetabsolute - - Absolute RSRP threshold for SDT procedure includes sdt-RSRP-Threshold and cg-SDT-RSRP-ThresholdSSB - - The offsetabsolute is to be inherited from RRC parameter of 1 Rx. RSRP absolute configuration margin which is - to be introduced for absolute RSRP THLDs for RA-related procedures.

Proposal 1a: Send an LS to RAN2 capturing the Proposal 1 above to include cg-SDT-RSRP-ThresholdSSB among the candidate of 1 Rx. RSRP absolute configuration margin.

Observation 4: For TA validation of cg-SDT procedure, separate offset of offsetRSRPChange,cg-SDT is required since it is a different measure and shows different value range compared with offsetabsolute.

Proposal 2: Introduce separate offset of offsetRSRPChange, cg-SDT for TA validation of cg-SDT procedure as below.

For FR1, RedCap UE determines the RSRP change related threshold for SDT procedure as follows: • UE using 2 Rx branches determines any of the above threshold (H1) based on existing signalling and RSRP change range defined in TS 38.133*. • For RSRP change threshold for TA validation of cg-SDT procedure, UE using 1 Rx branch determines any of the above threshold (H2) as H2 = H1 + offsetRSRPChange, cg-SDT.

*) It would be inherited from TS 38.331 RRC specification.

Proposal 2a: Send an LS to RAN2 capturing the Proposal 2 above.

- References

- R4-2208071, “Discussion on the remaining issues for 1 Rx. RedCap UEs”

- R4-2210592, “WF on RedCap RRM requirements”

- R2-2204216, “Introduction of Small Data Transmission for MAC spec“

- R4-2207128, “Big CR on RRM requirements for Rel-17 NR SDT in INACTIVE “

- R2-2206827, “Big CR on SDT corrections” for TS 38.331

- R4-2206951, “LS on configuration margin for 1 Rx RedCap UEs”

- R2- 2206504,  “Reply LS on configuring margin for 1 Rx RedCap UEs”

- R2-2205786, “RSRP thresholds for 1 Rx RedCap UEs”.

- R2-2206024, “Configuring margin for 1 Rx RedCap UEs”.

# Appendix.

3GPP TSG RAN WG4 Meeting #104-e	R4-221xxxx

Electronic Meeting, August 15 - August 26, 2022

Title:	Draft LS on the RSRP-related THLDs pf 1 Rx. RedCap UEs for SDT procedure

Response to:

Release:	Rel-17

Work Item:	NR_RedCap-Core

Source:	Intel [to be RAN WG4]

To:	RAN WG2

Cc:

Contact Person:

#### Name:	Ian Hwang

#### E-mail Address:	ian.hwang@intel.com

Attachments:	None

1. Overall Description:

RAN4 specifies the requirements for Random access based Small Data Transmissions (RA-SDT) and Configured Grant based Small Data Transmissions (CG-SDT) in TS 38.133 Clause 5.7 and Clause 5.5 respectively. As in RAN2 specifications, sdt-RSRP-Threshold defines the reliable channel conditions for CG-SDT and RA-SDT. For CG-SDT, additional cg-SDT-RSRP-ThresholdSSB is introduced for the same purpose and gNB can also configure cg-SDT-RSRP-ChangeThreshold for TA validation.

During RAN4#102-e, RAN4 sent an LS to RAN2 in R4-2206951, where a need to define separate RSRP thresholds (e.g., for random access procedures) for 1 Rx UEs was discussed and it was decided that a specified offset to the configured threshold for 2Rx UEs may be applied for 1Rx UEs.

During this meeting, RAN4 agreed to apply this 1 Rx. RSRP threshold offset to the thresholds of sdt-RSRP-Threshold and cg-SDT-RSRP-ThresholdSSB as well, i.e.,

- UE using 2 Rx branches determines these thresholds (H1) based on existing signaling defined in TS 38.133.

- UE using 1 Rx branch determines aby of these thresholds (H2) as follows:

H2 = H1 + offset

Where, “offset” is fixed value in dB and corresponds to the magnitude of the difference between RSRP accuracies for 1 Rx branch and 2Rx branches. RAN4 will further inform RAN2 about the value of the “offset” in future meeting.

Moreover, RAN4 agreed to define the threshold cg-SDT-RSRP-ChangeThreshold for 1 Rx UEs in a similar manner but with a separate RSRP change offset, i.e.,

- UE using 2 Rx branches determines cg-SDT-RSRP-ChangeThreshold (H1) based on existing signaling defined in TS 38.133.

- UE using 1 Rx branch determines cg-SDT-RSRP-ChangeThreshold (H2) as follows:

H2 = H1 + offsetcg-SDT,RSRP-Change

Where, “offsetcg-SDT,RSRP-Change ” is fixed value in dB and corresponds to the magnitude of the difference between RSRP Change accuracies for 1 Rx branch and 2Rx branches. RAN4 will further inform RAN2 about the value of the “offset” in future meeting.

2. Actions:

To: RAN WG2

ACTION: RAN4 kindly asks RAN2 to take the above information into consideration.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 Meeting #105		 	      	14 Nov – 18 Nov 2022, F2F Meeting