3GPP TSG-RAN WG4 Meeting #104-e		R4-2212143

Electronic Meeting, August 15 – August 26, 2022

Agenda item:	9.18.4.1

Source:	Intel Corporation

Title:	Discussion on testing for SDT procedures of RedCap UEs

Document for:	Discussion

- Introduction

The latest test case work split in RRC_INACTIVE for RedCap UE after last meeting [1] is as below. Comparing Rel-17 SDT WI, test aspects of HD-FDD and 1 Rx. would be addressed for FR1 while only HD-FDD needs to be revised for FR2.

Test cases for TS 38.133 in FR1 for 1 Rx and 2 Rx UEs

| Section | Type of test case | Volunteering company | Phase |
| --- | --- | --- | --- |
| ## A.6.2 | ## A.6.2 SA:RRC_INACTIVE state mobility for FR1 #### SA: RRC_INACTIVE state mobility | // Following the legacy requirements, no INACTIVE mode specific test cases are introduced for tests that are verified in IDLE mode. | 1 |
|  | #### Test for eDRX requirements | // Depends on whether requirements are identical to IDLE mode or different. | 1 |
|  | #### Test for SDT | // Introduce test cases similar to those agreed in SDT WI, but consider the new aspects HD-FDD and 1 Rx. | 1 |

Test cases for TS 38.133 in FR2 for 2 Rx UEs

| Section | Type of test case | Volunteering company | Phase |
| --- | --- | --- | --- |
| A.7.2 | A.7.2 SA: RRC_INACTIVE state mobility | // Following the legacy requirements, no INACTIVE mode specific test cases are introduced for tests that are verified in IDLE mode. | 1 |
|  | Test for eDRX requirements | // Depends on whether requirements are identical to IDLE mode or different. | 1 |
|  | Test for SDT | // Introduce test cases similar to those agreed in SDT WI, but consider the new aspects HD-FDD and 1 Rx. | 1 |

- In this paper we will provide our views on the RRM test cases of SDT procedure for Rel-17 RedCap UEs after reviewing the status of test case discussion on Rel-17 SDT WI.

- Discussion

The CR text on Rel-17 SDT test case is not available and is expected to be finalized RAN4 #104-e. The expected changes from RedCap aspects would be discussed after reviewing the status of Rel-17 SDT WI as well as Rel-17 RedCap WI.

## Status of Rel. 17 SDT Test Cases

The SDT procedure is for small-data transmission in RRC INACTIVE and both RA-SDT and CG-SDT procedures are defined. In this section, we will briefly revisit the related SDT parameters.

### Introduction

In MAC specification for SDT procedure, RRC configures the following parameters:

-	sdt-DataVolumeThreshold: data volume threshold for the UE to determine whether to perform SDT procedure

-	sdt-RSRP-Threshold: RSRP threshold for UE to determine whether to perform SDT procedure

-	cg-SDT-RSRP-ThresholdSSB: an RSRP threshold configured for SSB selection for CG-SDT.

In addition to these absolute RSRP related threshold values, RSRP-change threshold is defined for TA validation of cg-SDT procedure as below [4]. The RSRP-change is the difference between the first RSRP (RSRP1) measurement and the second RSRP (RSRP2) measurements i.e. RSRP-change = | RSRP(T1’) - RSRP(T2’)|.

### 5.x.3 TA validation requirements When cg-SDT-RSRP-ChangeThreshold [TS 38.331] is configured for TA validation based on the RSRP change criterion according to clause 5.8.2.x in [TS 38.321], the UE is allowed to transmit using CG-SDT using the timing derived using the latest available NTA value as specified in subclause 7.1 provided that - the first RSRP (RSRP1) measurement and the second RSRP (RSRP2) measurements used in the TA validation are valid measurements and, - timing alignment validation for transmission using CG-SDT is valid according to the validation criteria in clause 5.8.2.x in [TS 38.321]. RSRP1 and RSRP2 are considered valid provided that the conditions in Table 5.x.3-1 and Table 5.x.3-2 are met for FR1 and FR2-1. Table 5.x.3-1 Valid measurement for FR1 Measurement; FR1. RSRP1; (T1 – min(640ms, M1*TDRX)) ≤ T1’ ≤ (T1 + min(640ms, M1*TDRX)). RSRP2; (T2 – min(640ms, M1*TDRX)) ≤ T2’ ≤ T2. Table 5.x.3-2 Valid measurement for FR2-1 Measurement; FR2-1. RSRP1; (T1 – [X1]) ≤ T1’ ≤ (T1 + [X1]). RSRP2; (T2 – [X1]) ≤ T2’ ≤ T2. If at least one of RSRP1 and RSRP2 is considered to be invalid based on the above conditions, then the UE shall not validate the CG-SDT using RSRP1 and RSRP2 and shall not transmit using CG-SDT. Additionally, the UE shall not transmit in an CG-SDT occasion that occurs more than [Z ms] after T2. Where: - T1 is the time when the latest NTAwas obtained by the UE via Timing Advance Command MAC control element. - T1’ is the time when the UE has completed RSRP1. - T2 is the time when the UE performs TA validation as defined in clause 5.8.2.x in [TS 38.321] for transmission using CG-SDT. - T2’ is the time when the UE has completed RSRP2. - TDRX is the DRX cycle length in ms. - M1 the scaling factor as defined in clause 4.2.2.2.

### Recent Update on SDT WI

During the RAN4 discussions on SDT WI, a few clarifications and decisions were made both in RRC core part and performance part [2][3].

RRM Core Part - X1 value for FR2: - max{480ms, 8*SMTC periodicity} - Specs texts for the time duration between T2 and the actual CG-SDT transmission is clarified as: - If at least one of RSRP1 and RSRP2 is considered to be invalid based on the above conditions, then the UE shall not validate the CG-SDT using RSRP1 and RSRP2 and shall not transmit using CG-SDT. Additionally, the UE shall not transmit in an CG-SDT occasion that occurs more than 640 ms after T2 - PRS measurement has a lower priority than SDT - eDRX is not considered for TA validation - T1 definition inconsistency between RAN2 and RAN4 is resolved as: - When changing from RRC Connected to RRC INACTIVE, T1 would be the time when RRCRelease with CG-SDT configuration suspendConfig is received - If TAC command is received while in RRC INACTIVE, T1 is the time when the latest MAC CE TA command is received - Further discuss below: - [If TAC command is not received while in RRC INACTIVE , T1 is the time when the latest RRCRelease is received] - An LS to RAN2 (R4-2211122) is sent out

RRM Performance Part - No RRM test cases to be introduced for RA-SDT - No RRM test cases to be introduced for verifying relaxation on inter-freq and inter-RAT measurement for SDT - No RRM test cases to be introduced for verifying specific scheduling restrictions for SDT - Whether or not to introduce RRM test cases for TA validation for CG-SDT - An LS to RAN5 (R4-2210621) is sent out on its testing feasibility

Observation 1: Test cases for CG-SDT would be major test cases for SDT WI if introduced.

Observation 2: TA validation for CG-SDT is under question in RAN5 on the feasibility of testing UE initiated SDT data transmissions in RRC INACTIVE [2].

During the detailed test cases on CG-SDT procedure, RAN4 delegate in SDT WI expressed the prefierence as below under the assumption that CG-SDT test feasibility is confirmed in RAN 5 [2].

Test Case Proposals on CG-SDT - Option 1: Can TE trigger UL data transmission at the UE? - Option 2: Is “when to transmit the UL after data arrival” up to UE implementation? - Option 3: the case where UE shall not transmit with CG-SDT - Option 4: the case where UE shall transmit with CG-SDT - Option 5: RSRP should be increasing or decreasing from RSRP1 - Option 6: RSRP2 should meet the RSRP threshold when UE measured RSRP2 within [640ms] from T3 - Option 7: RSRP2 should not meet the RSRP threshold when UE measured RSRP2 more than [640ms] prior to T3 Group View on Test Case Preferences - A consensus is observed on Option 3 and 4 that should be considered to define RRM testing for CG-SDT if agreed to introduce such test cases, and Option 1 and 2 relate to feasibility. And one company thought Option 5/6/7 are already contained in Option 3 and 4.

## Potential Changes on SDT Test Cases for RedCap UEs

### 1 Rx. reception for FR1

As pointed out [4][5], the configuring margins for 1 Rx. needs to be addressed for reliable SDT procedure under 1 Rx. measurement accuracy degradation. Assuming the adoption of two separate configurable offsets in maintenance discussion on core part – one for absolute RSRP THLD and the other for RSRP Change TLHD for TA validation of CG-SDT procedure, the following aspects needs to be reflected in test cases for CG-SDT.

- Proposal 1: For 1 Rx. RedCap UEs in FR1, consider the test case with offsetRSRP if test case associated with sdt-RSRP-Threshold and cg-SDT-RSRP-ThresholdSSB is introduced in Rel-17 SDT test.

- Proposal 2: For 1 Rx. RedCap UEs in FR1, consider the test case with offsetRSRPChange,CG-SDT under different RSRP change set-up if TA-validation for CG-SDT is introduced in Rel-17 SDT test.

### HD-FDD operation

The following operations can be considered as scope even though they are explicitly mentioned in the LS to RAN2 in Section 2.1.1.

RRM Performance Part - No RRM test cases to be introduced for RA-SDT - No RRM test cases to be introduced for verifying relaxation on inter-freq and inter-RAT measurement for SDT - No RRM test cases to be introduced for verifying specific scheduling restrictions for SDT - Whether or not to introduce RRM test cases for TA validation for CG-SDT - An LS to RAN5 (R4-2210621) is sent out on its testing feasibility

  - RAN4 made the following decision for UE demodulation/CSI requirements in HD-FDD test [6]

    - DDDSU applied for HD-FDD with 1 Rx. UE

    - FFS for the UE demodulation and CSI reporting requirements for 2Rx RedCap UE supporting HD-FDD in FDD bands

- Proposal 3: Reuse DL/UL pattern for the UE demodulation/CSI requirements as a baseline for SDT RRM test of RedCap UEs.

# Conclusion

The following are the observations and proposals based on the analysis provided in this paper:

Observation 1: Test cases for CG-SDT would be major test cases for SDT WI if introduced.

Observation 2: TA validation for CG-SDT is under question for test feasibility in RAN5.

Proposal 1: For 1 Rx. RedCap UEs in FR1, consider the test case with offsetRSRP if test case associated with sdt-RSRP-Threshold and cg-SDT-RSRP-ThresholdSSB is introduced in Rel-17 SDT test.

Proposal 2: For 1 Rx. RedCap UEs in FR1, consider the test case with offsetRSRPChange,CG-SDT under different RSRP change set-up if TA-validation for CG-SDT is introduced in Rel-17 SDT test.

Proposal 3: Reuse DL/UL pattern for the UE demodulation/CSI requirements as a baseline for SDT RRM test of RedCap UEs.

- References

- R2-2210595,  “Test case list for RedCap RRM performance part”

- R4-2210304,  Email discussion summary for [103-e][232] NR_SmallData_INACTIVE

- RP-221309, Status Report to TSG on “NR small data transmissions in INACTIVE state”

- R2-2206504,  “Reply LS on configuring margin for 1 Rx RedCap UEs”

- R4- 2212141,  “Discussion on the RSRP-related THLDs of 1 Rx RedCap UEs for SDT procedure”

- RP-221162, Status Report to TSG on “Support of reduced capability NR devices”