3GPP TSG-RAN WG4 #104-e	R4-2212147

# Electronic Meeting, August 15th – 26th, 2022

Agenda item:	11.11.2

Source: 	Qualcomm Incorporated

Title: 	Issues for Non-collocated Deployments

Document for:	Discussion

# Introduction

New WID to introduce support for non-collocated intra-band deployments was agreed in [1]. The current work is limited to introducing support for 4Rx as support for 2Rx was already introduced in previous releases. In this paper we discuss some of the implementation issues related to 4Rx support and how these related to the maximum power imbalance and receive time difference (RTD) between the aggregated CCs.

# Discussion

Intra-band non-collocated deployments are characterized by larger power imbalance and MRTD (maximum relative time difference) between the CCs received at the UE. The baseline RF architecture is expected to be the typical one for intra-band non-contiguous aggregation in which the antenna and LNA is shared among all the aggregated CCs(signals from all CCs will pass through the same RF front end).

Observation 1: The RF architecture is with a shared antenna and LNA among all the aggregated CCs.

The power imbalance between the CCs could lead to signal distortion, especially on the weaker CC as the gain of the LNA will have to be aligned to the stronger carrier. Large MRTD will create issues in aligning the LNA gain switch timing. Since the gain switch can only be aligned to the slot boundary of one of the CCs, the gain change will happen during the useful symbol on the other CC distorting the received signal.

Observation 2: Handling of power imbalance and large MRTD(>CP) are the challenges in intra-band non-collocated deployments.

Until recently, only intra-band collocated deployments were supported because of the issues/challenges highlighted above. Type II UEs were recently introduced to enable deployments with a larger power imbalance. The specifications allow for a 25dB power imbalance with a sensitivity degradation of 1dB. The requirements are defined only for 2 Rx based on the assumption that a UE equipped with 4Rx (baseline in the FR1 TDD bands >2.4GHz) will use 2Rx for each CC. In this arrangement, RTD > CP can be tolerated as the UE can align the LNA gain switch timing to the slot boundary of each CC. It should be noted that the signals from all the CCs will still pass through the same RF front end, hence, the LNAs will still have to handle a power imbalance of 25dBs on all 4 receivers.

Observation 3: the RF front end still has to handle the power imbalance irrespective of the number of receivers used for each CC.

Observation 4: Splitting the receivers among different CCs enables handling of larger receive time difference at the UE.

Based on the above observations, the UE should be able to handle a power imbalance of up to 25dB, however, a RTD > CP cannot be handled by the same receiver. In order to enable 4Rx on each CC, the actual RTD should be within the CP such that the LNA gain switching does not distort the useful part of the signal. If the switch happens in the useful part of the signal, the SNR on one of the carriers will be impacted. The performance degradation is rather difficult to assess as it depends on several factors such as actual RTD, which channels are affected (reference signals, control channel or data channel), the actual SNR of the affected signals, etc. This analysis will be very time consuming and results are likely to vary among companies. Another solution to enable 4Rx on each CC would be to have 8 receivers, however, this is not feasible due to the large volume and cost required for such implementation.

Observation 5: RTD should be within the CP to enable 4Rx on each CC.

Observation 6: Performance degradation due to LNA signal distortion is difficult to characterize.

# Conclusion

In this paper we discussed the RF architecture for intra-band aggregation and the consequences related to power imbalance, MRTD and number of receivers to be supported on each CC. We made the following observations:

Observation 1: The RF architecture is with a shared antenna and LNA among all the aggregated CCs.

Observation 2: Handling of power imbalance and large MRTD(>CP) are the challenges in intra-band non-collocated deployments.

Observation 3: the RF front end still has to handle the power imbalance irrespective of the number of receivers used for each CC.

Observation 4: Splitting the receivers among different CCs enables handling of larger receive time difference at the UE.

Observation 5: RTD should be within the CP to enable 4Rx on each CC.

Observation 6: Performance degradation due to LNA signal distortion is difficult to characterize.

# References

- RP-221809, “New WID: Support of intra-band non-collocated EN-DC/NR-CA deployment”, KDDI