3GPP TSG-RAN WG4 Meeting # 104-e 	R4-2212158

# Electronic Meeting, 15th – 26th August, 2022

Title:	NR NTN UE frequency error requirement testing

Agenda Item:	9.11.4.1

Source:	MediaTek Inc.

Document for:		Discussion

# Introduction

This document follows up on the NR NTN UE Frequency Error maintenance discussions from RAN4#103-e and proposes a way forward on the test case.

- Discussion

## 2.1	Status as of RAN4#103-e

At RAN4#103-e, there were some discussions on the existing definition of the NTN UE Frequency Error, which resulted in a modification to the definition, resulting in the new definition below:

### 6.4.1	Frequency error

The NTN satellite UE basic measurement interval of modulated carrier frequency is 1 UL slot. The NTN satellite UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift according to 3GPP TS 38.300 [9] clause 16.14.2. The mean value of basic measurements of NTN UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency.

[NOTE:	The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift. For the test case, the location of the UE is explicitly provided to the UE from the test equipment.]

Requirement will be verified for at least two cases of which one has zero Doppler conditions.

There were discussions on how to test the Frequency Error with the new definition, which resulted in the additional note and final statement on verification. However, it was agreed to consider testing aspects further at RAN4#104-e.

## 2.2	Further analysis

As part of any test case, the level of UE pre-compensation needs to be able to be mirrored by the Test Equipment (in terms of what it expects to receive). The 2 specified inputs parameters to derive the level of precompensation are:

- UE location

- Satellite ephemeris data

- Epoch time

- SFN

Doppler shift modelling in the test case

Doppler shift cannot be modelled purely as a shift in the carrier frequency, as delay and delay drift rate will also vary with a change in doppler, and not modelling those characteristics would likely affect the UE reception, and also the Test Equipment ability to estimate the frequency error on uplink. Hence the Test Equipment would need to properly emulate the doppler channel effects within the DL signal and take this into account in the UL estimated signal.

Proposal 1: Clarify the intended modelling of doppler shift in the requirements description of inform RAN5.

Another aspect to consider with a non-zero doppler scenario is the variation of the doppler shift. The satellite ephemeris data provided to the UE (EphemerisInfo in SIB19) requires/enables the UE to “predict” the location of the satellite at any given time (using epoch time and SFN relation also provided). Any prediction would need to also predict the gravitational impacts on the satellite. Therefore, if we want to use real satellite ephemeris data in the test case and a real prediction by the UE, then the test case will naturally model a varying doppler shift.

As the satellite ephemeris data and UE location should be known at the TE, then it seems feasible for the Test Equipment to be able to apply the same shift as the UE predicts that it needs.

For the agreed zero doppler case, a GSO constellation can be modelled with the GSO satellite at the nadir point.

Finally, it should be clarified that only UEs supporting NGSO would be required to pass the non-zero doppler test, and maybe vice versa for the NGSO UEs and the zero doppler test.

Proposal 2: For NGSO, recommend to RAN5 that the frequency error be measured under varying doppler shift for the non-zero doppler shift case. For zero doppler shift case, a GSO satellite is proposed to be modelled.

Proposal 3: GSO-only NTN-supporting UEs should not be required to pass the non-zero doppler test, and maybe NGSO-only NTN-supporting UEs could not be required to pass the zero doppler test.

UE acquisition of its location and knowledge by Test Equipment

The two methods that we have identified for the UE to acquire its location are:

- GNSS simulator approach: Use a GNSS simulator, as used in UE Positioning test case.

- AT command approach: Use existing defined AT command: “Update UE Location Information”, defined in TS 38.509 to provide the UE with location coordinates, as used in some existing 3GPP test cases today.

We believe that the GNSS simulator approach may be cumbersome and complex. It has the following drawbacks:

- It combines a demod test case and an RF test case, so more complexity.

- The GNSS signal power levels would need to be decided appropriately by RAN4 to acquire the satellite with sufficient accuracy within a certain time.

- Acquisition of the GNSS location would take some time so increase test time.

The AT command approach alternative does not have the above issues. It exists in specifications today, requires a minimal software addition to the UE, and minimises test time.

Proposal 4: Re-use the existing AT command: “Update UE Location Information”, defined in TS 38.509 to provide the UE with location coordinates for usage with precompensation in the Frequency Error test case.

# Proposals

Proposal 1: Clarify the intended modelling of doppler shift in the requirements description of inform RAN5.

Proposal 2: For NGSO, recommend to RAN5 that the frequency error be measured under varying doppler shift for the non-zero doppler shift case. For zero doppler shift case, a GSO satellite is proposed be modelled.

Proposal 3: GSO-only NTN-supporting UEs should not be required to pass the non-zero doppler test, and maybe NGSO-only NTN supporting UEs could not be required to pass the zero doppler test.

Proposal 4: Re-use the existing AT command: “Update UE Location Information”, defined in TS 38.509 to provide the UE with location coordinates for usage with precompensation in the Frequency Error test case.