3GPP TSG RAN WG4 #103-e meeting 	R4-2212159

Electronic Meeting, 15th – 26th August 2022

### Agenda Item:	11.6.4.1

### Source:	Meta

### Title:	On the principles of the lower MSD discussion for UE device type

Document for:	Approval

# Introduction

In the RAN #95-e meeting, RAN approved the FR1 RF further enhancement WI [1] in Rel-18. In the approved new WI, RAN4 will evaluate and specify the additional RF requirements for specific FR1 RF UE devices if the evaluation outcomes are feasible as followings:

- Objectives - Enable 4Tx on a single carrier for CPE/FWA/Vehicle/Industrial devices - Enable 8Rx for CPE/FWA/Vehicle/Industrial devices - Investigate the feasibility of lower MSD for inter-band CA/EN-DC/DC combinations Specially, In this paper, we focus on the lower MSD evaluation as follows detail objectives Investigate the feasibility of lower MSD for inter-band CA/EN-DC/DC combinations [RAN4] - Select a limited set of band combinations (2-4 combinations) to cover all types of MSD (harmonic, harmonic mixing, IMD and cross band isolation) - Study how the MSD performance can be improved for the example band combinations - Study of MSD improvement with different MSD sources (harmonics, IMD2/3/4/5, cross band isolation and harmonic mixing) - Study the feasibility of and options for allowing a UE to signal improved lower MSD performance capability for combinations where MSD is allowed - Aim to conclude the study phase by RAN#99, and further discuss in RAN#99 how to handle the objective based on the study progress.

In this paper, we provide our views on the principles to evaluate the lower MSD issues for inter-band CA/EN-DC/DC combinations.

# Discussion on the principle of lower MSD evaluation

RF specifications in Rel-15 only supported the PC3 inter-band NR DC/CA band combinations and then, both PC2 and PC3 inter-band DC/CA band combinations were additionally introduced in Rel-16. During the Rel-17 timeframe, lower MSDs were observed from several UEs against the specified requirements for the inter-band DC band combination in TS 38.101-3. RAN Plenary decided to investigate a feasibility of an MSD improvement to specify a lower MSD capability.

During the RAN#95-e, while there were intensively discussed on example CA/DC band combinations and how to improve MSD for various IMD orders to detail the objectives for lower MSD issues which was captured in the moderator summary [2], the example CA/DC band combinations and whether the scope of the lower MSD is applicable to both PC2 and PC3 inter-band NR CA/DC or not were not decided.

In our view, only PC2 inter-band CA/DC band combinations are prioritized as the MSD requirements for PC3 inter-band CA/DC have been specified since Rel-13 LTE-A CA band combinations which requires huge workload to replace or update the existing band combinations.

Proposal #1: RAN4 prioritizes only on the PC2 inter-band CA/DC band combinations for the lower MSD discussion.

If RAN4 decides to investigate on both PC3 and PC2 inter-band CA/DC, then it is reasonable to consider only NR inter-band DC/CA band combinations which could be supported as release independent manner from Rel-18 as a new capability signalling for the lower MSD capable UEs in Rel-18 is needed to be introduced by RAN2.

Proposal #2: If RAN4 decides to investigate the lower MSD issues for both PC3 and PC2 inter-band CA/DC UE, then it is reasonable to consider only NR inter-band DC/CA band combinations which could be supported as release independent manner from Rel-18.

For each type of MSD sources, RAN4 can evaluate the required MSD levels based on the following example CA band combinations:

Example CA band combinations for each MSD sources: - Harmonics problems - 2nd Harmonic case: CA_n1A-n77A, CA_n3A-n77A - 3rd Harmonic case: CA_n7A-n8A - 4th Harmonic case: CA_n5A-n77A - 5th Harmonic case: CA_n8A-n79A - Harmonic mixing problems - 2nd Harmonic mixing: CA_n3A-n77A - 3rd Harmonic mixing: CA_n28A-n40A - 4th Harmonic mixing: CA_n5A-n77A - 5th Harmonic mixing: CA_n13A-n77A - IMD due to dual uplink transmission - 2nd IMD: CA_n1A-n77A - 3rd IMD: CA_n1A-n3A - 4th IMD: CA_n1A-n77A - 5th IMD: CA_n5A-n77A - Leakage due to cross band coupling (or isolation) - CA_n41A-n77A - Aggressor from n41A to Victim in n77A - Aggressor from n77A to Victim in n41A

Proposal #3: RNA4 considers the above example CA band combinations for the the lower MSD evaluation during the study phase (until RAN#99-e meeting). The MSD analysis outcomes of inter-band CA band combinations will be applicable to the inter-band DC band combinations.

# Discussion on UE RF architecture and RF component assumptions

# 3.1 UE RF architecture for MSD evaluation

In Rel-15 NR DC for NSA UE, it was assumed both antenna-shared and antenna-separated UE RF architectures for LTE and 5G NR system to support UL-MIMO and 4-Rx diversity [3], where the main Tx/Rx antenna can be shared by both LTE and NR systems. Also, it was assumed a separate antenna RF architecture for the NSA UE to support n77/n78/n79 operating bands. In Rel-16, the one-chip solution was introduced for both LTE and NR modems with chip technology advancement.

Example RF architectures in TR 38.860-12

UE RF architecture w/ sharing antennas

UE RF architecture w/ separate antennas

Figure 3-1: Example RF architectures (from TR 38.860-12)

Therefore, it is reasonable to consider the same assumption (i.e., both antenna-shared and antenna-separated UE RF architectures) to evaluate lower MSDs for the different MSD sources in the WID (i.e., harmonics, harmonic mixing, cross-band coupling (or isolation), and IMD due to dual uplink transmission) for both inter-band CA/DC band combinations.

Figure 3-1 shown the candidate RF architectures for inter-band CA band combinations in LTE-A CA features in TR 36.860-12 wheretriplexer/quadplexer or a new RF component can be used instead of diplexer to distinguish the NR operating band.

Based on the observations, we propose as follow

Proposal #4: The example RF architectures in TR36.860-12 can be reused to evaluate lower MSD according to the different MSD sources.

# 3.2 Proposed MSD evaluation assumptions

For the MSD evaluation, following parameters can be considered as assumptions. The detail operating band specific levels in Table 3-2 are proposed by UE vendors to evaluate the required MSD values.

Table 3-1: UE RF Front-end component isolation parameters

| Isolation Parameter | Value (dB) | Comment |
| --- | --- | --- |
| Antenna to Antenna | 10 | Main antenna to diversity antenna for smartphone type UE |
| PA (out) to PA (in) | 60 | PCB isolation (PA forward mixing) |
| Triplexer | 20 | High/low band isolation |
| Diplexer | 25 | High/low band isolation |
| PA (out) to PA (out) | 60 | L-H/H-L cross-band |
| PA (out) to PA (out) | 50 | H-H cross-band |
| LNA (in) to PA (out) | 60 | L-H/H-L cross-band |
| LNA (in) to PA (out) | 50 | H-H cross-band |

Table 3-2: RF parameters for the dual uplink transmission

| parameters | level |
| --- | --- |
| Total NF (dB) | 9 dB 10dB in n77/n78/n79 |
| RFIC Noise for Rx band (dBm/Hz) | Operating Band specific level |
| PA Noise for Rx band (dBm/Hz) | Operating Band specific level |
| PA Gain for Rx band (dB) | 28 for PC3 UE 31 for PC2 UE |
| Duplexer Tx/Rx isolation (dB) | Operating Band specific level |
| RF Front-End Loss (dB) | 4 |
| Diversity gain (dB) | 3 |
| Ant. Switch IP2/IP3/IP4/IP5 | Operating Band specific levels |
| Diplexer IP2/IP3/IP4/IP5 | Operating Band specific levels |
| Duplexer IP2/IP3/IP4/IP5 | Operating Band specific levels |
| PA Forward IP2/IP3/IP4/IP5 | Operating Band specific levels |
| PA reversed IP2/IP3/IP4/IP5 | Operating Band specific levels |
| LNA IP2/IP3/IP4/IP5 | Operating Band specific levels |

Proposal #5: The above RF parameters in Table 3-1 and Table 3-2 are considered to evaluate improved MSDs.

# 3.3 Guidelines for new “lowest” or new “highest” UL or DL CBW for MSD analysis

In the approved WF [6], we prefer to keep and apply the agreements into the lower MSD values discussion. One clarification point is that the new MSD table format for cross band isolation as follow

Table 3-3. New MSD table format for cross-band isolation

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band | Power class |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Interference |  |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) | source |  |
| n1 | n3 | 1945 | 50 | 15 | 128 (RBstart=0) | 1877.5 | 5 | FFS | ACLR1 | PC3 |
| n1 | n40 | 1970 | 20 | 15 | 100 (RBstart=6) | 2302.5 | 5 | FFS | >ACLR2 | PC3 |
| n3 | n41 | 1765 | 40 | 15 | 50 (RBstart=166) | 2501 | 10 | FFS | >ACLR2 | PC3 |
| n3 | n74 | 1730 | 40 | 15 | 50 (RBstart=0) | 1515.5 | 5 | FFS | >ACLR2 | PC3 |
| n7 | n40 | 2525 | 50 | 15 | 45 (RBstart=0) | 2397.5 | 5 | FFS | >ACLR2 | PC3 |
| n34 | n3 | 2017.5 | 15 | 15 | 75 (RBstart=0) | 1877.5 | 5 | FFS | >ACLR2 | PC3 |
| n38 | n1 | 2590 | 40 | 15 | 216 (RBstart=0) | 2167.5 | 5 | FFS | >ACLR2 | PC3 |
| n38 | n25 | 2590 | 40 | 15 | 216 (RBstart=0) | 1992.5 | 5 | FFS | >ACLR2 | PC3 |
| n38 | n78 | 2600 | 40 | 15 | 216 (RBstart=0) | 3305 | 10 | FFS | >ACLR2 | PC3 |
| n40 | n1 | 2340 | 80 | 30 | 216 (RBstart=0) | 2167.5 | 5 | FFS | >ACLR2 | PC3 |
| n41 | n1 | 2546 | 100 | 30 | 270 (RBstart=0) | 2167.5 | 5 | FFS | >ACLR2 | PC3 |
| n41 | n3 | 2546 | 100 | 30 | 270 (RBstart=0) | 1877.5 | 5 | FFS | >ACLR2 | PC3, PC2 |
| n41 | n25 | 2546 | 100 | 30 | 270 (RBstart=0) | 1992.5 | 5 | FFS | >ACLR2 | PC3, PC2, PC1.5 |
| n41 | n66 | 2546 | 100 | 30 | 270 (RBstart=0) | 2197.5 | 5 | FFS | >ACLR2 | PC3, PC2, PC1.5 |
| n41 | n48 | 2640 | 100 | 30 | 270 (RBstart=3) | 3552.5 | 5 | FFS | >ACLR2 | PC3 |
| n41 | n70 | 2546 | 100 | 30 | 270 (RBstart=0) | 2017.5 | 5 | FFS | >ACLR2 | PC3 |
| n41 | n77 | 2640 | 100 | 30 | 270 (RBstart=3) | 3305 | 10 | FFS | >ACLR2 | PC3, PC2, PC1.5 |
| n41 | n78 | 2640 | 100 | 30 | 270 (RBstart=3) | 3305 | 10 | FFS | >ACLR2 | PC3 |
| n77 | n2 | 3350 | 100 | 30 | 270 (RBstart=0) | 1987.5 | 5 | FFS | >ACLR2 | PC2, PC1.5 |
| SUL band combinations |  |  |  |  |  |  |  |  |  |  |
| n95 | n41 | 2017.5 | 15 | 15 | 75 (RBstart=4) | 2501 | 10 | FFS | >ACLR2 | PC3 |
| n97 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2501 | 10 | FFS | ACLR1 & ACLR2 | PC3 |

For the CA_n41-n97 UE, the frequency separation from the upper edge of uplink in n97 to the lower edge of downlink in n41 is 96MHz, the small portion (less than 2.5MHz) can be included in first ACLR (noted as ACLR1 in the table above) frequency range. When both ACLR1 and ACLR2 ranges are impacted in the victim carrier due to cross band isolation, RAN4 can designate with “ACLR1 & ACLR2” due to interference perspectives for the interference source in new MSD Table.  The CA_n18-n28 is also in similar situations from the interference source perspectives. From this perspective, we think it makes senses to change the “ACLR1” to “ACLR1 & ACLR2” for CA_n18-n28 in Table 7.3A.6-1 in TS 38.101-1.

Proposal #6: When both ACLR1 and ACLR2 ranges are impacted in the victim carrier due to cross band isolation, RAN4 indicate “ACLR1 & ACLR2” as the interference source in the new MSD Table.

Typically, ACRL1 and ACLR2 refer to UTRA ACLR1 and UTRA ACLR2, respectively, unless there is clear indication or explanation. To avoid such an ambiguity, it is necessary to have a separate indication rather than just ACLR1 or ACLR2.

Proposal #7: To avoid confusion, it is proposed to use NR_ACLR1 and/or NR_ACLR2 to indicate a cross-band interference source(s), instead of using just ACLR1 and/or ACLR2.

# Conclusions

In this contribution, we provide the basic principle to evaluate lower MSD according to different source. Also, we provided the example RF architecture and detail RF parameters for MSD investigation.

Based on the above observation in session 2 and 3, we propose as follows

Proposal #1: RAN4 prioritizes only on the PC2 inter-band CA/DC band combinations for the lower MSD discussion.

Proposal #2: If RAN4 decides to investigate the lower MSD issues for both PC3 and PC2 inter-band CA/DC UE, then it is reasonable to consider only NR inter-band DC/CA band combinations which could be supported as release independent manner from Rel-18.

Proposal #3: RNA4 considers the above example CA band combinations for the the lower MSD evaluation during the study phase (until RAN#99-e meeting). The MSD analysis outcomes of inter-band CA band combinations will be applicable to the inter-band DC band combinations.

Proposal #4: The example RF architectures in TR36.860-12 can be reused to evaluate lower MSD according to the different MSD sources.

Proposal #5: The above RF parameters in Table 3-1 and Table 3-2 are considered to evaluate improved MSDs.

Proposal #6: When both ACLR1 and ACLR2 ranges are impacted in the victim carrier due to cross band isolation, RAN4 indicate “ACLR1 & ACLR2” as the interference source in the new MSD Table.

Proposal #7: To avoid confusion, it is proposed to use NR_ACLR1 and/or NR_ACLR2 to indicate a cross-band interference source(s), instead of using just ACLR1 and/or ACLR2.

# Reference

[1] RP-220987, “New WID on Further RF requirements enhancements for NR FR1,” Moderator (Huawei)

[2] RP-220863, “Moderator summary for discussion [95e-03-RAN4-R18-UEFR1RF],” Moderator (Huawei)

[3] TR37.863-01-01 v15.4.0

[4] TS38.101-1

[5] TS38.101-3

[6] R4-2210565, “WF on criteria on Rel-17 enhanced MSD table format,” Skyworks