3GPP TSG-RAN WG4 Meeting # 104-e	R4-2212164

# Electronic Meeting, August 15 - 26, 2022

Source: 	Skyworks Solutions, Inc.

Title: 	Coexistence aspects for the new 1670-1675MHz TDD band

Agenda Item:	12.3.2	UE RF requirements	[LTE_TDD_1670_1675MHz -Core]

Document for:	Approval

# Introduction

In Release 18 WI to introduce a new LTE TDD band in the 1670-1675MHz range [1], a factor to consider is the coexistence aspects with existing bands, and whether the new band can be protected by legacy bands. In this contribution we examine neighboring bands and their filter capabilities, to evaluate potential coexistence issues.

# Discussion

## New band spectrum environment

In Figure 1, we provide a graphical representation of neighboring bands including the new 1670-1675MHz LTE TDD band (denoted band 105 for convenience and highlighted in yellow).

Figure 1: Neighboring bands including the new 1670-1675MHz LTE TDD band (105) in the 1520-1800MHz range

Observations on protection of the new band by legacy bands:

- Band n24/n99 UL is at 10MHz distance with maximum CBW of 10MHz

- ACLR2 interference at -25dBm/MHz from SEM

- n24/n99 UL filter would require 25dB rejection at the new band frequency to meet -50dBm/MHz coexistence level

- Band n255 UL is at 10MHz distance with maximum CBW of 20MHz

- ACLR1 interference at -13dBm/MHz from SEM

- n255 UL filter would require 37dB rejection at the new band frequency to meet -50dBm/MHz coexistence level

- n70 with 20MHz gap and maximum CBW of 15MHz =>

- ACLR2 @ -30dBm/MHz from SEM

- n70+n66 UL filter would require 20dB rejection at the new band frequency to meet -50dBm/MHz coexistence level

- n66 UL filter is at 35MHz distance with maximum CBW of 45MHz

- ACLR1 @ -13dBm/MHz from SEM

- n70+n66 UL filter would require 37dB rejection at the new band frequency to meet -50dBm/MHz coexistence level

Current state of the art UE design constraints

- Same duplexer should target n24/n99 and n255

- n70 UL is consolidated with n66 UL in one Tx filter (n70 DL consolidated with n25 DL)

- Legacy device had no DL to protect in between n24 UL and n70 UL

- Current n24/n99/n255 and n70+n66 design provide low or moderate rejection at 1670-1675MHz

- For n24/n99 if rejection is required at 1670-1675MHz, it cannot compromise the filter performance for GNSS

- -50dBm/MHz coexistence protection level is a challenge for (in order of magnitude):

  - Band n255 UL (upper half ACLR1 overlap)

  - Band n66 UL when co-band with n70 (end of ACLR1 overlap)

  - Band n24 UL (beginning of ACLR2 overlap)

  - Band n70 UL (middle of ACLR2 overlap)

- Relaxed coexistence requirement is needed at least for some of the UL channels in terms of position and bandwidth.

Observations on protection of legacy bands by the new band:

- The new band closest DL band to be protected, is band n24 and band n255 at 110MHz distance

- There should be no issue for the new band to protect neighbor DL bands

- Note that 2nd harmonic of the new band falls in band n77, but outside of the range used in North America

- Also note that driving design criteria for the new band filter would is related to the in-device coexistence with GNSS bands, at 60MHz distance

Proposal on protection of legacy bands by the new 1670-1675MHz TDD band: given the distance to US legacy DL bands and the fact that H2 falls outside the n77 US range, -50dBm/MHz protection can be granted for all US bands.

With the above observations, the key issue is on how to specify legacy bands coexistence with the new band. This has to accounts for legacy UEs and their filter performance and co-banding implementations.

Proposal on protection of the new 1670-1675MHz TDD band by legacy bands:

- -50dBm/MHz protection level by band n24/n99, n70 and n66 is not specified

- Relaxed protection can be specified but must account for legacy devices:

  - Band n24 UL filter is focussed on the critical protection of the GNSS bands on the lower frequency side which is regulatory and thus provides only a small rejection at the new band frequencies at the higher frequency side

  - Band n70 and n66 UL use a consolidated UL filter, which only provides moderate rejection at the new band frequencies

  - The relaxation could be limited to UL BW above a given value and/or at a given position

  - For some cases, it may be feasible to provide the -50dBm/MHz protection level but only up to a given channel bandwidth, while not specifying protection for a higher CBW (for n66 for example)

# Conclusions

In this contribution, we provided an evaluation of the new 1670-1675MHz potential co-existence issues with surrounding bands, and make the following proposals.

Proposal on protection of legacy bands by the new 1670-1675MHz TDD band: given the distance to US legacy DL bands and the fact that H2 falls outside the n77 US range, -50dBm/MHz protection can be granted for all US bands.

Proposal on protection of the new 1670-1675MHz TDD band by legacy bands:

- -50dBm/MHz protection level by band n24/n99, n70 and n66 is not specified

- Relaxed protection can be specified but must account for legacy devices:

  - Band n24 UL filter is focussed on the critical protection of the GNSS bands on the lower frequency side which is regulatory and thus provides only a small rejection at the new band frequencies at the higher frequency side

  - Band n70 and n66 UL use a consolidated UL filter, which only provides moderate rejection at the new band frequencies

  - The relaxation could be limited to UL BW above a given value and/or at a given position

  - For some cases, it may be feasible to provide the -50dBm/MHz protection level but only up to a given channel bandwidth, while not specifying protection for a higher CBW (for n66 for example)

# References

- RP-220380 New WI: Introduction of LTE TDD band in 1670-1675 MHz, Ligado Networks, RAN#95-e