3GPP TSG-RAN WG4 Meeting #104-e	R4-2212205

Online Meeting, Aug 15 – 26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | DraftCR | rev |  | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR – Test cases for NB-IoT DL 16-QAM demodulation performance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh4_LTE_eMTC6-Perf |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce test cases for NB-IoT DL 16-QAM demodulation performance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added test case for DL 16-QAM demodulation performance HD-FDD NB-IoT UE in standalone mode. - Added test case for DL 16-QAM demodulation performance TDD NB-IoT UE in standalone mode. - Added new reference measurement channels for HD-FDD and TDD 16-QAM NPDSCH in standalone mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Performance requirements for NB-IoT DL 16-QAM would be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.12.1, A.3.12.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of change 1>

### 8.12.1	NPDSCH

8.12.1.1	Half-duplex FDD

The parameters specified in Table 8.12.1.1-1 are valid for all half-duplex FDD tests unless otherwise stated.

Table 8.12.1.1-1: Common Test Parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| Number of HARQ processes per component carrier (Note 2) | Processes | 1 or 2 |
| Maximum number of HARQ transmission |  | 4 |
| Cyclic Prefix |  | Normal |
| eutraControlRegionSize-r13 |  | 3 for In-band and N/A for Standalone/Guard-band unless otherwise stated |
| downlinkBitmap-r13 and dl-Gap-r13 |  | Not configured |
| dl-GapNonAnchor-r13 and downlinkBitmapNonAnchor-r13 |  | Not configured |
| Unused REs or RB (Note 1) |  | OCNG |
| OCNG pattern |  | NB.OP.1 |
| Note 1: For in-band mode, the REs for transmission of LTE signals including PDCCH, CRS should be filled by OCNG. Note 2: 1 is applicable for both UE Category NB1 and NB2; 2 is only applicable for UE Category NB2 if UE supports it. |  |  |

Table 8.12.1.1-2: Test Parameters of related NPDCCH and NPUSCH format 2 configurations

| Parameter | Unit | Value |
| --- | --- | --- |
| DCI format |  | DCI format N1 |
| scheduling delay field () |  | For tests in 8.12.1.1.4: 1 For tests in 8.12.1.1.5: 0 for the first HARQ process; 2 for the second HARQ process if configured For others: 1 for the first HARQ process; 2 for the second HARQ process if configured |
| (ack-NACK-NumRepetitions-r13) |  | 1 |
| ACK/NACK resource field |  | 0 |
| Reference channel for NPDCCH |  | R.NB.3 FDD for one NRS antenna port; R.NB.4 FDD for two NRS antenna ports |
| (npdcch-Offset-USS-r13) |  | 0 |

< End of change 1>

< Start of change 2>

##### 8.12.1.1.5	Minimum Requirements for Standalone for UE with 16-QAM

The requirements are specified in Table 8.12.1.1.5-2, with the addition of the parameters in Table 8.12.1.1.5-1 and the downlink physical channel setup according to Annex C.3.6. The purpose of these tests is to verify the NPDSCH performance with 16-QAM.

Table 8.12.1.1.5-1: Test Parameters for 16-QAM NPDSCH under Standalone

| Parameter | Unit |  | Test 1 |
| --- | --- | --- | --- |
| at antenna port |  | dBm/15kHz | -93 (Note 1) |
|  |  | dBm/15kHz | -96 (Note 2) |
| NPDCCH repetition number |  | subframe | 4 for Test 1 |
| (npdcch-NumRepetitions-r13) |  | subframe | 8 for Test 1 |
| (nPDCCH-startSF-USS-r13) |  |  | 1.5 |
| Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission. |  |  |  |

Table 8.12.1.1.5-2: Minimum performance for 16-QAM NPDSCH under Standalone with 1 NRS port

| Test number | Bandwidth | Carrer Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Non-anchor | R.NB.9 FDD | 1 | EPA5 | 1 | 1x1 | 70% | [19.5] | NB2 |

< End of change 2>

< Start of change 3>

Table 8.12.1.2-2: Test Parameters of related NPDCCH and NPUSCH format 2 configurations

| Parameter | Unit | Value |
| --- | --- | --- |
| DCI format |  | DCI format N1 |
| scheduling delay field () |  | For tests in 8.12.1.2.4: 0 for the first HARQ process; 2 for the second HARQ process if configured For others: 1 for the first HARQ process; 2 for the second HARQ process if configured |
| (ack-NACK-NumRepetitions-r13) |  | 1 |
| ACK/NACK resource field |  | 0 |
| Reference channel for NPDCCH |  | R.NB.3 TDD for one NRS antenna port; R.NB.4 TDD for two NRS antenna ports |
| (npdcch-Offset-USS-r13) |  | 0 |

< End of change 3>

< Start of change 4>

##### 8.12.1.2.4	Minimum Requirements for Standalone for UE with 16-QAM

The requirements are specified in Table 8.12.1.2.4-2, with the addition of the parameters in Table 8.12.1.2.4-1 and the downlink physical channel setup according to Annex C.3.6. The purpose of these tests is to verify the NPDSCH performance with 16-QAM.

Table 8.12.1.2.4-1: Test Parameters for 16-QAM NPDSCH under Standalone

| Parameter | Unit |  | Test 1 |
| --- | --- | --- | --- |
| at antenna port |  | dBm/15kHz | -93 (Note 1) |
|  |  | dBm/15kHz | -96 (Note 2) |
| NPDCCH repetition number |  | Subframe | 4 for Test 1 |
| (npdcch-NumRepetitions-r13) |  | Subframe | 8 for Test 1 |
| (nPDCCH-startSF-USS-r13) |  |  | 4 |
| Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission. |  |  |  |

Table 8.12.1.2.4-2: Minimum performance for 16-QAM NPDSCH under Standalone with 1 NRS port

| Test number | Bandwidth | Carrer Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Non-anchor | R.NB.8 TDD | 1 | EPA5 | 1 | 1x1 | 70% | [19.5] | NB2 |

< End of change 4>

< Start of change 5>

Table A.3.12.2.1-2: NPDSCH Reference Channel with 1Tx Antenna for UE Category NB2 for FDD

| Parameter | Unit | Value | Value | Value |
| --- | --- | --- | --- | --- |
| Reference channel |  | R.NB.7 FDD | R.NB.8 FDD | R.NB.9 FDD |
| Carrier Type |  | Non-anchor | Non-anchor | Non-anchor |
| Channel bandwidth | KHz | 200 | 200 | 200 |
| Allocated subframes per Radio Frame |  | Note 1 | Note 1 | Note 1 |
| Modulation |  | QPSK | QPSK | 16-QAM |
| ITBS/ISF |  | 9/5 | 6/7 | 21/7 |
| Target Coding Rate |  | ½ | 1/3 | 0.78 |
| Coding Rate |  | 0.5 | 0.32 | 0.78 |
| Information Bit Payload |  |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 | Bits | 936 | 1032 | 4968 |
| For Sub-Frame 0,5 | Bits | 936 | 1032 | 4968 |
| For Sub-Frame 4,9 | Bits | 936 | 1032 | 4968 |
| Number of Code Blocks |  |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 |  | 1 | 1 | 1 |
| For Sub-Frame 0,5 | Bits | 1 | 1 | 1 |
| For Sub-Frame 4,9 | Bits | 1 | 1 | 1 |
| Binary Channel Bits |  |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 | Bits | 320 | 320 | 640 |
| For Sub-Frame 0,5 | Bits | 320 | 320 | 640 |
| For Sub-Frame 4,9 | Bits | 320 | 320 | 640 |
| Max. Average Throughput | Bps | Note 2 | Note 2 | Note 2 |
| UE Category |  | NB2 | NB2 | NB2 |
| Note 1: It shall depend on the specific NPDSCH scheduling. Note 2: Maximum Average Throughput equals to sum of TB(i) divided by sum of T(i), where TB(i) is the TB size of NPDSCH over ith NPDSCH scheduling period, and T(i) is the total time consisting of NPDCCH transmission duration, NPDCCH to NPDSCH scheduling delay, NPDSCH transmission duration, NPDSCH to NPUSCH format 2 scheduling delay, NPUSCH format 2 transmission duration, possible delay between NPUSCH format 2 and NPDCCH for next NPDSCH scheduling and subframes used for NPSS/NSSS/NPBCH/NB-SIB1/NB-SIB2 transmission during the ith NPDSCH scheduling period. |  |  |  |  |

Table A.3.12.2.1-2a: NPDSCH Reference Channel with 1Tx Antenna for UE Category NB2 for TDD

| Parameter | Unit | Value | Value |
| --- | --- | --- | --- |
| Reference channel |  | R.NB.7 TDD | R.NB.8 TDD |
| Carrier Type |  | Non-anchor | Non-anchor |
| Channel bandwidth | KHz | 200 | 200 |
| Uplink-Downlink Configuration (Note 7) |  | 4 | 4 |
| Allocated subframes per Radio Frame |  | Note 1 | Note 1 |
| Modulation |  | QPSK | 16-QAM |
| ITBS/ISF |  | 9/5 | 21/7 |
| Target Coding Rate |  | 1/2 | 0.78 |
| Coding Rate |  | 0.5 | 0.78 |
| Information Bit Payload |  |  |  |
| For Sub-Frames 1,6,7,8 | Bits | 936 | 4968 |
| For Sub-Frame 0,5 | Bits | Note 2 | Note 2 |
| For Sub-Frame 4,9 | Bits | 936 | 4968 |
| Number of Code Blocks |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 |  | 1 | 1 |
| For Sub-Frame 0,5 | Bits | Note 3 | Note 3 |
| For Sub-Frame 4,9 | Bits | 1 | 1 |
| Binary Channel Bits |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 | Bits | 320 | 640 |
| For Sub-Frame 0,5 | Bits | Note 4 | Note 4 |
| For Sub-Frame 4,9 | Bits | 320 | 640 |
| Max. Average Throughput | Bps | Note 5 | Note 5 |
| UE Category |  | NB2 | NB2 |
| Note 1: It shall depend on the specific NPDSCH scheduling. Note 2: N/A when mod 2 = 0, otherwise 936 for R.NB.7 TDD and 4968 for R.NB.8 TDD. Note 3: N/A when mod 2 = 0, otherwise 1. Note 4: N/A when mod 2 = 0, otherwise 320 for R.NB.7 TDD and 640 for R.NB.8 TDD.. Note 5: Maximum Average Throughput equals to sum of TB(i) divided by sum of T(i), where TB(i) is the TB size of NPDSCH over ith NPDSCH scheduling period, and T(i) is the total time consisting of NPDCCH transmission duration, NPDCCH to NPDSCH scheduling delay, NPDSCH transmission duration, NPDSCH to NPUSCH format 2 scheduling delay, NPUSCH format 2 transmission duration, possible delay between NPUSCH format 2 and NPDCCH for next NPDSCH scheduling and subframes used for NPSS/NSSS/NPBCH/NB-SIB1/NB-SIB2 transmission during the ith NPDSCH scheduling period. |  |  |  |

< End of change 5>