3GPP TSG RAN WG4 Meeting #104-e		 R4-2212220

# Electronic Meeting, Aug 15th – 26th, 2022

Agenda Item:	10.17.2

Source: 	MediaTek Inc.

Title: 	Discussion on downlink interruption for dynamic Tx switching in R-18

Document for:	Discussion

1. Introduction

According to the approved WID on Multi-carrier enhancements for NR, Tx switching is further enhanced to support dynamic switching on more bands. The contribution provides some discussion

2. Discussion

According to the approved WID on Multi-carrier enhancements for NR, Tx switching is further enhanced to support dynamic switching across 3 or 4 bands. The second bullet in the scope of the WID[1] is copied below,

2. Study and if necessary specify following enhancements for multi-carrier UL operation [RAN1, RAN2, RAN4]

- UL Tx switching schemes across up to 3 or 4 bands with restriction of up to 2 Tx simultaneous transmission for FR1 UEs, including mechanisms to enable more configured UL bands than its simultaneous transmission capability and to support dynamic Tx carrier switching across the configured bands for both single TAG and multiple TAGs configurations (RAN1, RAN4)

  - UE capability and RRC configuration related signalling (RAN2)

  - Note: strive for RAN1/2 design agnostic with the number of bands, i.e., common design between 3 and 4 bands

  - Note: no additional TAG is introduced for UL transmission on a carrier without corresponding DL carrier

  - Note: this objective does not target to extend the SUL framework to support more than 1 SUL for 1 NUL

- Switching time and other RF aspects, and RRM requirements for above UL Tx switching schemes across up to 3 or 4 bands (RAN4)

  - Note: Prioritize UL Tx switching across up to 3 bands is to be addressed first and then that for up to 4 bands can also be addressed

In R-16 Tx switching discussion, it was mentioned LO would be affected due to load change or phase adjustment of additional routing to 2nd TX chain to keep both uplink carriers coherent in case 2 no matter case 2 is running in TX diversity mode or UL MIMO. Considering UL CA/DC operation, it would often share uplink resource, ex: digital baseband, DSP…etc., on uplink carriers. Uplink interruption on both Tx carriers shall be allowed for above consideration. There may also be intermodulation interference and transient spurs fall in receiving range of some bands thus DL interruption is also required to be considered [2].

Proposal 1: Interruption/exception requirements for both uplink and downlink transmission shall be applied in R-18 Multi-carrier enhancements also

For 3 or 4 bands scenario, UE may take longer time for downlink DCI parsing, however, UE must finish it before start of triggering Tx switching or UE would fail on Tx preparation. After DCI parsing, It is no difference for setting new Tx configurations for one of the Tx chains being switched from one band to another band, we don’t see the need to have longer Tx switching time under 3 or 4 bands operation in comparison to 2 bands. So for NR CA, since it is always synchronized operation, the procedure has no difference for one Tx switched to another Tx thus the switching time values set is not expected to be different. The existing UE capability signalling can be re-used but it may need to be extended to 3 or 4 bands cases due to more complicated Tx switching behaviour that are implementation dependent. For UL CA, timing difference between two carriers (MTTD) shall be considered for network scheduling and handled by base station when Tx switching is triggered as shown in Figure 1.

Figure 1 Time chart including MTTD

Proposal 2: Tx switching time from UE hardware preparation perspective for NR CA, the procedure has no difference for one Tx switched to another Tx thus the set of values for switching time can be re-used. But in Rel-18, the switching time may not be same with that of R-17/16 due to the band pairs of switching may belong to different higher order combinations or implementation.

Proposal 3: The UE capability signalling structure may be extended to 3 or 4 bands cases due to more complicated Tx switching behaviour that are implementation dependent.

Proposal 4: Tx switching time shall not be suppressed when there is MTTD between uplink carriers. The network scheduling shall include considering additional MTTD when Tx switching is triggered.

In the WID, multi-TAG operation is also included. In our understanding multi-TAG scenario may include NR CA/DC or MRDC. We’ve discussed NR CA above. Here we continue discuss NR DC and MR DC. Due to asynchronous DC timing difference between CGs can be up to (+ -)500us (equal to arbitrary numbers), we propose Tx switching only applies for synchronized operation. There would be no difference on UE hardware preparation procedure thus the extension of existing UE capability signalling structure of Tx switching can be considered first.  Addition of MTTD shall also be considered for network scheduling and handled by base station when Tx switching is triggered.

Proposal 5: For NR DC and MR DC, Tx switching only applies for synchronized operation. The existing UE capability signalling values set can still be re-used. But the UE capability signalling structure may be extended to 3 or 4 bands cases due to more complicated Tx switching behaviour that are implementation dependent. And Tx switching time shall not be suppressed when there is MTTD between uplink carriers. The network scheduling shall include additional MTTD when Tx switching is triggered.

Proposal 6: For MR DC, the assumption of co-located and synchronized operation remains unchanged. Tx switching only applies for the bands are co-located and synchronized.

For NR stand-alone combination including SUL band, since SUL band must have same TA as NUL, we see no difference between two bands Tx switching and 3 ro 4 bands Tx switching on Tx switching perspective. The existing capability can be re-used here. This is same on MR DC combos that including SUL bands except when there is ULSUP configuration. Considering following configuration that includes ULSUP configuration, the SUL band shares hardware path with E-UTRA uplink. If the ULSUP operation is enabled with Tx switching at the same time, there may be a problem for network scheduling as well as the complexity on UE implementation. We suggest to handle this issue as low priority or not considered in R-18.

| DC_1A_SUL_n79A-n84A | DC_1A_n79A, DC_1A_n84A_ULSUP-TDM_n79A |
| --- | --- |

Proposal 7: The existing UE capability can be re-used for 3 or 4 bands configuration that includes SUL band except when there is ULSUP configuration.

Proposal 8: For R-18 Tx switching, the combos including ULSUP are not considered.

3. Conclusion

Proposal 1: Interruption/exception requirements for both uplink and downlink transmission shall be applied in R-18 Multi-carrier enhancements also

Proposal 2: Tx switching time from UE hardware preparation perspective for NR CA, the procedure has no difference for one Tx switched to another Tx thus the set of values for switching time can be re-used. But in Rel-18, the switching time may not be same with that of R-17/16 due to the band pairs of switching may belong to different higher order combinations or implementation.

Proposal 3: The UE capability signalling structure may be extended to 3 or 4 bands cases due to more complicated Tx switching behaviour that are implementation dependent.

Proposal 4: Tx switching time shall not be suppressed when there is MTTD between uplink carriers. The network scheduling shall include considering additional MTTD when Tx switching is triggered.

Proposal 5: For NR DC and MR DC, Tx switching only applies for synchronized operation. The existing UE capability signalling values set can still be re-used. But the UE capability signalling structure may be extended to 3 or 4 bands cases due to more complicated Tx switching behaviour that are implementation dependent. And Tx switching time shall not be suppressed when there is MTTD between uplink carriers. The network scheduling shall include additional MTTD when Tx switching is triggered.

Proposal 6: For MR DC, the assumption of co-located and synchronized operation remains unchanged. Tx switching only applies for the bands are co-located and synchronized.

Proposal 7: The existing UE capability can be re-used for 3 or 4 bands configuration that includes SUL band except when there is ULSUP configuration.

Proposal 8: For R-18 Tx switching, the combos including ULSUP are not considered.

Reference:

- RP-213577 WID on Multi-carrier enhancements, RAN#94

- R4-1914272 Switching time between NR between FR1 uplink carriers, RAN4#93