3GPP TSG-RAN WG4 Meeting #104-e 	R4-2212225

Electronic meeting, 15th – 24th August, 2022

Source:	Ericsson

Title:	General view on demodulation requirements for NR extended to 71GHz

Agenda item:	9.14.8.1

Document for:	Discussion

# Introduction

In RAN4#103-e, companies discussed the general issues for FR2-2 demodulation requirements. Following agreements and open issues are captured in the WF [1].

Issue 1-1-1: Whether to define FR2-2 demodulation requirement without LBT model for both DL and UL

Rel-17 RAN4 focused on defining FR2-2 demodulation requirement without LBT model for UL/DL.

It’s not precluded to discuss and specify specific requirements with LBT model in future releases.

Issue 1-2-1: How to consider phase noise model for the FR2-2 demodulation simulation

Assume PN model in Receiver side with CPE compensation for initial simulation purpose for both DL and UL

- FFS when and how to consider ICI including performance gain, implementation complexity and test feasibility

Issue 1-2-2: Whether TE PN impact is ignored or not in the simulations

Invite TE vendors to give comment if FR2-2 phase noise impact by TE can be ignored or not.

In case there is no input in the next meeting, the group does not consider TE PN impact.

Since there is no comment from TE vendors in this meeting, we will not consider the Tx PN impact in the simulations. The Tx PN impact is considered in the impairment results pending on BS implementation.

Issue 1-3-1: How to consider the delay profile for TDLA channel model for FR2-2?

FFS the delay profile details.

Issue 1-3-2: How to consider the minimum tap resolution for TDL channel model?

Invite TE vendors to confirm the minimum tap resolution for TDL channel model they could deliver.

- Proposal 1: 2ns delay resolution

- Proposal 2: 2.5ns delay resolution

- Proposal 3: TBA

Issue 1-3-3: Channel model for UL requirements

| SCS | CBW | MCS | Channel model | SCS | CBW | MCS | Channel model | SCS | CBW | MCS | Channel model |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 120 | 100 | 4 | TDLA10-650 | 480 | 400 | 4 | TDLA10-650 | FFS 960 | 400 | 4 | TDLA10-650 |
|  |  | 16 | TDLA10-200 FFS TDLA10-650 |  |  | 16 | TDLA10-200 FFS TDLA10-650 |  |  | 16 | TDLA10-200 FFS TDLA10-650 |
|  |  | 20 | TDLD10-200 |  |  | 20 | TDLD10-200 |  |  | 20 | TDLD10-200 |
|  | 400 | 4 | TDLA10-650 |  | FFS 1600 | 4 | TDLA10-650 |  | FFS 2000 | 4 | TDLA10-650 |
|  |  | 16 | TDLA10-200 FFS TDLA10-650 |  |  | 16 | TDLA10-200 FFS TDLA10-650 |  |  | 16 | TDLA10-200 FFS TDLA10-650 |
|  |  | 20 | TDLD10-200 |  |  | 20 | TDLD10-200 |  |  | 20 | TDLD10-200 |

Note 1: Interested companies can provide simulation results for other cases not included in above table, as needed.

In this contribution, open general issues of FR2-2 demodulation requirements are further analyzed.

# Discussion

## 2.1	PN impact

Based on our simulations results [2] [3], the PN impact is small for UL cases but obvious for some DL cases. To avoid confusion, the discussion on this will be split in UL and DL separately.

Observation 1: The PN impact is a bit different for DL and UL cases.

## 2.2	Channel model

In previous meetings, it seems 10ns delay spread is a tentative agreement. There are two options for the channel tap resolution based on previous WF. Smaller resolution could support larger correlated channel bandwidth, so 2ns would be better if it’s possible for TE vendors.

To define a delay profile on TE, a proper number of taps should be defined. In previous release, 12 taps are agreed as the tap number, but it would be better that TE vendors can give confirmation on above parameters.

Proposal 1: Invite TE vendors to give confirmations on the resolution of tap delay and the number of taps for TDLA10 and TDLD10.

Just for reference, an example delay profile for TDLA10 with 2ns resolution and 12 taps is captured in Table 2.2-1. The final RMS delay is 10.01ns.

Table 2.2-1 Example of delay profile for TDLA10 with 12 taps and 2ns tap resolution

| Tap# | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -16.1 | Rayleigh |
| 2 | 4 | 0 | Rayleigh |
| 3 | 6 | -4.0 | Rayleigh |
| 4 | 8 | -10.2 | Rayleigh |
| 5 | 18 | -8.8 | Rayleigh |
| 6 | 22 | -13.7 | Rayleigh |
| 7 | 24 | -17.9 | Rayleigh |
| 8 | 26 | -13.5 | Rayleigh |
| 9 | 30 | -14.0 | Rayleigh |
| 10 | 40 | -15.4 | Rayleigh |
| 11 | 46 | -15.6 | Rayleigh |
| 12 | 96 | -23 | Rayleigh |

Based on our simulation results [2] [3], there is no big performance difference between 650Hz and 200Hz Doppler shift. For lower MCS such as MCS 4/17, it would be better to use higher Doppler shift for the requirement to cover the worst case.

Proposal 2: Use 650Hz Doppler shift for FR2-2 NLOS channel model.

# Conclusions

Observation 1: The PN impact is a bit different for DL and UL cases.

Proposal 1: Invite TE vendors to give confirmations on the resolution of tap delay and the number of taps for TDLA10 and TDLD10.

Proposal 2: Use 650Hz Doppler shift for FR2-2 NLOS channel model.

# References

[1]	R4-2210664, WF on general and BS aspects for FR2-2 demodulation requirements, Intel Corporation

[2]	R4-2212571, Simulation results for FR2-2 PDSCH demodulation requirements, Ericsson

[3]	R4-2212229, Simulation results for FR2-2 PUSCH demodulation requirements, Ericsson