3GPP TSG-RAN WG4 Meeting #104-e 	R4-2212229

Electronic meeting, 15th – 24th August, 2022

Source:	Ericsson

Title:	Simulation results for FR2-2 PUSCH demodulation requirements

Agenda item:	9.14.8.3.1

Document for:	Information

# Introduction

In the last RAN4 meeting, companies have discussions on the NR extended to 71GHz demodulation requirements. In this contribution, simulation results based on previous meeting WF [1] are delivered for further discussion.

# Simulation result

Table 2.1 Simulation parameters for FR2-2 normal PUSCH demodulation

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled and enabled |
| Default TDD UL-DL pattern (Note 1) |  | 120kHz SCS:3D1S1U, S=10D:2G:2U 480kHz SCS: 14D2S4U, S1=12D:2G:0U, S2=0D:6G:8U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | B |
|  | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain resource | RB assignment | 120kHz SCS: 100M 480kHz SCS: 400M |
|  | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS | Frequency density (KPT-RS) | 2 |
| configuration | Time density (LPT-RS) | 1 |
| Test metric | Normalized throughput | 70% |
| Antenna | Tx and Rx configuration | 1Tx 2Rx, 2Tx2Rx |
| Channel model |  | TDLA10-650, TDLA10-200, TDLD10-200 |
| MCS | 64QAM MCS table index | 4,16, 20 |
| Phase noise | Model sets in TR38.808 | Set#1 |
| Note1: Companies deliver two sets of ideal simulation results for requirement discussion. Result set#1 is without phase noise and set#2 is with phase noise. • No Tx phase noise is modelled • Rx Phase noise is modelled only to find feasible FRC configuration (i.e. achieve maximum throughput and loss in comparison to scenarios without Rx phase noise is less than 1 dB) • 70 GHz carrier frequency is assumed Note2: CPE compensation is considered. ICI compensation is considered only for MCS20 |  |  |

Table 2.2 Simulation results for FR2-2 normal PUSCH demodulation, 120kHz SCS, 100MHz

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix | Fraction of maximum throughput | MCS | PN Model | Precoding | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | No CPE | CPE |
| 1 | 2 | Normal | TDLA10-650 | 70% | 4 | Set1 | Disabled | -3.02 | -2.55 |
|  |  |  |  |  |  |  | Enabled | -3.1 | -2.8 |
|  |  |  | TDLA10-200 |  | 16 | Set1 | Disabled | 7.27 | 7.75 |
|  |  |  | TDLA10-650 |  | 16 | Set1 | Disabled | 8.04 | 8.46 |
|  |  |  | TDLD10-200 |  | 20 | Set1 | Disabled | 10.1 | 11 |
| 2 | 2 | Normal | TDLA10-650 | 70% | 4 | Set1 | Disabled | 0.52 | 1.03 |
|  |  |  | TDLA10-200 |  | 16 | Set1 | Disabled | 14.7 | 15.2 |
|  |  |  | TDLA10-650 |  | 16 | Set1 | Disabled | 15.3 | 15.9 |

Table 2.3 Simulation results for FR2-2 normal PUSCH demodulation, 480kHz SCS, 400MHz

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix | Fraction of maximum throughput | MCS | PN Model | Precoding | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | No CPE | CPE |
| 1 | 2 | Normal | TDLA10-650 | 70% | 4 | Set1 | Disabled | -2.71 | -2.36 |
|  |  |  |  |  |  |  | Enabled |  |  |
|  |  |  | TDLA10-200 |  | 16 | Set1 | Disabled | 7.22 | 7.72 |
|  |  |  | TDLA10-650 |  | 16 | Set1 | Disabled | 8.27 | 8.55 |
|  |  |  | TDLD10-200 |  | 20 | Set1 | Disabled | 10.45 | 11.31 |
| 2 | 2 | Normal | TDLA10-650 | 70% | 4 | Set1 | Disabled | 1.12 | 1.46 |
|  |  |  | TDLA10-200 |  | 16 | Set1 | Disabled | 14.7 | 15.1 |
|  |  |  | TDLA10-650 |  | 16 | Set1 | Disabled | 15.3 | 15.5 |

# Conclusions

Simulation results for FR2-2 PUSCH demodulation are delivered.

# References

[1]	R4-2210664, WF for NR extended to 71GHz BS demodulation, Intel, RAN4#103-e