3GPP TSG-RAN WG4 Meeting #104-e 	R4-2212237

Electronic meeting, 15th – 24th August, 2022

Source:	Ericsson

Title:	Discussion on SAN PRACH demodulation requirements

Agenda item:	9.11.7.2.3

Document for:	Discussion

# Introduction

In the last RAN4#103-e meeting, companies have discussions on the non-terrestrial network demodulation requirements. Following WF [1] was agreed for SAN PRACH demodulation requirement issues.

Preamble Format

Agreements

- Including format 0, B4 and C2

- FFS for format 2 and A2

SCS

Agreements

- Long preamble format (if agreed): 1.25kHz

- Short preamble format (if agreed): 15kHz and 30kHz SCS.

Sequence length for short preamble format

Agreements

- Only consider LRA=139 for short preamble format.

Antenna configuration

Agreements

- Align with the PUSCH

Time error tolerance

Agreements

- Use same time error tolerance of AWGN as Rel-15 FR1 for 1.25kHz, 15kHz and 30kHz SCS.

- Use the same method to calculate time error tolerance of multi-path channel as Rel-15: 0.522μ+Tdelay where Tdelay is corresponding to the tap delay of the propagation channel

    - Option 1: The largest delay of the propagation channel

    - Option 2: The second largest delay of the propagation channel

Frequency offset for PRACH Channel model

Agreements

- AWGN: 0Hz

- Fading channel: 200Hz (0.1ppm@2GHz)

Ncs

Agreements

- Use the same Ncs, Logical sequence index and v value as Rel-15 normal PRACH requirement, i.e.

| Burst format | SCS (kHz) | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- |
| 0 | 1.25 | 13 | 22 | 32 |
| [A2],B4,C2 | 15 | 23 | 0 | 0 |
|  | 30 | 46 | 0 | 0 |

In this contribution, open issues for SAN PRACH demodulation will be further analyzed.

# Discussion

Preamble Format

Regarding the UE pre-compensation, timing error between different UE would be quite small and the propagation delay impact could also be ignored. The remaining issue might be the total PRACH signal power considering very large propagation path loss. The PRACH detection is based on the preamble correlation, so the preamble duration would basically show the power level after correlation.

FSPL and Rx G/T for GEO, LEO600 and LEO1200 in Table 2-1 below (based on link budget in TR38.821). Total gain = Rx G/T – FSPL, and relative gain is the total gain difference compared to GEO. We can see that the gain difference for LEO600 is 13.4dB and LEO1200 is 8dB.

Table 2-1 Path gain for different NTN deployment

| Deployment | FSPL [dB] | Rx G/T [dB] | Total Gain [dB] | Relative Gain [dB] |
| --- | --- | --- | --- | --- |
| GEO | 190.4 | 19 | -171.4 | 0 |
| LEO600 | 159.1 | 1.1 | -158.0 | 13.4 |
| LEO1200 | 164.5 | 1.1 | -163.4 | 8 |

Table 2-2 includes all formats duration without CP /GP. The relative gain is calculated by 10*log(FormatX_duration/Format2_duration).

Table 2-2 Preamble format duration gain

| Format | Preamble length [Ts] | Relative Gain [dB] |
| --- | --- | --- |
| 0 | 24756 | -6.0 |
| 1 | 49512 | -3.0 |
| 2 | 99024 | 0.0 |
| 3 | 24576 | -6.0 |
| A1 | 4096 | -13.8 |
| A2 | 8192 | -10.8 |
| A3 | 12288 | -9.0 |
| B0 | 2048 | -16.8 |
| B1 | 4096 | -13.8 |
| B2 | 8192 | -10.8 |
| B3 | 12288 | -9.0 |
| B4 | 24576 | -6.0 |
| C0 | 2048 | -16.8 |
| C1 | 4096 | -13.8 |
| C2 | 8192 | -10.8 |

For example, if we assume format 2 could be used for GEO in the worst case, then formats with relative gain <13.4dB could be possibly applied for LEO600 and formats with relative gain <8dB could be possibly applied for LEO1200.  In that case, format 0/1/2/3/A2/A3/B2/B3/B4/C2 can also be applied to LEO600, and format 0/1/2/3/B4 can also be applied to LEO1200.

Regarding the typical formats supported by current vendors, format 0, B4 and C2 could be enough for checking performance. Format 2 has longer preamble length and coverage, but its format structure is similar as format 0. A2 is expected to have similar performance as C2 based on previous simulations in Rel-15 and Rel-16. In that case, it’s no need to add extra tests for format 2 and A2.

Proposal 1: Don’t consider format 2 and A2 for the SAN PRACH demodulation requirements.

Time error tolerance

Based on our simulation results [2], there is no clear difference between the largest tap delay and the second largest tap delay for NTN-TDLA100-220/ NTN-TDLA150-220 channel.  Anyway, the power for the largest delay tap is not very small that can be ignored, so we still prefer to take the largest tap delay for time error tolerance.

Proposal 2: Take the largest tap delay of NTN-TDL-A for time error tolerance calculation.

# Conclusions

Proposal 1: Don’t consider format 2 and A2 for the SAN PRACH demodulation requirements.

Proposal 2: Take the largest tap delay of NTN-TDL-A for time error tolerance calculation.

# References

[1]	R4-2210662, WF for NTN SAN demodulation requirements, Huawei, HiSilicon

[2]	R4-2212240, Simulation results for SAN PRACH demodulation requirements, Ericsson

[3]