3GPP TSG-RAN WG4 Meeting #104-e 	R4-2212239

Electronic meeting, 15th – 24th August, 2022

Source:	Ericsson

Title:	Simulation results for NR SAN PUCCH demodulation requirements

Agenda item:	9.11.7.2.2

Document for:	Information

# Introduction

In the last RAN4 meeting, companies have discussions on the SAN PUCCH demodulation requirements. In this contribution, simulation results based on previous meeting WF [1] are delivered for further discussion.

# Simulation result

Table 2-1 Simulation parameters for NTN PUCCH

| Parameter | Format 0 | Format 1 | Format 2 |  | Format 3 | Format 4 |
| --- | --- | --- | --- | --- | --- | --- |
| number of UCI information bits | 1 | 2 | 4 | 22 | 16 | 22 |
| Number of PRBs | 1 | 1 | 4 | 4 | 1 | 1 |
| Number of symbols | 1 | 14 | 1 | 2 | 14 | 14 |
| First PRB prior to frequency hopping | 0 | 0 | 0 | 0 | 0 | 0 |
| Intra-slot frequency hopping | N/A | Enabled | N/A | Enabled | Enabled | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |  |  |  |  |  |
| Group and sequence hopping | neither | neither | Neither | Neither | Neither | Neither |
| Hopping ID | 0 | 0 | 0 | 0 | 0 | 0 |
| Initial cyclic shift | 0 | 0 | 0 | 0 | 0 |  |
| First symbol | 13 | 0 | 13 | 12 | 0 | 0 |
| Index of orthogonal cover code (timeDomainOCC) | N/A | 0 | N/A | N/A | N/A | n0 |
| Length of the orthogonal cover code | N/A | N/A | N/A | N/A | N/A | N2 |
| Test Metric | D2A error <1% and ACK miss <1% | NACK -> ACK <0.1% ACK miss <1% | ACK miss <1% | UCI BLER<1% | UCI BLER<1% | UCI BLER<1% |

Table 2-2.1 Simulation results for NTN PUCCH PF0 with 15kHz SCS

| Number | Number of | Propagation conditions and | Number of PUSCH PRB |  |
| --- | --- | --- | --- | --- |
| of TX antennas | RX antennas | correlation matrix |  | SNR [dB] |
| 1 | 1 | NTN-TDLA100-200 | 4 | 15.3 |
|  |  | NTN-TDLC10-200 | 4 | 12.7 |
| 1 | 2 | NTN-TDLA100-200 | 4 | 6.9 |
|  |  | NTN-TDLC10-200 | 4 | 4.6 |

Table 2-2.2 Simulation results for NTN PUCCH PF0 with 30kHz SCS

| Number | Number of | Propagation conditions and | Number of PUSCH PRB |  |
| --- | --- | --- | --- | --- |
| of TX antennas | RX antennas | correlation matrix |  | SNR [dB] |
| 1 | 1 | NTN-TDLA100-200 | 4 | 16 |
|  |  | NTN-TDLC10-200 | 4 | 12 |
| 1 | 2 | NTN-TDLA100-200 | 4 | 6.9 |
|  |  | NTN-TDLC10-200 | 4 | 4.7 |

Table 2-3.1 Simulation results for NTN PUCCH PF1 with 15kHz SCS, NACK->ACK <0.1%

| Number | Number of | Propagation conditions and | Number of PUSCH PRB |  |
| --- | --- | --- | --- | --- |
| of TX antennas | RX antennas | correlation matrix |  | SNR [dB] |
| 1 | 1 | NTN-TDLA100-200 | 4 | -2.3 |
|  |  | NTN-TDLC10-200 | 4 | -2.4 |
| 1 | 2 | NTN-TDLA100-200 | 4 | -7.4 |
|  |  | NTN-TDLC10-200 | 4 | -7.9 |

Table 2-3.2 Simulation results for NTN PUCCH PF1 with 30kHz SCS, NACK->ACK <0.1%

| Number | Number of | Propagation conditions and | Number of PUSCH PRB |  |
| --- | --- | --- | --- | --- |
| of TX antennas | RX antennas | correlation matrix |  | SNR [dB] |
| 1 | 1 | NTN-TDLA100-200 | 4 | 0.6 |
|  |  | NTN-TDLC10-200 | 4 | -1.8 |
| 1 | 2 | NTN-TDLA100-200 | 4 | -6.2 |
|  |  | NTN-TDLC10-200 | 4 | -7.6 |

Table 2-4.1 Simulation results for NTN PUCCH PF1 with 15kHz SCS, ACK miss <1%

| Number | Number of | Propagation conditions and | Number of PUSCH PRB |  |
| --- | --- | --- | --- | --- |
| of TX antennas | RX antennas | correlation matrix |  | SNR [dB] |
| 1 | 1 | NTN-TDLA100-200 | 4 | 0.2 |
|  |  | NTN-TDLC10-200 | 4 | -0.4 |
| 1 | 2 | NTN-TDLA100-200 | 4 | -6.2 |
|  |  | NTN-TDLC10-200 | 4 | -6.5 |

Table 2-4.2 Simulation results for NTN PUCCH PF1 with 30kHz SCS, ACK miss <1%

| Number | Number of | Propagation conditions and | Number of PUSCH PRB |  |
| --- | --- | --- | --- | --- |
| of TX antennas | RX antennas | correlation matrix |  | SNR [dB] |
| 1 | 1 | NTN-TDLA100-200 | 4 | 2.7 |
|  |  | NTN-TDLC10-200 | 4 | 1 |
| 1 | 2 | NTN-TDLA100-200 | 4 | -4.1 |
|  |  | NTN-TDLC10-200 | 4 | -5.8 |

Table 2-5.1 Simulation results for NTN PUCCH PF2 with 15kHz SCS, ACK miss <1%

| Number | Number of | Propagation conditions and | Number of PUSCH PRB |  |
| --- | --- | --- | --- | --- |
| of TX antennas | RX antennas | correlation matrix |  | SNR [dB] |
| 1 | 1 | NTN-TDLA100-200 | 4 | 12.7 |
|  |  | NTN-TDLC10-200 | 4 | 11 |
| 1 | 2 | NTN-TDLA100-200 | 4 | 3 |
|  |  | NTN-TDLC10-200 | 4 | 0.4 |

Table 2-5.2 Simulation results for NTN PUCCH PF2 with 30kHz SCS, ACK miss <1%

| Number | Number of | Propagation conditions and | Number of PUSCH PRB |  |
| --- | --- | --- | --- | --- |
| of TX antennas | RX antennas | correlation matrix |  | SNR [dB] |
| 1 | 1 | NTN-TDLA100-200 | 4 | 11 |
|  |  | NTN-TDLC10-200 | 4 | 10.4 |
| 1 | 2 | NTN-TDLA100-200 | 4 | 3.1 |
|  |  | NTN-TDLC10-200 | 4 | 0.6 |

Table 2-6.1 Simulation results for NTN PUCCH PF2 with 15kHz SCS, UCI BLER <1%

| Number | Number of | Propagation conditions and | Number of PUSCH PRB |  |
| --- | --- | --- | --- | --- |
| of TX antennas | RX antennas | correlation matrix |  | SNR [dB] |
| 1 | 1 | NTN-TDLA100-200 | 4 | 9.2 |
|  |  | NTN-TDLC10-200 | 4 | 10.9 |
| 1 | 2 | NTN-TDLA100-200 | 4 | 2.7 |
|  |  | NTN-TDLC10-200 | 4 | 2.7 |

Table 2-6.2 Simulation results for NTN PUCCH PF2 with 30kHz SCS, UCI BLER <1%

| Number | Number of | Propagation conditions and | Number of PUSCH PRB |  |
| --- | --- | --- | --- | --- |
| of TX antennas | RX antennas | correlation matrix |  | SNR [dB] |
| 1 | 1 | NTN-TDLA100-200 | 4 | 11.7 |
|  |  | NTN-TDLC10-200 | 4 | 10.9 |
| 1 | 2 | NTN-TDLA100-200 | 4 | 4.5 |
|  |  | NTN-TDLC10-200 | 4 | 2.8 |

Table 2-7.1 Simulation results for NTN PUCCH PF3 with 15kHz SCS, UCI BLER <1%

| Number | Number of | Propagation conditions and | Number of PUSCH PRB |  |
| --- | --- | --- | --- | --- |
| of TX antennas | RX antennas | correlation matrix |  | SNR [dB] |
| 1 | 1 | NTN-TDLA100-200 | 4 | 4 |
|  |  | NTN-TDLC10-200 | 4 | 3.7 |
| 1 | 2 | NTN-TDLA100-200 | 4 | -2.5 |
|  |  | NTN-TDLC10-200 | 4 | -2.8 |

Table 2-7.2 Simulation results for NTN PUCCH PF3 with 30kHz SCS, UCI BLER <1%

| Number | Number of | Propagation conditions and | Number of PUSCH PRB |  |
| --- | --- | --- | --- | --- |
| of TX antennas | RX antennas | correlation matrix |  | SNR [dB] |
| 1 | 1 | NTN-TDLA100-200 | 4 | 6.9 |
|  |  | NTN-TDLC10-200 | 4 | 5 |
| 1 | 2 | NTN-TDLA100-200 | 4 | -0.4 |
|  |  | NTN-TDLC10-200 | 4 | -2.3 |

Table 2-8.1 Simulation results for NTN PUCCH PF4 with 15kHz SCS, UCI BLER <1%

| Number | Number of | Propagation conditions and | Number of PUSCH PRB | Additional DM-RS pos |  |
| --- | --- | --- | --- | --- | --- |
| of TX antennas | RX antennas | correlation matrix |  |  | SNR [dB] |
| 1 | 1 | NTN-TDLA100-200 | 4 | Pos0 | 6.8 |
|  |  |  |  | Pos1 | 6.8 |
|  |  | NTN-TDLC10-200 | 4 | Pos0 | 6.3 |
|  |  |  |  | Pos1 | 6.6 |
| 1 | 2 | NTN-TDLA100-200 | 4 | Pos0 | 0.2 |
|  |  |  |  | Pos1 | -0.4 |
|  |  | NTN-TDLC10-200 | 4 | Pos0 | -0.1 |
|  |  |  |  | Pos1 | -0.7 |

Table 2-8.2 Simulation results for NTN PUCCH PF4 with 30kHz SCS, UCI BLER <1%

| Number | Number of | Propagation conditions and | Number of PUSCH PRB | Additional DM-RS pos |  |
| --- | --- | --- | --- | --- | --- |
| of TX antennas | RX antennas | correlation matrix |  |  | SNR [dB] |
| 1 | 1 | NTN-TDLA100-200 | 4 | Pos0 | 9.5 |
|  |  |  |  | Pos1 | 9.5 |
|  |  | NTN-TDLC10-200 | 4 | Pos0 | 7.7 |
|  |  |  |  | Pos1 | 7.6 |
| 1 | 2 | NTN-TDLA100-200 | 4 | Pos0 | 2.2 |
|  |  |  |  | Pos1 | 1.8 |
|  |  | NTN-TDLC10-200 | 4 | Pos0 | 0.2 |
|  |  |  |  | Pos1 | -0.3 |

# Conclusions

Simulation results for SAN PUCCH demodulation requirements are delivered.

# References

[1]	R4-2210662, WF for NTN NR SAN demodulation, Huawei HI Silicon, RAN4#103-e

# Annex A Channel model for NTN simulation

Table A-1 NTN-TDLA RMS delay spread (resolution 5ns)

| Tap # | Normalized delay | Power in [dB] | Fading distribution | Delay spread |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 100ns | 150ns | 200ns |
| 1 | 0 | 0 | Rayleigh | 0 | 0 | 0 |
| 2 | 1.0811 | -4.675 | Rayleigh | 110 | 160 | 215 |
| 3 | 2.8416 | -6.482 | Rayleigh | 285 | 425 | 570 |

Table A-2 NTN-TDLC10 delay profile (resolution 5ns)

| Tap # | Normalized delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.394 | LOS path |
|  | 0 | -3.62 | Rayleigh |
| 2 | 80 | -16.37 | Rayleigh |
| NOTE: The first tap follows a Ricean distribution with a K-factor of K1 = 3 dB and a mean power of 0 dB. |  |  |  |