3GPP TSG-RAN WG4 Meeting #104-e 	R4-2212242

Electronic meeting, 15th – 24th August, 2022

Source:	Ericsson

Title:	Discussion on PUCCH demodulation requirements for NR coverage enhancement

Agenda item:	9.16.2.2

Document for:	Discussion

# Introduction

In previous RAN4#103-e meeting, companies discussed the PUCCH requirement for NR coverage enhancement. The agreements are captured in WF [1].

Issue 2-1-1: Configured TDW length for JCE in BS PUCCH demod requirements

Agreement: – Agree to follow PUSCH conclusion for configured TDW length for JCE in BS PUCCH demod requirements

Issue 2-1-2: Number of repetitions for BS PUCCH demodulation requirements with JCE

Agreement: – Agree to follow PUSCH conclusion for the number of repetitions for BS PUCCH demodulation requirements with JCE

Issue 2-1-3: Frequency hopping for BS PUCCH demodulation requirements with JCE

Agreement: – Agree to disable Inter-slot frequency hopping for BS PUCCH demodulation requirements with JCE.

Issue 2-1-4: Antenna configuration for BS PUCCH demodulation requirements with JCE

Agreement: – Agree to use 2 Rx only for BS PUCCH demodulation requirements with JCE.

Issue 2-1-5: Frequency range coverage for BS PUCCH demodulation requirements with JCE

Agreement: – Agree to use the same approach as PUSCH JCE for frequency range coverage for BS PUCCH demodulation requirements with JCE

Issue 2-1-7: Other parameters for BS PUCCH demodulation requirements with JCE

Agreement: Agree to apply the existing test parameters specified in Rel-15 for PUCCH requirement with JCE as starting point

In this contribution, remaining open issues of PUCCH with JCE demodulation requirements are further analyzed.

# Discussion

Issue 2-1-6: DMRS configuration for PUCCH format 3

- Option 1: only cover without additional DMRS

- Option 2: Cover both with and without additional DMRS

- Make decision on this issue in the next meeting.

- Encourage companies to bring simulation results for both DMRS1+1 and DMRS1+0 for the next meeting

For long OFDM symbol allocation, DM-RS 1+1 configuration should be more typical for JCE feature in coverage limited scenario. Network would schedule more DM-RS to improve the channel estimation performance to get better throughput rather than reducing DM-RS. In that case, only DM-RS 1+1 configuration could be enough for JCE demodulation requirement.

On the other hand, there is a manufacture declaration D.104 for additional DM-RS for PUCCH format 3.  It might be suitable to cover both DM-RS 1+0 and 1+1 configurations to avoid no requirements for a BS declare only support one configuration. But in previous release, there are requirements only defined for one DM-RS configuration even we have corresponding manufacture declarations in PUSCH or PUCCH. It seems not an issue to define requirements only for the typical one.

Based on our simulation results [2], DM-RS 1+0 and 1+1 have very similar performance under current assumptions. An applicability rule could be added if companies agree to use one requirement to cover both configurations.

Proposal 1: Introduce PUCCH format 3 with JCE demodulation requirements for only DM-RS 1+1 configurations.

# Conclusions

Proposal 1: Introduce PUCCH format 3 with JCE demodulation requirements for only DM-RS 1+1 configurations.

# References

[1]	R4-2210667, WF for PUCCH demodulation requirement for Rel-17 NR coverage enhancement, Huawei Hi Silicon, RAN4#103-e

[2]	R4-2212244, Simulation results for PUCCH with JCE, Ericsson