3GPP TSG-RAN WG4 Meeting #104-e 	R4-2212243

Electronic meeting, 15th – 24th August, 2022

Source:	Ericsson

Title:	Simulation results for NR coverage enhancement PUSCH demodulation requirements

Agenda item:	9.16.2.1

Document for:	Information

# Introduction

In the last RAN4 meeting, companies have discussions on the NR coverage enhancement demodulation requirements. In this contribution, simulation results based on previous meeting WF [1] are delivered for further discussion.

# Simulation result

## 2.1	PUSCH with TBoMS

Table 2.1-1 Simulation parameters for PUSCH with TBoMS for FR1

| Parameter |  | Value |
| --- | --- | --- |
| Test metric |  | SNR@70% throughput |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern |  | 15 kHz SCS: FDD and 3D1S1U, S=10D:2G:2U 30 kHz SCS: FDD and 7D1S2U, S=6D:4G:4U |
| Antenna Configuration |  | 1Tx2RX |
| Propagation conditions and correlation matrix |  | TDLB100-400 Low |
| MCS |  | MCS2 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A and B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | Slot assignment | 4 slots for FDD 2 slots for TDD |
|  | Repetition | Disabled |
| Frequency domain resource assignment | RB assignment | 5 PRBs |
|  | Frequency hopping | Disabled |

Table 2.1-2 Simulation results for PUSCH with TBoMS, 5PRB, 15kHz SCS

| Tx | Rx | channel | MCS | Available slots | DM-RS | Mapping type | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLB100-400 | 2 | 2 | 1+1 | A | -4.16 |
|  |  |  |  |  |  | B | -4.09 |
|  |  |  |  | 4 | 1+1 | A | -4.35 |
|  |  |  |  |  |  | B | -4.16 |

Table 2.1-3 Simulation results for PUSCH with TBoMS, 5PRB, 30kHz SCS

| Tx | Rx | channel | MCS | Available slots | DM-RS | Mapping type | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLB100-400 | 2 | 2 | 1+1 | A | -4.32 |
|  |  |  |  |  |  | B | -4.3 |
|  |  |  |  | 4 | 1+1 | A | -4.47 |
|  |  |  |  |  |  | B | -4.43 |

Table 2.1-4 Simulation parameters for PUSCH with TBoMS for FR2

| Parameter |  | Value |
| --- | --- | --- |
| Test metric |  | SNR@70% throughput |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern |  | 60kHz/120kHz SCS: 3D1S1U, S=10D:2G:2U |
| Antenna Configuration |  | 1Tx2RX |
| Propagation conditions and correlation matrix |  | TDLA30-300 Low |
| MCS |  | MCS2 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | B |
|  | Start symbol | 0 |
|  | Allocation length | 10 |
|  | Slot assignment | 2 slots |
|  | Repetition | Disabled |
| Frequency domain resource assignment | RB assignment | 5 PRBs |
|  | Frequency hopping | Disabled |
| PT-RS configuration | Frequency density (KPT-RS) | Disabled |
|  | Time density (LPT-RS) | Disabled |

Table 2.1-5 Simulation results for PUSCH with TBoMS, 5PRB, 60kHz SCS

| Tx | Rx | channel | MCS | Available slots | DM-RS | Mapping type | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA30-300 | 2 | 2 | 1+1 | B | -4.65 |

Table 2.1-3 Simulation results for PUSCH with TBoMS, 5PRB, 120kHz SCS

| Tx | Rx | channel | MCS | Available slots | DM-RS | Mapping type | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA30-300 | 2 | 2 | 1+1 | B | -4.37 |

## 2.2	PUSCH with JCE

Table 2.2-1 Simulation assumptions for PUSCH with JCE for FR1

| Parameter |  | Value |
| --- | --- | --- |
| Test metric |  | SNR@70% throughput |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern |  | 15 kHz SCS: FDD and 7D1S2U, S=6D:4G:4U 30 kHz SCS: FDD and 7D1S2U, S=6D:4G:4U |
| Antena Configuration |  | 1Tx 2/4/8Rx |
| Propagation conditions and correlation matrix |  | TDLA30-10 Low |
| Code rate |  | MCS4 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | [0, 2, 3, 1] for FDD [0, 3, 0, 3] for TDD |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | Pos0 and pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A and B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH repetition type | back-to-back PUSCH, Type A |
|  | PUSCH repetition factor | 2 for TDD, 8 for FDD |
|  | AvailableSlotCounting | Enabled |
| JCE configuration | PUSCH-TimeDomainWindowLength (i.e. cTDW) | 2 for TDD, 8 for FDD |
|  | Actural window length (aTDW) | 2 for TDD, 8 for FDD |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disable |
| Code block group based PUSCH transmission |  | Disabled |
| Phase offset model |  | Disabled |
| Initial CFO |  | 0ppm |

Table 2.2-2 Simulation results for PUSCH with JCE, 15kHz SCS, 5MHz BW

| Tx | Rx | channel | MCS | cTDW | DM-RS | Mapping type | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA30-10 | 4 | 2 | 1+0 | A | -6.32 |
|  |  |  |  |  |  | B | -6.32 |
|  |  |  |  |  | 1+1 | A | -6.47 |
|  |  |  |  |  |  | B | -6.48 |
|  |  |  |  | 8 | 1+0 | A | -12.1 |
|  |  |  |  |  |  | B | -12.1 |
|  |  |  |  |  | 1+1 | A | -12.3 |
|  |  |  |  |  |  | B | -12.3 |
| 1 | 4 | TDLA30-10 | 4 | 2 | 1+0 | A | -11.6 |
|  |  |  |  |  |  | B | -11.6 |
|  |  |  |  |  | 1+1 | A | -11.7 |
|  |  |  |  |  |  | B | -11.7 |
|  |  |  |  | 8 | 1+0 | A | -15.9 |
|  |  |  |  |  |  | B | -15.9 |
|  |  |  |  |  | 1+1 | A | -16.1 |
|  |  |  |  |  |  | B | -16.1 |
| 1 | 8 | TDLA30-10 | 4 | 2 | 1+0 | A | -13.6 |
|  |  |  |  |  |  | B | -13.6 |
|  |  |  |  |  | 1+1 | A | -13.7 |
|  |  |  |  |  |  | B | -13.7 |
|  |  |  |  | 8 | 1+0 | A | -19.5 |
|  |  |  |  |  |  | B | -19.5 |
|  |  |  |  |  | 1+1 | A | -19.5 |
|  |  |  |  |  |  | B | -19.5 |

Table 2.2-3 Simulation results for PUSCH with JCE, 30kHz SCS, 10MHz BW

| Tx | Rx | channel | MCS | cTDW | DM-RS | Mapping type | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA30-10 | 4 | 2 | 1+0 | A | -6.74 |
|  |  |  |  |  |  | B | -6.73 |
|  |  |  |  |  | 1+1 | A | -6.82 |
|  |  |  |  |  |  | B | -6.82 |
|  |  |  |  | 8 | 1+0 | A | -12.1 |
|  |  |  |  |  |  | B | -12.1 |
|  |  |  |  |  | 1+1 | A | -12.1 |
|  |  |  |  |  |  | B | -12.1 |
| 1 | 4 | TDLA30-10 | 4 | 2 | 1+0 | A | -9.18 |
|  |  |  |  |  |  | B | -9.17 |
|  |  |  |  |  | 1+1 | A | -9.34 |
|  |  |  |  |  |  | B | -9.34 |
|  |  |  |  | 8 | 1+0 | A | -15.9 |
|  |  |  |  |  |  | B | -15.9 |
|  |  |  |  |  | 1+1 | A | -16 |
|  |  |  |  |  |  | B | -16 |
| 1 | 8 | TDLA30-10 | 4 | 2 | 1+0 | A | -12.1 |
|  |  |  |  |  |  | B | -12.1 |
|  |  |  |  |  | 1+1 | A | -12.4 |
|  |  |  |  |  |  | B | -12.4 |
|  |  |  |  | 8 | 1+0 | A | -17.4 |
|  |  |  |  |  |  | B | -17.4 |
|  |  |  |  |  | 1+1 | A | -17.6 |
|  |  |  |  |  |  | B | -17.6 |

Table 2.2-4 Simulation assumptions for PUSCH with JCE for FR2

| Parameter |  | Value |
| --- | --- | --- |
| Test metric |  | SNR@70% throughput |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern |  | 60kHz/120kHz SCS: DDSUU, S=10G:2G:2U |
| Antena Configuration |  | 1Tx 2Rx |
| Propagation conditions and correlation matrix |  | TDLA30-75 Low |
| Code rate |  | MCS4 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | [0, 2, 3, 1] for TDD |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH repetition type | back-to-back PUSCH, Type A |
|  | PUSCH repetition factor | 2 slots |
|  | AvailableSlotCounting | Enabled |
| JCE configuration | PUSCH-TimeDomainWindowLength (i.e. cTDW) | 2 slots |
|  | Actural window length (aTDW) | 2 slots |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disable |
| PT-RS configuration | Frequency density (KPT-RS) | Disabled |
|  | Time density (LPT-RS) | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Phase offset model |  | Disabled |
| Initial CFO |  | 0ppm |

Table 2.2-5 Simulation results for PUSCH with JCE, 60kHz SCS, 50MHz BW

| Tx | Rx | channel | MCS | cTDW | DM-RS | Mapping type | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA30-75 | 4 | 2 | 1+1 | B | -6.53 |

Table 2.2-6 Simulation results for PUSCH with JCE, 120kHz SCS, 50MHz BW

| Tx | Rx | channel | MCS | cTDW | DM-RS | Mapping type | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA30-75 | 4 | 2 | 1+1 | B | -6.58 |

# Conclusions

Simulation results for NR coverage enhancement PUSCH with TBoMS and JCE requirement are delivered.

# References

[1]	R4-2210666, WF for NR PUSCH coverage enhancement, China Telecom, RAN4#103-e