3GPP TSG-RAN WG4 Meeting #104-e 	R4-2212248

Electronic meeting, 15th – 24th August, 2022

Source:	Ericsson

Title:	Discussion on sub-slot PUCCH repetition demodulation requirements

Agenda item:	9.21.3.1

Document for:	Discussion

# Introduction

In the last RAN4 meeting, companies have discussions on the URLLC demodulation requirements. The agreements are captured in WF [1] as follows.

Issue 1-1-1: Introduce PUSCH requirements

Agreements:

Do not introduce PUSCH requirements.

Issue 1-2-1: Introduce PUCCH requirements

Agreements:

Option 2a as starting point and further confirm by RAN4#104-e meeting.

- Option 2a: Develop PUCCH requirements for PF0, with sub slot repetition and inter sub-slot hopping, but condition introduction in the specification on the availability of at least 2 simulation inputs

Issue 2-1-1: Introduce CQI requirements

Agreements:

Do not introduce CQI requirements for subband 4-bits based CQI reporting.

In this contribution, open issues for sub-slot PUCCH repetition are further analyzed.

# Discussion

Supporting sub-slot PUCCH repetition HARQ-ACK, BS needs new functional implementation in to process multiple PUCCH within a slot. It would be necessary to test this feature according to the manufacture declaration.

Proposal 1: Define new requirement for sub-slot PUCCH repetition HARQ-ACK.

To reduce the test and simulation effort, only 1 test case of format 0 could be enough for the requirement. Referring Rel-15 configurations and some companies’ proposal in the last meeting, following parameters configurations in Table 2-1 could be applied.

Table 2-1: Test Parameters

| Parameter | Test |
| --- | --- |
| Number of sub-slots | 2 |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| Number of symbols in a sub-slot | 2 |
| First PRB prior to frequency hopping | 0 |
| Intra-sub-slot frequency hopping | N/A |
| Inter-sub-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 5, 12 |

Other parameters could be as follows to limit the number of tests.

- Channel model: TDLC-300-100 Low

- SCS and CBW: 5MHz 15kHz SCS and 10MHz 30kHz SCS

- Antenna configuration: 1Tx2Rx

Proposal 2: Take above parameter configurations for sub-slot PUCCH format 0 repetition demodulation requirements if it is agreed to be introduced.

# Conclusions

Proposal 1: Define new requirement for sub-slot PUCCH repetition HARQ-ACK.

Proposal 2: Take following parameter configurations for sub-slot PUCCH format 0 repetition demodulation requirements if it is agreed to be introduced.

Test Parameters

| Parameter | Test |
| --- | --- |
| Number of sub-slots | 2 |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| Number of symbols in a sub-slot | 2 |
| First PRB prior to frequency hopping | 0 |
| Intra-sub-slot frequency hopping | N/A |
| Inter-sub-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 5, 12 |
| Channel model | TDLC-300-100 Low |
| SCS and CBW | 5MHz 15kHz SCS and 10MHz 30kHz SCS |
| Antenna configuration | 1Tx2Rx |

# References

[1]	R4-2210673, WF for enhanced IIOT and URLLC demod and CSI, Nokia, RAN4#103-e