3GPP TSG- RAN WG4 Meeting #104-e	R4-2212292

# Electronic Meeting, Aug 15 – Aug 26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR |  | rev |  | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS38.101-4 PDSCH TDD demodulation requirements for inter-cell interference MMSE-IRC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_enh2-Perf |  |  |  |  |  | Date: |  |  | 2022-07-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The agreement for TBD part in PDSCH demodulation requirements for inter-cell interference MMSE-IRC has been achieved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Remove the PDCCH configuration from test parameter table 2) Add the SNR values 3) Remove the square bracket on Interference Model |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PDSCH demodulation requirements for inter-cell interference MMSE-IRC will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2.16, 5.2.3.2.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | Y |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

-----------------Change 1---------------------

##### 5.2.2.2.16	Minimum requirements for PDSCH with inter-cell interference

The performance requirements are specified in Table 5.2.2.2.16-3, with the addition of test parameters in Table 5.2.2.2.16-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.2.16-1.

Table 5.2.2.2.16-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH performance under 2 receive antenna conditions, when transmission from the serving cell is interfered by 1 or 2 interfering cells. | 1-1, 1-2 |

Table 5.2.2.2.16-2: Test parameters

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 |
|  |  |  | Enabled | Enabled | Enabled for test 1-1 Disabled for test 1-2 |
| Duplex mode |  |  | TDD |  |  |
| TDD UL-DL pattern |  |  | FR1.30-1 |  |  |
| Active DL BWP index |  |  | 1 |  |  |
| Physical cell ID |  |  | 0 | 1 | 2 |
| Transmission rank |  |  | 1 | Random rank with 70% and 30% probability for rank 1 and rank 2 | Random rank with 70% and 30% probability for rank 1 and rank 2 for Test 1-1 N/A for Test 1-2 |
| Time offset between Cells |  | us | 0 | 1.5 | -0.5 |
| Frequency shift between Cells |  | Hz | 0 | 300 | -100 |
| Interference Model |  |  | N/A | As specified in B.6.2 |  |
| INR |  | dB | N/A | 7.77 for Test 1-1 7.58 for Test 1-2 | 2.29 for Test 1-1 N/A for Test 1-2 |
| SSB configuration | SSB position in burst |  | First SSB in Slot #0 | First SSB in Slot #0 for Test 1-1 Second SSB in Slot #0 for Test 1-2 | First SSB in Slot #0 for Test 1-1 N/A for Test 1-2 |
|  | SSB periodicity | ms | 20 | 20 | 20 |
| PDSCH configuration | Mapping type |  | Type A |  |  |
|  | k0 |  | 0 |  |  |
|  | Starting symbol (S) |  | 2 |  |  |
|  | Length (L) |  | 12 |  |  |
|  | PDSCH aggregation factor |  | 1 |  |  |
|  | PRB bundling type |  | Static |  |  |
|  | PRB bundling size |  | 2 |  |  |
|  | Resource allocation type |  | Type 0 |  |  |
|  | RBG size |  | Config2 |  |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |  |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |  |
|  | Number of additional DMRS |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |  |
| Number of HARQ Processes |  |  | 8 |  |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |  |  |

Table 5.2.2.2.16-3: Minimum performance for PDSCH with rank 1 and with inter-cell interference

| Test num | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell1 |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-2.1 TDD | 40 / 30 | 16QAM, 0.48 | TDLC300-100 | 2x2, ULA Low | 70 | [TBD] |
| 1-2 | R.PDSCH.2-2.1 TDD | 40 / 30 | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | [TBD] |

<Unchanged sections skipped>

-----------------Change 2---------------------

5.2.3.2.16	Minimum requirements for PDSCH with inter-cell interference

The performance requirements are specified in Table 5.2.3.2.16-3, with the addition of test parameters in Table 5.2.3.2.16-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.2.16-1.

Table 5.2.3.2.16-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH performance under 4 receive antenna conditions, when the PDSCH transmission from the serving cell is interfered by 1 or 2 interfering cells. | 1-1, 1-2 |

Table 5.2.3.2.16-2: Test parameters

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 |
|  |  |  | Enabled | Enabled | Enabled for test 1-1 Disabled for test 1-2 |
| Duplex mode |  |  | TDD |  |  |
| TDD UL-DL pattern |  |  | FR1.30-1 |  |  |
| Active DL BWP index |  |  | 1 |  |  |
| Physical cell ID |  |  | 0 | 1 | 2 |
| Transmission rank |  |  | 1 | Random rank with 70% and 30% probability for rank 1 and rank 2 | Random rank with 70% and 30% probability for rank 1 and rank 2 for Test 1-1 N/A for Test 1-2 |
| Time offset between Cells |  | us | 0 | 1.5 | -0.5 |
| Frequency shift between Cells |  | Hz | 0 | 300 | -100 |
| Interference Model |  |  | N/A | As specified in B.6.2 |  |
| INR |  | dB | N/A | 7.77 for Test 1-1 7.58 for Test 1-2 | 2.29 for Test 1-1 N/A for Test 1-2 |
| SSB configuration | SSB position in burst |  | First SSB in Slot #0 | First SSB in Slot #0 for Test 1-1 Second SSB in Slot #0 for Test 1-2 | First SSB in Slot #0 for Test 1-1 N/A for Test 1-2 |
|  | SSB periodicity | ms | 20 | 20 | 20 |
| PDSCH configuration | Mapping type |  | Type A |  |  |
|  | k0 |  | 0 |  |  |
|  | Starting symbol (S) |  | 2 |  |  |
|  | Length (L) |  | 12 |  |  |
|  | PDSCH aggregation factor |  | 1 |  |  |
|  | PRB bundling type |  | Static |  |  |
|  | PRB bundling size |  | 2 |  |  |
|  | Resource allocation type |  | Type 0 |  |  |
|  | RBG size |  | Config2 |  |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |  |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |  |
|  | Number of additional DMRS |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |  |
| Number of HARQ Processes |  |  | 8 |  |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |  |  |

Table 5.2.3.2.16-3: Minimum performance for PDSCH with rank 1 and with inter-cell interference

| Test num | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell1 |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-2.1 TDD | 40 / 30 | 16QAM, 0.48 | TDLC300-100 | 2x4, ULA Low | 70 | [TBD] |
| 1-2 | R.PDSCH.2-2.1 TDD | 40 / 30 | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | [TBD] |

<< End of Changes >>