3GPP TSG-RAN WG4 Meeting # 104-e      	R4-2212300

Electronic Meeting, Aug 15 – Aug 26, 2022

Source:	CMCC

Title:	Discussion on test cases for RLM/BFD measurement relaxation

Agenda Item:	9.12.2

Document for:	Discussion

- Introduction

In RAN4#103-e meeting, the performance part discussion for RLM/BFD relaxation has been triggered, and the test case list has been approved, a way forward was agreed in [1]. In this contribution, we further discuss the test cases design according to the conclusion of RLM/BFD relaxation methodology and core part requirements.

- Discussion

Issue 2-2-1: Test case for exiting criteria

- Proposal 1 (revised): Define at least some of OOS TCs (for example RLM) to validate correct UE behavior when the UE needs to exit from the relaxation mode.

- FFS configure N310 = 2 to validate

We support to verify correct UE behavior when the UE needs to exit from the relaxation mode.

However, we don’t think setting N310=2 can validate UE exiting behavior. Because the OOS indication period is Max(10ms, 1.5 × DRX_cycle_length, 1.5 × TRLM-RS,M)) regardless of UE is in relaxation mode or not. Even through UE doesn’t exit relaxation mode, the OOS will also be reported in time.

Observation 1: Setting N310=2 can’t validate the UE behavior of exiting relaxation mode

Therefore, we don’t see strong motivation of configuring N310 to 2 at the current stage. We prefer to reuse N310=1 as legacy test case configuration.

Proposal 1: Configure N310 to 1.

Issue 2-2-4: DRX period setting

- DRX periods of 40ms and 80ms are distributed in respective test cases, e,g. different values for FR1/FR2, EN-DC/SA, SSB/CSI-RS.

- The following proposals for the values of DRX cycle length will be discussed in R4#104-e:

  - Proposal 1: 40ms for TC1, 80ms for TC4, 40ms for TC5, 40ms for TC13, 40ms for TC16

  - Other proposals are not precluded.

According to the above agreements, we can see that the relaxation factor K is different for different DRX period in FR1. In order to verify correct UE relaxation behavior, we would like to set different DRX cycles for FR1 tests, which are TC1 and TC4. We support the Proposal 1, 40ms for TC1 and 80ms for TC4.

To be mentioned, if we introduce DRX cycle=80ms test case, we should also introduce a new DRX configuration in TS38.133 Section A.3.3.X, since the current spec doesn’t involve DRX cycle=80ms configuration.

Proposal 2: 40ms for TC1, 80ms for TC4, 40ms for TC5, 40ms for TC13, 40ms for TC16

Proposal 3: If the DRX cycle=80ms test case is introduced, a new DRX configuration should also be added in TS38.133 Section A.3.3.X.

Issue 2-3-1: Test set up on relaxation criterion

- Option 1: only good serving cell quality criterion is configured

- Option 2: both low mobility criterion and good serving cell quality criterion are configured

- Option 4 (new): Test cases need to be for both combinations.

We support Option 2. We think it is important to verify that whether UE can evaluate the low mobility criterion and good serving cell quality criterion in every test case. Besides, in the test, the movement state of UE can’t be evaluated by network. In this case, both low mobility criterion and good serving cell quality criterion should be configured and evaluated.

Proposal 4: In each test case, both low mobility criterion and good serving cell quality criterion should be configured and evaluated.

Issue 2-4-4: RLM OOS test – SNR setting

- Reuse the SNR variation setting configurations in legacy RLM out-of-sync tests for RLM measurement relaxation test cases.

- For SNR values,

| Test |  | SNR value of RLM-RS#1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | SNR1 | SNR2 | SNR3 |
| Relax RLM testing | FR1 | 1dB | Y11 dB | -15dB |
|  | FR2 | 2dB | Y12 dB | -15dB |

  - where Y11/Y12 dB (SNR2) are FFS

    - Option 1: -4.5 dB

    - Option 2: set SNR2 as SNR1

    - Option 3: reuse the legacy values. (i.e., -7dB in FR1, -6dB in FR2)

  - whether to configure RLM-RS#2 is FFS

Issue 2-4-6: BFD test – SNR setting

- For SNR values,

| Test |  | SNR value of BFD-RS#1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | SNR1 | SNR2 | SNR3 |
| Relax BFD testing | FR1 | 5dB | Y21 dB | -12dB |
|  | FR2 | 6dB | Y22 dB | -12dB |

- where Y21/Y22 dB (SNR2) are FFS

  - Option 1: -0.5 dB

  - Option 2: set SNR2 as SNR1

  - Option 3: reuse the legacy values. (i.e., -3dB in FR1, -2dB in FR2)

- whether to configure BFD-RS#2 is FFS

In the last meeting, RAN4 agreed that if the good serving cell quality criterion defined in clause 5.7.13.2 of TS 38.331 is not fulfilled for all resources in the set of resources for radio link monitoring, then UE is no longer allowed to relax RLM measurements and apply the relaxed radio link monitoring. Therefore, to verify the correct UE exiting behavior, we support to define test cases under the multiple RLM/BFD-RSs configured scenario.

Agreement in RAN4#103-e meeting: - For entering condition, - Option 1: good serving cell quality criterion is fulfilled when the radio link quality is better than the threshold (Qin + X dB) for at least one resource in the set of resources for RLM/BFD. - For exiting condition, - Option 1: good serving cell quality criterion is not fulfilled when the radio link quality is worse than the threshold (Qin + X dB) for all resource in the set of resources for RLM/BFD.

With the spirit of enlarging the test coverage, and reducing the rest burden, we think the single RLM/BFD-RS scenario and two RLM/BFD-RSs scenario can be distributed in difference TCs. For example, TC1 and TC16 configure two RLM/BFD-RSs, TC4, TC5 and TC13 configure single RLM/BFD-RS.

Proposal 5: Distribute the test cases under the multiple RLM/BFD-RSs configured scenario and the single RLM/BFD-RS configured scenario. For example, TC1 and TC16 configure two RLM/BFD-RSs, TC4, TC5 and TC13 configure single RLM/BFD-RS.

In the case of two RLM/BFD-RSs are configured, for the SNR value of RLM/BFD-RS#1, we prefer to reuse the legacy values. For the SNR value of RLM/BFD-RS#2, we think setting SNR2 as SNR1 is more reasonable. We take RLM as an example, the SNR and Timing settings for RLM tests are shown as in Figure 1.

Figure 1. The SNR and Timing settings for RLM tests when two RLM-RSs configured

According to the exiting criteria agreed in previous meetings, UE is no longer allowed to relax RLM measurements after the time point B.

In the case of single RLM/BFD-RSs are configured, for the SNR value of RLM/BFD-RS#1, we prefer to reuse the legacy values. We take RLM as an example, the SNR and Timing settings for RLM tests are shown as in Figure 2.

Figure 2. The SNR and Timing settings for RLM tests when single RLM-RS configured

Proposal 6:

- In the case of two RSs, we prefer to reuse the legacy SNR values for RS#1, set SNR2 as SNR1 for RS#2.

- In the case of single RS, we prefer to reuse the legacy SNR values for RS#1.

- Conclusion

In this contribution, we give our consideration of how to test RLM/BFD relaxation, the proposals are provided as follows:

Observation 1: Setting N310=2 can’t validate the UE behavior of exiting relaxation mode

Proposal 1: Configure N310 to 1.

Proposal 2: 40ms for TC1, 80ms for TC4, 40ms for TC5, 40ms for TC13, 40ms for TC16

Proposal 3: If the DRX cycle=80ms test case is introduced, a new DRX configuration should also be added in TS38.133 Section A.3.3.X.

Proposal 4: In each test case, both low mobility criterion and good serving cell quality criterion should be configured and evaluated.

Proposal 5: Distribute the test cases under the multiple RLM/BFD-RSs configured scenario and the single RLM/BFD-RS configured scenario. For example, TC1 and TC16 configure two RLM/BFD-RSs, TC4, TC5 and TC13 configure single RLM/BFD-RS.

Proposal 6:

- In the case of two RSs, we prefer to reuse the legacy SNR values for RS#1, set SNR2 as SNR1 for RS#2.

- In the case of single RS, we prefer to reuse the legacy SNR values for RS#1.

- References

R4-2210613, WF on RLM/BFD relaxation for UE Power Saving enhancements.