3GPP TSG-RAN WG4 Meeting # 104-e										R4-2212312

Electronic Meeting, August 15 – August 26, 2022

Agenda item:	11.13.2.2

Source: 	CMCC

Title: 	Self interference and CLI study of SBFD

Document for:	Discussion

# 1. Introduction

RAN1 send the LS to RAN4 to query about self-interference and inter-sub band CLI performance at both gNB side and UE side. In this contribution, we focus on the discussion of how to reply LS to RAN1 with our initial thought.

# 2. Discussion

## 2.1 Self-interference

SI cancelation can be carried out from following three domains, spatial domain isolation, RF domain suppression and digital domain cancellation. Spatial domain isolation includes Tx/Rx spatial isolation by vertical deposition, inserting isolation material between Tx and Rx antenna, near-field antenna radiation and etc. RF domain suppression can be divided into two categories, one by simulating Tx signal by attenuator, phase shifter and the other by using sub-band filter at Tx and Rx side.

About self-interference, RAN1 totally raises 5 questions and following show our initial thought for each question.

### 2.1.1 Question 1-1

- Question 1-1: What is the value range of RSI αSI(n,m) for each frequency range, and under what assumptions on the self-interference suppression means the value range of RSI is provided?

  - RAN1 understands the RSI can be described per subband, per RB, or per subcarrier depending on the granularity of the frequency unit, and it is up to RAN4 to provide the RSI in which granularity.

One assumption of residual self-interference is that it leads to degraded REFSENSE by 1dB, so residual self-interference equals to N-6dB. For all three BS classes, the required residual self-interference are calculated as below and the design of SBFD should target at following limit.

I=-114dBm/MHz+5dB(NF)-6dB=-115dBm/MHz for WA

I=-114dBm/MHz+10dB(NF)-6dB=-110dBm/MHz for MR

I=-114dBm/MHz+13dB(NF)-6dB=-107dBm/MHz for LA

- For WA with 46dBm output power per 20MHz, the self-interference cancelation ratio should be 33dBm/MHz-(-115)=148dB

- For MR with 38dBm output power per 100MHz, the self-interference cancelation ratio should be 18dBm/MHz-(-110)=128dB

- For LA with 24dBm output power per 100MHz, the self-interference cancelation ratio should be 4dBm/MHz-(-107)=111dB

So for SBFD design, the target is as calculated above.

Observation 1: the target SBFD self-interference cancelation ratio should be 148dB for WA, 128dB for MR and 111dB for LA.

As for the frequency granularity of RSI, per PRB basis is more preferred because RAN1’s system level simulation is PRB basis. Besides, PRB basis could show more details and help for RAN1’s scheme design. If we reply RSI per sub-band basis, we also need to explain how to translate such limit into per PRB basis.

Observation 2: per PRB basis RSI is more preferred to facilitate RAN1’ simulation and scheme design.

### 2.1.2 Question 1-2

  - Question 1-2: Whether it is possible for RAN4 to provide RAN1 the respective capabilities of different self-interference suppression means? e.g., is it possible to provide the separate estimates for spatial isolation, subband frequency isolation, beamform nulling/isolation, and digital cancellation, etc., as below?

    - dB(αSI(m,n))= dB(αSI-spatial)+dB(αSI-frequency (m,n))+dB(αSI-beam)+dB(αSI-digtial) +…

      - αSI-spatial denotes the spatial isolation.

      - αSI-frequency (m,n) denotes the suband frequency isolation between the Tx frequency unit m and the Rx frequency unit n.

      - αSI-beam denotes the beamform nulling or beam isolation.

      - αSI-digtial denotes the digital cancellation capability.

Different SI suppression means are related to each other. For example, if antenna domain and RF domain could achieve much better SI suppression, the achievable SI suppression of digital domain maybe reduced. So it’s hard to provide separate estimates. From RAN4’s perspective, it’s better to reply one SI value range to RAN1 to reflect overall SI suppression including spatial, RF and digital domain. We could also add some examples about separate estimates to help RAN1 understand different achievable capability of different means.

Observation 3: it seems more reasonable to show one SI value range to reflect overall SI suppression capability. If necessary, we could add some examples of separate estimates for different means with the overall value range assumption.

### 2.1.3 Question 1-3

  - Question 1-3: Whether it is possible to simplify the RSI as frequency flat model, and under which condition(s) the dependency of the RSI on frequency can be ignored?

Non-linearity characteristics of Tx RF component is the main reason for frequency non-flat model. Better PA performance is required to reduce non-linearity impact which in general could be regarded as frequency flat at adjacent channel at least for FR1. But if sub-band filter is utilized for SI cancellation, it requires certain transition guard band and may not be regarded as frequency flat unless we define a relatively larger transition guard band without Tx and Rx..

Observation 4: for gNB without sub-band filter for SI cancellation, RSI could be simplified as frequency flat at least for FR1. But for gNB with sub-band filter, the attenuation mask is not flat unless defining larger transition guard band.

### 2.1.4 Question 1-4

  - Question 1-4: The feasibility of provided value range of RSI regarding factors such as blocking, AGC, etc.

To reflect feasibility of blocking, we need to make sure residual self-interference doesn’t block Rx chain at Rx LNA input port. Therefore, besides the overall SI suppression as discussed in question 1-1, we also need to consider SI suppression at LNA input port. And overall SI suppression is based on this SI suppression for blocking.

Proposal 1: we need two kinds of RSI value range, one as overall RSI considering Tx/Rx spatial isolation, RF attenuation and digital cancellation, the other as RSI at LNA input port to avoid blocking.

### 2.1.5 Question 1-5

  - Question 1-5: Does RSI have any dependency with the following factors or any other factors? What are the dependencies?

    - gNB’s antenna aspects, e.g., the assumed antenna architecture, the number of transmit chains and receive chains, etc.

    - Frequency aspects, e.g., the frequency distance between the Tx frequency unit m and the Rx frequency unit n, the number of RBs allocated for DL transmission, etc.

    - Beam aspects, e.g., Tx/Rx beam-pair for FR1/FR2 especially for clutter echo, etc.

SI cancellation implementation is diverse and it is very hard to reflect all aspects to RAN1. One method is to categorize RSI by different gNB classes. Each gNB class represent one kind of typical RF and antenna architecture. RAN4 could reply RSI for each gNB class to RAN1 with one typical assumption to describe the dependencies on antenna aspects, frequency aspects and RF aspects.

Proposal 2: RAN4 is suggested to reply RSI for each gNB class to RAN1 with one typical assumption to describe the dependencies on antenna aspects, frequency aspects and RF aspects.

## 2.2 Inter-sub band interference suppression

In theory, inter-sub band interference suppression method maybe the same as SI suppression, including spatial domain isolation, RF attenuation and digital domain cancellation.

About inter-sub band interference, RAN1 totally raises 3 questions and following show our initial thought for each question.

### 2.2.1 Question 2-1

- Question 2-1: Whether it is feasible to consider above two aspects for gNB-gNB and UE-UE co-channel inter-sub band CLI modelling in system level simulation? Are there any other aspects should also be taken into account?

In current RAN4 spec, both Leakage at Tx side and selectivity at Rx side are considered, e.g. the definition of ACLR and ACS. So it’s feasible to consider two aspects as described in RAN1’s LS. Besides, for co-site scenario, blocking issues should also be considered. Almost all Tx power could fall into Rx and may block gNB’s Rx at LNA input port. Therefore, we should make sure the design of inter-sub band interference cancellation at Rx side could avoid Rx non-linearity.

Observation 5: it’s feasible to consider Tx leakage and Rx selectivity as described in RAN1’s LS. Besides, blocking issue should also be carefully studied for co-site scenario.

### 2.2.2 Question 2-2

- Question 2-2: For a specific pair of DL frequency unit m (e.g., subband/RB m) and UL frequency unit n (e.g., subband/RB n) of gNB-gNB link, where the DL frequency unit m and UL frequency unit n are in the same carrier and non-overlapping in frequency, and assuming the aggressor gNB transmits on the DL frequency unit m and the victim gNB receives on the UL frequency unit n,

  - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 1 (defined above) at the gNB transmitter?

  - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 2 (defined above) at the gNB receiver?

  - How to model the above interferences for the following two cases:

    - inter-site gNB-gNB co-channel inter-subband CLI

    - co-site inter-sector co-channel inter-subband CLI

There is no Tx related inter-sub band requirement in current gNB spec, so new RF requirements should be considered. ACLR like method is preferred. DPD algorithm plays a great role in legacy ACLR performance and the same algorithm also applies for inter-sub band cases. From this point of view, the same value as ACLR, i.e. 45dB may also apply for inter-sub band emission requirements.

Proposal 1: it’s suggested to further check whether legacy ACLR limit of gNB is also applicable for inter-sub band case or not.

At gNB side, legacy ICS is inter-sub band like requirements and is used to specify selectivity when gNB receive multiple UE signals with different PRB configurations in the same carrier. Legacy ACS is used to specify selectivity between adjacent carriers. From our understanding, it’s digital filter characteristics that mainly contributes to ACS requirements. If full duplex gNB configure digital filter with the sub band bandwidth, ACS requirement could also be applicable for inter-sub band case. Compared with ICS, ACS could achieve better selectivity. So maybe ACS could be taken as the baseline for Rx inter-sub band selectivity instead of ICS.

Observation 6: instead of ICS, ACS at gNB side may be taken as the baseline for inter-sub band Rx selectivity requirements.

About the two scenarios in RAN1’s LS, inter-site and co-site inter-sector scenarios, the main differences are listed as below

- Antenna isolation is different for such two scenarios. Antenna isolation of co-site inter-sector is near-field antenna characteristics but for inter-site, antenna isolation is far-field antenna characteristics.

- Digital interference cancellation is different for such two scenarios. For co-site scenario, different sector may share the same based band processor, if so, digital interference cancellation could be utilized for inter-sub band interference cancellation. But for inter-site scenario, there is no possibility for such digital interference cancellation.

Observation 7: for inter sub-band CLI, the main differences between co-site inter sector and inter site scenarios include following two aspects:

- Antenna isolation of co-site inter-sector is near-field antenna characteristics but for inter-site, antenna isolation is far-field antenna characteristics.

- For co-site scenario, digital interference cancellation could be utilized. But for inter-site scenario, there is no possibility for such digital interference cancellation.

### 2.2.3 Question 2-3

- Question 2-3: For a specific pair of DL frequency unit m (e.g., subband/RB m) and UL frequency unit n (e.g., subband/RB n) of UE-UE link, where the DL frequency unit m and UL frequency unit n are in the same carrier and non-overlapping in frequency, and assuming the aggressor UE transmits on the UL frequency unit n and the victim UE receives on the DL frequency unit m,

  - How to model the interference from UL frequency unit n to DL frequency unit m due to Aspect 1 (defined above) at the UE transmitter?

  - How to model the interference from UL frequency unit n to DL frequency unit m due to Aspect 2 at the UE receiver?

In the WID, it’s emphasized that R17 only considers sub band full duplex at gNB side. It’s better to make sure legacy UE could also work in SBFD network.

Legacy IBE is inter-sub band like requirements at UE side. Since R17 only consider gNB side SBFD, UE side will not be optimized to fit for the operation in SBFD network. It’s better to reuse legacy IBE requirements for evaluation.

Proposal 2: It’s better to reuse legacy IBE requirements for evaluation.

RAN4 doesn’t specify legacy Rx requirements for inter-sub band case at UE side. If assume 0dB Rx selectivity, we are afraid UE working in SBFD network may meet inter-sub band interference issue. It’s better to study how to avoid such interference at UE side. Following lists two options to avoid UE-UE inter-sub band interference.

- Option 1: RAN4 collect the minimum requirements from UE vendors that could reflect all legacy UE’s Rx selectivity for inter-sub band case and reply it to RAN1 for their simulation and scheme design.

- Option 2: RAN4 send the LS to RAN1 and ask RAN1 design schemes to avoid UE-UE inter-sub band interference with 0dB Rx inter-sub band selectivity requirements

Observation 8: above two options are listed to avoid UE-UE inter-sub band interference based on the assumption that legacy UE could also work in SBFD network.

# 3. Conclusions

In this contribution, initial thoughts about all 8 questions from RAN1 are discussed with following observations and proposals:

Observation 1: Target SI cancelation ratio should be 148dB for WA, 128dB for MR and 111dB for LA.

Observation 2: per PRB basis RSI is more preferred to facilitate RAN1’ simulation and scheme design.

Observation 3: it seems more reasonable to show one SI value range to reflect overall SI suppression capability. If necessary, we could add some examples of separate estimates for different means under the overall value range assumption.

Observation 4: for gNB without sub-band filter for SI cancellation, RSI could be simplified as frequency flat at least for FR1. But for gNB with sub-band filter, the attenuation mask is not flat unless defining larger transition guard band.

Observation 5: it’s feasible to consider Tx leakage and Rx selectivity as described in RAN1’s LS for inter-sub band CLI. Besides, blocking issue should also be carefully studied for co-site scenario.

Proposal 1: it’s suggested to further check whether legacy ACLR limit of gNB is also applicable for inter-sub band case or not.

Observation 7: for inter sub-band CLI, the main differences between co-site inter sector and inter site scenarios include following two aspects:

- Antenna isolation of co-site inter-sector is near-field antenna characteristics but for inter-site, antenna isolation is far-field antenna characteristics.

- For co-site scenario, digital interference cancellation could be utilized. But for inter-site scenario, there is no possibility for such digital interference cancellation.

Proposal 2: It’s better to reuse legacy IBE requirements for evaluation.

Observation 8: two options are listed to avoid UE-UE inter-sub band interference based on the assumption that legacy UE could also work in SBFD network.

- Option 1: RAN4 collect minimum requirements from UE vendors that could reflect all legacy UE’s Rx selectivity for inter-sub band case and reply it to RAN1 for their simulation and scheme design.

- Option 2: RAN4 send the LS to RAN1 and ask RAN1 design schemes to avoid UE-UE inter-sub band interference with 0dB Rx inter-sub band selectivity requirements

# 4. Reference

[1] R1-2205543, LS on interference modelling for duplex evolution, CMCC