3GPP TSG-RAN WG4 Meeting # 104-e										R4-2212313

Electronic Meeting, August 15 – August 26, 2022

Agenda item:	11.13.2.1

Source: 	CMCC

Title: 	adjacent channel interference analysis for SBFD

Document for:	Discussion

# 1. Introduction

RAN1 send the LS to RAN4 to query about adjacent channel Tx leakage and Rx selectivity at both gNB side and UE side [1].

In this contribution, we focus on the discussion of simulation for adjacent channel scenario.

# 2. Discussion

## 2.1 Summary of CLI study in R16

CLI is studied in R16 and following two tables show simulation results as captured in TR 38.828.

Table 1: summary of simulation results for R16 CLI BS-BS

| BS<->BS | Macro->macro | Indoor->macro | Indoor->indoor | macro->Indoor (FR1) Micro->micro (FR2) |
| --- | --- | --- | --- | --- |
| FR1 | Performance degradation | No degradation | Provided BS and UE have similar power, no performance degradation Performance degradation if BS has higher power | No degradation |
| FR2 | Performance degradation | No degradation | The performance degradation has some dependency on how power control is operated | Performance degradation is observed by some companies but other companies observe no performance degradation. The distance between micros should be taken care. |
| Note: co-site scenario is not considered |  |  |  |  |

Observation 1: legacy ACLR/ACS limits will lead to performance degradation for macro gNB- macro gNB scenario for both FR1 and FR2. But as long as care is taken with some solutions, in other scenarios, legacy ACLR and ACS limits may still apply.

Table 2: Summary of simulation results for R16 CLI UE-UE

| UE<->UE | Macro->macro | Indoor->macro | Indoor->indoor | macro->Indoor (FR1) Micro->micro (FR2) |
| --- | --- | --- | --- | --- |
| FR1 | no | no | no | Performance degradation is observed by some companies. But the conclusion is UE-UE interference could be ignored as long as care is taken |
| FR2 | no | No | no | No |
| Note: co-site scenario is not considered |  |  |  |  |

Observation 2: legacy ACLR/ACS limits still apply for UE-UE CLI scenario for FR2. For FR1, legacy ACLR/ACS limits still applies for most scenarios except for macro->indoor, for which scenario, as long as care is taken with some solutions, legacy ACLR and ACS limits also apply.

## 2.2 Co-existence study between legacy TDD and sub-band full duplex (SBFD)

As discussed in RAN#96 plenary meeting, legacy UL symbol for SBFD operation as 2nd priority is accepted, no intended suspension of continuation of work in WGs. we both consider UL symbols and DL symbols as SBFD operation cases in following.

### 2.2.1 Deployment scenario

For FR1, 4GHz is suggested as example frequency, the same as R16 CLI study and also aligned with RAN1 as typical 5G spectrum, e.g. n77, n79. For FR2, 30GHz is suggested as example frequency, the same as R16 CLI study and also aligned with RAN1.

Proposal 1: 4GHz and 30GHz are suggested as example frequency for FR1 and FR2 respectively.

For FR1, urban macro and indoor office are suggested as the baseline deployment scenario, the same as legacy R16 CLI. For dense urban scenario with 1 layer(micro) or 2-layer(macro+micro) scenarios, they can be regarded as optional. If companies contribute to such cases, we should also take final simulation results into consideration. Besides, to further reduce workload, the scenario that is verified legacy limits applicable could not be reevaluated.

Proposal 2: Deployment scenarios for adjacent channel evaluation are as below for FR1.

- urban macro and indoor as the baseline

  - We can only focus on the scenario that legacy limits not applicable as verified in R16 CLI to reduce workload.

- dense urban scenario with 1 layer (micro) or 2-layer (macro + micro) scenarios are optional, if companies contribute to them, final simulation results should also be considered.

For FR2, the same deployment as in R16 CLI could be the baseline. So dense urban macro, dense urban micro and indoor scenarios all should be considered. To reduce workload, we can only focus on the scenario that legacy limits not applicable as verified in R16 CLI to reduce workload.

Proposal 3: Deployment scenario for adjacent channel evaluation are as below for FR2

- urban macro, indoor and urban micro all as the baseline

  - We can only focus on the scenario that legacy limits not applicable as verified in R16 CLI to reduce workload.

### 2.2.2 ACLR model

In R16 CLI, both aggressor and victims are assumed to utilize the whole carrier. ACLR/ACS values could be directly used without any scaling.

For legacy TDD network interfering SBFD network scenario, aggressor legacy gNB would use all RB configurations per carrier for DL Tx but only part of adjacent SBFD carrier will be used for UL Rx. So total received adjacent channel interference at SBFD gNB side is not equal to Pout – ACLR and we need to know equivalent emission mask as ACLR to help calculate actual received interference at adjacent SBFD carrier. As illustrated in following fig, we should at first model the ACLR equivalent emission mask for legacy TDD carrier based on ACLR value.

Proposal 4: we should at first model ACLR equivalent emission mask for legacy TDD carrier to help calculate received sub-band interference at adjacent SBFD carrier.

One option is to assume emission mask as frequency flat. Another option is to use OBUE mask but total emission power among adjacent channel should be scaled to ACLR value rather than directly reusing OBUE values.

Observation 3: some options to model ACLR equivalent emission mask among adjacent channel for legacy TDD.

- Option 1: frequency flat among adjacent channel

- Option 2: reuse OBUE spectrum model but scaling the limit so that total emission power among adjacent channel equal to ACLR value.

For SBFD network interfering legacy TDD network scenario, aggressor SBFD gNB would only use DL inter-sub band for Tx but total adjacent carrier will be used for UL Rx by legacy network. ACLR2, ACLR3 and other order ACLR may also need to be considered as illustrated in following fig.

Proposal 5: we should define ACLR1, ACLR2 and even higher order ACLR model for SBFD network to help evaluate interference.

### 2.2.3 simulation assumption

The simulation assumptions are listed as below for further discussion.

Table 3: simulation assumption for FR1

|  |  | FR1 |  |
| --- | --- | --- | --- |
|  | Parameters | Indoor office | Urban macro |
| SBFD configuration | Subband configuration for system bandwidth | For 100MHz system bandwidth with 30kHz SCS, (D,U,D)=(40M,20M,40M) or (D,U)=(80M,20M) |  |
| Deployment Scenarios | Layout | Indoor floor: (12BSs per 120m x 50m) [TR 38.828] 6 aggressor and 6 victim | Hexagonal grid, 19 macro sites, 3 sectors per site [TR 38.828] |
|  | Inter-BS (2D) distance | 20m [TR 38.828] | 500m [TR 38.828] |
|  | Minimum BS-UE (2D) distance | 0m [TR 38.828] | 35m [TR 38.828] |
|  | Minimum UE-UE (2D) distance | 1m~3m [TR 38.828 ] RAN1 assume 3m | 3m |
|  | cell selection criteria | based on RSRP |  |
|  | UE distribution | 100% Indoor | 20% Outdoor in cars 80% Indoor in houses FFS: whether to consider 50%, 50% cases as in TR 38.828 |
|  |  | For legacy network: 1UE per sector; For SBFD network: the same number as sub band. | For legacy network: 1UE per sector; For SBFD network: the same number as sub band. |
|  | channel bandwidth | 100MHz |  |
|  | scheduled channel bandwidth per UE | legacy DL: 100MHz legacy UL: 100MHz SBFD DL: the same bandwidth as DL sub-band bandwidth SBFD UL: the same bandwidth as UL sub-band bandwidth |  |
|  | pathloss model | Indoor (Aggressor) → Macro (Victim): - BS-to-BS: indoor + penetration loss see TR 38.828 - BS-to-UE: indoor + penetration loss see TR 38.828 - UE-to-UE: Outdoor UE – Outdoor UE + penetration loss see TR 38.828 Indoor (Aggressor) → Indoor (Victim) - BS-to-BS: indoor see TR 38. 828 - BS-to-UE: indoor see TR 38. 828 - UE-to-UE: indoor see TR 38. 828 | Macro(Aggressor) → Macro(Victim) - Macro-to-UE: UMa see TR 38.828 - Macro-to-Macro: UMa (h_UE = 25 m) see TR 38.828 - UE-to-UE: Outdoor UE – Outdoor UE see TR 38.828 + penetration loss see TR 38.828 |
|  | LOS model | TR 38.828 | TR 38.828 |
|  | Handover margin (dB) | 3 dB [TR 38.828 Table 5.2.1.4-1] |  |
|  | UE attachment | Based on RSRP [TR 38.828] |  |
|  | Evaluation metric | SINR/throughput |  |
|  | Wrapping around method | No wrapping around | Geographical distance based wrapping |
| BS | BS Tx power | 24dBm | 49dBm |
|  | BS antenna configurations | (Mg,Ng,M,N,P)= (1,1,2,1,2) (dH,dV)=(0.5,0.5)λ, +45°, -45° polarization | (Mg,Ng,M,N,P)=(1,1,8,8,2) (dH,dV)=(0.5,0.8)λ, +45°, -45° polarization [TR 38.828] Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: TX power is specified per polarization, a single polarization may be simulated under the assumption of polarization match. |
|  | BS antenna pattern | as defined in 38.901 | as defined in 38.901 |
|  | BS antenna element gain | 3.5 dBi (assuming 1.8dB loss) [TR 38.828] | 5 dBi (assuming antenna 1.8dB loss) [TR 38.828] |
|  | BS antenna height | 3 m [TR 38.828] | 25 m [TR 38.828] |
|  | Noise figure | 13 | 5 |
| UE | UE max Tx power | 23dBm | 23dBm |
|  | UE min Tx power | -33dBm/100MHz | -33dBm/100MHz |
|  | UE noise figure | 9 dB [TR 38.828] | 9 dB [TR 38.828] |
|  | UE antenna height | 1.5m [TR 38.828] | Outdoor UEs: 1.5 m; Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) [TR 36.873 Table 6-1] Outdoor UEs: 1.5 m; Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(1,Nfl) where Nfl=1 [TR 38.828] |
|  | UE antenna gain | 0 dBi-Omni |  |
|  | Power control | as defined in Section 5.2.3.4 TR 38.828 |  |

Observation 4: simulation assumption for FR1 is listed in table 3.

# 3. Conclusions

In this contribution, simulation for adjacent channel scenario are discussed with following observations and proposals:

Observation 1: legacy ACLR/ACS limits will lead to performance degradation for macro gNB- macro gNB scenario for both FR1 and FR2. But as long as care is taken with some solutions, in other scenarios, legacy ACLR and ACS limits may still apply.

Observation 2: legacy ACLR/ACS limits still apply for UE-UE CLI scenario for FR2. For FR1, legacy ACLR/ACS limits still applies for most scenarios except for macro->indoor, for which scenario, as long as care is taken with some solutions, legacy ACLR and ACS limits may also apply.

Proposal 1: 4GHz and 30GHz are suggested as example frequency for FR1 and FR2 respectively.

Proposal 2: Deployment scenarios for adjacent channel evaluation are as below for FR1.

- urban macro and indoor as the baseline

  - We can only focus on the scenario that legacy limits not applicable as verified in R16 CLI to reduce workload.

- dense urban scenario with 1 layer (micro) or 2-layer (macro + micro) scenarios are optional, if companies contribute to them, final simulation results should also be considered.

Proposal 3: Deployment scenario for adjacent channel evaluation are as below for FR2

- urban macro, indoor and urban micro all as the baseline

  - We can only focus on the scenario that legacy limits not applicable as verified in R16 CLI to reduce workload.

Proposal 4: we should at first model ACLR equivalent emission mask for legacy TDD carrier to help calculate received sub-band interference at adjacent SBFD carrier.

Observation 3: some options to model ACLR equivalent emission mask among adjacent channel for legacy TDD.

- Option 1: frequency flat among adjacent channel

- Option 2: reuse OBUE spectrum model but scaling the limit so that total emission power among adjacent channel equal to ACLR value.

Proposal 5: we should define ACLR1, ACLR2 and even higher order ACLR model for SBFD network to help evaluate interference.

Observation 4: simulation assumption for FR1 is listed in table 3.

# 4. Reference

[1] R1-2205543, LS on interference modelling for duplex evolution, CMCC