3GPP TSG-RAN WG4 Meeting # 104-e										R4-2212315

Electronic Meeting, August 15 – August 26, 2022

Agenda item:	11.12.2

Source: 	CMCC

Title: 	ATG simulation assumption

Document for:	Discussion

# 1. Introduction

A new WID on Air-to-ground network for NR is approved in RAN #95 e-meeting [1]. One objective is to perform FR1 co-existence evaluation between ATG and IMT terrestrial network.

In this contribution, we focus on the discussion of simulation assumption for ATG.

# 2. Discussion

## 2.1 interference scenario

Compared with TN network, ATG has larger cell coverage, leading to larger D-U gap. To enhance SU with less gap, larger TDD configuration periodicity maybe designed. From this point of view, ATG network and TN network may not be synchronized and CLI interference occurs for TDD band.

Observation 1: ATG and TN network may not be synchronized for TDD operation due to ATG larger D-U gap.

But in RAN4, normally ACLR and ACS are derived assuming synchronization operation between TDD network. so one choice is to derive ACLR and ACS based on the synchronization operation assumption and further check whether final ACLR and ACS applicable for non-synchronization scenario.

Proposal 1: at first phase, focus on the scenarios that ATG and TN systems are synchronization operation.

Following table shows the co-existence interference scenario between ATG and TN network when operating in TDD band.

Table 1: co-existence interference scenario between ATG and TN network

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with ATG | ATG DL | TN DL | Applicable for ATG and TN network operating in band n1, n78 and n79. |  |
| 2 | TN with ATG | ATG UL | TN UL | Applicable for ATG and TN network operating in band n1, n78 and n79. |  |
| 3 | TN with ATG | TN DL | ATG DL | Applicable for ATG and TN network operating in band n1, n78 and n79. |  |
| 4 | TN with ATG | TN UL | ATG UL | Applicable for ATG and TN network operating in band n1, n78 and n79. |  |
| 5 | TN with ATG | ATG UL | TN DL | Applicable for band n1 when co-exist with band n39 | Further check ACLR and ACS derived from case 1- 4 could still be applicable or not. |
| 6 | TN with ATG | TN DL | ATG UL | Applicable for band n39 when co-exist with band n1 | Further check ACLR and ACS derived from case 1- 4 could still be applicable or not. |
| 7 | TN with ATG | TN UL | ATG DL | Applicable for ATG and TN network operating in band n78 and n79 for non-synchronization operation | Further check ACLR and ACS derived from case 1- 4 could still be applicable or not. |
| 8 | TN with ATG | ATG DL | TN UL | Applicable for ATG and TN network operating in band n78 and n79 for non-synchronization operation | Further check ACLR and ACS derived from case 1- 4 could still be applicable or not. |

Proposal 2: co-existence interference scenarios are suggested as in table 1.

## 2.2 TN gNB parameters

TN parameters for co-existence study are given in following table.

Table 2. TN gNB simulation assumption

|  | NR |  |  |
| --- | --- | --- | --- |
| Carrier frequency in GHz | 2 | 3.5 | 4.9 |
| Size of each nominal channel BW in MHz | 20 | 100 | 100 |
| Environment | Deployment scenario related, see Table 3. | Deployment scenario related, see Table 3. | Deployment scenario related, see Table 3. |
| Network layout | 19-sites 57 sectors with wrap-around | 19-sites 57 sectors with wrap-around | 19-sites 57 sectors with wrap-around |
| Inter-site distance in meter | Deployment scenario related, see Table 3 | Deployment scenario related, see Table 3 | Deployment scenario related, see Table 3 |
| System loading and activity | Consider the active rate of 20% for Rural | Consider the active rate of 20% for Rural | Consider the active rate of 20% for Rural |
| Network location | Only one ATG BS is deployed in 19-sites TN network as in fig 1 Co-located scenario is not considered. | Only one ATG BS is deployed in 19-sites TN network as in fig 1 Co-located scenario is not considered. | Only one ATG BS is deployed in 19-sites TN network as in fig 1 Co-located scenario is not considered. |
| DL subcarrier spacing | 15kHz | 30kHz | 30kHz |
| UL | OFDMA | OFDMA | OFDMA |
| DL power control | No | No | No |
| UL power control | TR 36.942 | TR 36.942 | TR 36.942 |
| Frequency reuse | 1 | 1 | 1 |
| Number of scheduled UE per cell (DL) | 1 | 1 | 1 |
| Number of scheduled UE per cell (UL) | 3 | 3 | 3 |
| UE antenna height in meter | 1.5m | 1.5m | 1.5m |
| UE TX power in dBm | -40 to 23 | -40 to 23 | -40 to 23 |
| UE antenna gain in dBi | 0 | 0 | 0 |
| Building penetration loss | In pathloss model, TR 38.901 | In pathloss model, TR 38.901 | In pathloss model, TR 38.901 |
| Cell selection margin in dB | 3 | 3 | 3 |
| BS-MS min distance in meters | 35 | 35 | 35 |
| BS noise figure in dB | 5 | 5 | 5 |
| UE noise figure in dB | 9 | 9 | 9 |
| BS-UE path-loss model | TR 38.901 | TR 38.901 | TR 38.901 |
| Standard deviation of BS-UE log-normal shadow fading in dB | Deployment scenario related, referring to TR 38.901 | Deployment scenario related, referring to TR 38.901 | Deployment scenario related, referring to TR 38.901 |
| Shadowing correlation | Inter-cell 0.5 Intra-cell 1 | Inter-cell 0.5 Intra-cell 1 | Inter-cell 0.5 Intra-cell 1 |
| Link-level performance model | Throughput-SINR mapping | Throughput-SINR mapping | Throughput-SINR mapping |
| UE distribution | Uniform | Uniform | Uniform |
| Evaluation metrics | Throughput or SNR loss criteria | Throughput or SNR loss criteria | Throughput or SNR loss criteria |

Table 3: Deployment-related parameters of TN

|  | Rural Macro | Remarks |
| --- | --- | --- |
| ISD in meters | 7500 | ITU-R Report M.2292 |
| BS Antenna height in meters | 30 |  |
| UE Outdoor/indoor | 100% outdoor |  |
| UE height in meter | 1.5 | RP-200559 [12] 3GPP LS to ITU-R WP5D and ITU-R WP5D [IMT_Parameters] |

Table 4: ACLR/ACS for TN

|  |  | NR |
| --- | --- | --- |
| BS | ACLR | 45 dB |
|  | ACS | 46 dB |
| UE | ACLR | 30dB (ACLR1) 43dB (ACLR2) |
|  | ACS | 33 |

Fig 1, ATG network deployment

gNB output power and antenna pattern could refer to the same value as in TR 38.863 for AAS gNB.

Proposal 3: TN gNB simulation assumptions are suggested as in table 2-4. gNB output power and antenna pattern could refer to the same value as in TR 38.863 for AAS gNB.

## 2.3 TN UE parameters

UE parameters for co-existence study are given in following table.

Table 5: TN UE characteristics for system level simulations

| Characteristics | Handheld |
| --- | --- |
| Frequency band | n1, n78, n79 |
| Antenna type and configuration | (1, 1, 2) with omni-directional antenna element |
| Polarisation | Linear: +/-45°X-pol |
| Rx Antenna gain | 0 dBi per element |
| Antenna temperature | 290 K |
| Noise figure | 9 dB |
| Tx transmit power | 200 mW (23 dBm) |
| Tx antenna gain | 0 dBi per element |

Proposal 4: TN UE simulation assumptions are suggested as in table 5.

# 3. Conclusions

In this contribution, ATG simulation assumptions are discussed with following proposals:

Observation 1: ATG and TN network may not be synchronized for TDD operation due to ATG larger D-U gap.

Proposal 1: at first phase, focus on the scenarios that ATG and TN systems are synchronization operation.

Proposal 2: co-existence interference scenarios are suggested as in table 1.

Proposal 3: TN gNB simulation assumptions are suggested as in table 2-4. gNB output power and antenna pattern could refer to the same value as in TR 38.863 for AAS gNB.

Proposal 4: TN UE simulation assumptions are suggested as in table 5.

# 4. Reference

[1] RP-221369, Revised WID on Air-to-ground network for NR, CMCC