3GPP TSG-RAN WG4 #104-e																			R4-2212331

e-Meeting, Aug. 2022

Agenda item:	11.7.3

Source: 	Qualcomm Incorporated

Title: 	On initial access beam correspondence

Document for:	Approval

# Introduction

The approved WID [1] contains the following objectives:

Beam correspondence requirements for RRC_INACTIVE and initial access

- Specify UE beam correspondence requirements for initial access and RRC_INACTIVE state, for SSB-based beam correspondence without UL beam sweeping [RAN4 RF]

- For RRC_INACTIVE specify at least requirements for Random Access SDT and Configured Grant SDT

  - Requirements for other transmission within RRC_INACTIVE state are not precluded.

- For initial access, specify requirements and verification of beam correspondence requirements based on msg1 spherical coverage (at least)

- Study the potential impact on testability aspects (i.e., test time).

We share our views on a potential future RF requirement.

# Discussion

The WID lists 3 conditions, RA-SDT, CG-SDT and RACH, which we discuss below. As noted in the WID, a common thread binding them is dependence exclusively on SSB for beam correspondence.

## Signal types

### RACH

UL during legacy RACH consists of MSG1 followed by MSG3. MSG3 UL benefits from additional instructions from the network like TA, MCS, TPC, etc, in the intervening DL MSG2. MSG3 is much like PUSCH in structure. UL during 2 step RACH only includes MSGA. MSGA consists of a preamble part and a PUSCH part, with the relative power difference included in the system info part of the SSB. The RACH process is well understood as an initial connection process, and is mandatory for a functioning UE, whether or not it supports SSB-only beam correspondence.

### RA-SDT

RA-SDT has close similarity with RACH so their requirement treatment can be aligned.

### CG-SDT

For CG-SDT, the UE knows a-priori a time and frequency resource for an UL transmission. It re-uses previous TA, MCS from previous connection. Some criteria (TA validation, RSRP variation) need to be met for the CG-SDT resource to be valid.  If it fails, the UE goes to RA-SDT or connected mode. If the UE finds it valid to send a CG-SDT, the UL is in MSG3 format.

## BMRS

The WID specifies that the beam management reference signal ne SSB-based beam correspondence without reliance on UL beam sweeping. The corresponding capabilities for connected mode operation are beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16.

There is potential for overlapping requirement for a UE that supports both the capabilities listed. Specifically, we notice that MSG3 is like PUSCH in structure and therefore not necessary to retest for this type of UE.

Observation 1:  For a UE that supports beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16, it is not necessary to test for MSG3.

Observation 2: MSG1 is mandatory for a UE because it is part of the RACH, irrespective of whether it supports beamCorrespondenceSSB-based-r16.

## Requirement applicability

Proposal 1: The Rel-18 beam correspondence requirement applicability can therefore be summarised as:

|  | UE that supports beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16 | Other UEs |
| --- | --- | --- |
| MSG1/MSGA | Needs new requirement, mandatory | Needs new requirement, mandatory |
| MSG3 | No need for dedicated requirement due to overlap with PUSCH requirement | Needs new requirement, mandatory |

## MSG1/MSGA power expectation

The legacy power class definition applies to all signals. i.e the standard has no special treatment for RACH EIRP. Furthermore, the standard requirement for RACH EIRP also includes MPR (equivalent to MPR for equivalent DFT-s-QPSK).

From an implementation perspective, it is understood that the UE can refine its UL beams depending on target UL power based on SSB alone. The Rel-16 SSB based refinement feature established this aspect by setting EIRP expectations for PUSCH without any relaxations on account of the BMRS type (i.e because it is SSB).

The RF requirement is therefore a straightforward matter of simply co-opting the EIRP requirements for DFT-s-QPSK. As related information, in our understanding, the test method would have to command high RACH power through parameters in the system information to verify this RF requirement.

Proposal 2: MSG1 EIRP (peak and spherical) requirements are the same as those for single CC DFT-s-QPSK

# Conclusions

Observation 1:  For a UE that supports beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16, it is not necessary to test for MSG3.

Observation 2: MSG1 is mandatory for a UE because it is part of the RACH, irrespective of whether it supports beamCorrespondenceSSB-based-r16.

Proposal 1: The Rel-18 beam correspondence requirement applicability can therefore be summarised as:

|  | UE that supports beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16 | Other UEs |
| --- | --- | --- |
| MSG1/MSGA | Needs new requirement, mandatory | Needs new requirement, mandatory |
| MSG3 | No need for dedicated requirement due to overlap with PUSCH requirement | Needs new requirement, mandatory |

Proposal 2: MSG1 EIRP (peak and spherical) requirements are the same as those for single CC DFT-s-QPSK

# References

[1]	RP-220967, ‘NR RF requirements enhancement for frequency range 2 (FR2), Phase 3’, Nokia, RAN#95-e, Mar. 2022