3GPP TSG-RAN WG4 Meeting # 104-e 	R4-2212332

# Electronic Meeting, Aug. 2022

Source: Qualcomm Incorporated

Title: 	 System assumptions for multi-Rx chain DL reception

Agenda item:	11.8.2

Document for:	Discussion

# Introduction

The overall objective of the multi-Rx chain DL reception WI is to enable a UE to receive up to 4L ‘with simultaneous DL reception with two different QCL TypeD RSs on single component carrier’. In this contribution we seek to discuss RAN1 design and system assumptions that will enable this type of operation.

# Discussion

## Overall split of DL data using mTRP assumption

Fig 2.1-1: Generic mTRP connection by the UE

The legacy rank 2 DL capability is often referred to as ‘polarization MIMO’. Rank 2 implies there are two independent data streams in the PDSCH and they are both related to the same source signal in a QCL-D sense. For convenience of future reference, we define:

Definition: A beam management reference signal (or BMRS) is a source signal in a QCL-D sense, for association with other signals like PDSCH and PUSCH.

A BMRS is either an SSB or a CSI-RS with repetition parameter ‘ON’. Other kinds of CSI-RS do not qualify as a BMRS. The SSB referred to here what is included in the normal SSB burst.

Typically, polarization MIMO is implemented as one layer per transmitted polarization to take advantage of spatial orthogonality in this configuration. Legacy functionality for this feature in FR2 also assumed single TRP operation. A straightforward extension of this concept is to assume that this feature is enabled by simultaneous connection by the UE to 2 TRPs, and each connection has DL polarization MIMO support. The BMRS from the two TRPs are assumed to be orthogonal in the code domain.

Proposal 1: For this feature, the system assumes that the UE connects to 2 TRPs and can support polarization MIMO in DL with each.

In a multi-TRP construct, each TRP has an associated TCI state that informs the UE of the associated BMRS. The BMRS referenced in this context does not differ from those assumed for the Rel-16 inter-band CA feature in FR2. Recall also that the UE can indicate to the network its ability to use either kind of BMRS via the Rel-16 IEs beamCorrespondenceSSB-based-r16 and beamCorrespondenceCSI-RS-based-r16.

Proposal 2: The UE uses Rel-16 IEs beamCorrespondenceSSB-based-r16 and beamCorrespondenceCSI-RS-based-r16 to convey to the network what QCL-D source signal it can support for the multi-chain Rx feature.

It is well understood that CSI-RS-based BMRS is a more capable than SSB for beam management. A significant advantage is that the CSI-RS signal can be configured as a 2-port signal, allowing the UE a much more complete view of the channel, and consequently a better shot at finding the optimal beam(s).

Proposal 3: For this feature, if the network can provide a CSI-RS-based QCL-D source signal, it configures the 2-port type of the same from each TRP.

Any change to the UL configuration is out of scope for this WI and is assumed to be legacy single TCI state rank1.

## Initial actions to enable mTRP operation

### Step 0 - Starting condition

The starting point is assumed to be a UE that is connected to a single TRP (‘anchor TRP’), i.e. a basic connected mode. In this configuration, the UE is aware of the optimal beam to use for the TRP. The anchor TRP uses a mutually agreeable choice of BMRS, depending on the UE’s capabilities.

### Step 1 – network configures UE for group reporting

Group reporting is the primary indicator for the UE that network has multiple TRPs associated with the same cell ID and are available to connect. The reporting metric is L1-RSRP. Dedicated measurement resources in the form of reference signals (RS) are confirmed for the UE by the network, and can be either SSB or CSI-RS, depending on network implementation. These measurement resources are typically different from BMRS.

For the sake of simplicity, we assume the 2 TRPs are visible to the UE. Each TRP transmits multiple RS, distinguishable by their unique ids. More than one of those may be visible to the UE from each TRP. For group-based reporting, the UE must arrange the set of visible RSs into pairs that contain one RS id from each TRP. The UE then reports the pairwise L1-RSRPs in the joint report.

The beam type used by the UE to measure the RS is up to the UE’s implementation. They could be rough or refined to some extent.

After the report, the NW has a good shortlist of mTRP RS candidates, but it still does not know which pair of RS will maximize capacity/t-put. For this level of visibility, the network relies on a second layer of enquiries, specifically channel state feedback, or CSF.

Step-1a

UE sees id 30 from TRP1

01

30

31

32

33

62

63

01

30

31

32

33

62

63

Step-1b

UE sees id 33 and 62 from TRP2

01

30

31

Step-1c

UE measures RSRP of ids 30, 33 and 62 using concurrently formable beams

01

30

31

32

33

62

63

32

33

62

63

01

30

31

32

33

62

63

Fig 2.2.2-1: Group-based reporting by the UE

### Step 2 – network requests channel state feedback (CSF)

The network may choose one or more pairs of visible measurement resources in step 1 from the UE’s joint report to pursue as candidates for a successful mTRP connection. It then assigns as many CSF opportunities as there are pairs to measure. The network can choose from either R15- or R17-based CSI measurement/report methods.

Rel-17

Rel-15

Two ports NZP-CSI-RS for CSI

CMR1

IMR1

IMR2

CMR2

CSI process 1

(CPU 1)

CSI process 2

(CPU 2)

Step-2

CSI meas/report from two TRPs for each candidate pair that network wants to pursue for the UE

CMR-1

CMR-2

(can be in different slots)

Fig 2.2.3-1: CSF after group-based reporting

R15: two CSI process units (CPUs) are configured for each pair of visible RS from the two TRPs. In each CPU, the UE is presented with both RS, but one as the CMR (channel meas. resource) and the other acts as an IMR (interference meas. resource). The IMR helps capture interference from the other TRP. For CPU1, both CMR and IMR are QCL’ed to a source signal from TRP1, and for CPU2, both RSs are QCL’ed to a source signal from TRP2.

- R17: One CPU is configured with two CMRs, each QCL’ed to a source signal from a unique TRP

The beam type used by the UE to measure the RSs is up to the UE’s implementation. They could be rough or refined to some extent, like the case in step 1.

After evaluating CSF associated with each RS pair, the network can assign the second active TCI-state, which completes the basic mTRP configuration of the UE.

The choice of Rel-15 or Rel-17 framework does not impact the final operation of the system and therefore left as network choice, or TE choice in requirement context.

### Step 3 – beam refinement for 2 active TCI states

When the UE is configured with 2 active TCI states, the UE pursues any further beam refinement with a 2-active TCI state constraint. i.e. the UE tracks and optimizes two sets of beams, one set pointed to one TRP and another to the second, with the condition that both sets of beams can coexist at the same time (concurrent beams). This constraint is new for FR2 UEs in RAN4.

This stage of further beam refinement is relevant to a UE if it uses rough beams for scoping out the RS pairs chosen during joint reporting, but it has higher gain-beams at its disposal for each direction. After refinement, periodic reporting of CSF parameters will ensure the network has an up-to-date picture of the channel. It is up to the network how MCS, etc are chosen.

### mTRP connection summary

For setting up an RF requirement on the UE using optimal beams in the direction of each TRP, it is necessary to guarantee the network steps outlined above. While this information may not explicitly be captured in the standard, a common understanding will help establish a meaningful final UE RF requirement.

Proposal 4: The RF requirement on the UE assumes the following sequence of events:

- starting condition: basic connection between the UE and a single TRP

- network configures UE for joint reporting with other TRPs visible to the UE

- network requests CSF based on RS pairs reported by the UE with ‘joint reporting’

- network configures second active TCI state based on CSF

- network continues to provide resources to refine the UE beams for a 2-active TCI state condition

- (UE is ready to be evaluated)

Variations of this sequence are possible under some special conditions, like an interchange of steps 3 and 4 when only one pair of TRPs is visible. Such a variation would not affect the UE in context of the RF requirement which is unlikely to use more than 2 TRPs.

## DCI – single or multiple?

In addition to the above sequence of events, another important choice is whether a single-DCI or a multi-DCI scheme is employed by the network. A high-level comparison is below:

| Single DCI | Multi-DCI |
| --- | --- |
| - Single transport block split across both TRPs. Allows true 4x4 demod because of single TB - common MCS, RB allocation - At the Tx side, each 2-port TRP is enabled for NCJT with up to 4L (non-coherent joint transmission). | - Separate transport blocks - Independent MCS, RB allocations for each TRP (i.e spatial multiplexing is not guaranteed) - ‘PDSCH’ is structured as 2 parallel PDSCH channels, each with up to rank 2 – each layer pair shows up as interference to the other. It is up to UE implementation whether it can do joint 4L demod. |

Multi-DCI can be seen as added complexity for reliability and can be studied in the future as an evolution direction. Because of different adaption and constraints, it is likely that the UE will have different throughput performance for the two cases. Single DCI should be treated as a baseline.

Proposal 5: The single DCI scheme is adopted as a baseline for setting the UE RF requirement.

## UE Assumptions

For PC3, a nominally multi-panel device, the baseline condition with DL throughput benefit is one-panel per TRP. It is however impractical to expect all deployment scenarios to provide widely separated locations of the TRPs that can connect jointly to the UE. It is therefore necessary to consider the case where the TRPs appear close enough together that the same panel must be used for both TRPs. Further discussion is needed on how these cases will be handled.

# Conclusions

Proposal 1: For this feature, the system assumes that the UE connects to 2 TRPs and can support polarization MIMO in DL with each.

Proposal 2: The UE uses Rel-16 IEs beamCorrespondenceSSB-based-r16 and beamCorrespondenceCSI-RS-based-r16 to convey to the network what QCL-D source signal it can support for the multi-chain Rx feature.

Proposal 3: For this feature, if the network can provide a CSI-RS-based QCL-D source signal, it configures the 2-port type of the same from each TRP.

Proposal 4: The RF requirement on the UE assumes the following sequence of events:

- starting condition: basic connection between the UE and a single TRP

- network configures UE for joint reporting with other TRPs visible to the UE

- network requests CSF based on RS pairs reported by the UE with ‘joint reporting’

- network configures second active TCI state based on CSF

- network continues to provide resources to refine the UE beams for a 2-active TCI state condition

- (UE is ready to be evaluated)

Proposal 5: The single DCI scheme is adopted as a baseline for setting the UE RF requirement.

# Reference

[1]	RP-221753, ‘Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception’, Qualcomm, RAN#96, Jun. 2022