3GPP RAN WG4 Meeting #104-e		R4-2212354

Online, August 15th – 26th, 2022

Agenda item:	9.4.3

Source:	Apple

Title:	Rel-17 Intra-band UL CA DC default location clarification

WI/SI:	NR_RF_FR2_req_enh2-Core

Release:	Rel-17

Document for:	Approval

# 1	Introduction

The Rel-17 intra-band UL CA DC reporting framework has mostly been concluded in RAN4 #101e meeting [1] and communicated to RAN2 via an LS [2] in the same meeting. Unlike the Rel-15 and Rel-16 DC reporting mechanisms where UE would directly signal its UL DC location to the network, the Rel-17 method is composed of a default UL DC location in conjunction with a frequency offset relative to the default DC location chosen by the UE which can be communicated to the network. The default UL DC location is defined as the “mathematical center of the UE bandwidth rounded to the subcarrier grid defined for the component carrier on which the DC is located”. If the mathematical center of the UE bandwidth lands on a frequency where there is no subcarrier grid defined, the subcarrier grid of the nearest lower frequency component carrier shall be extended to cover the frequency of the mathematical DC location. On the other hand, the UE bandwidth has been defined as the “frequencies between lower edge of lowest frequency component and upper edge of highest frequency component”. However, it remains unclear on the definition of “edge” of the component carrier which could be interpreted as the frequency at the edge sub-carrier, or the frequency at the edge sub-carrier boundary. The frequency difference between the two definitions would be half of the sub-carrier spacing. In the case where the two edge component carriers are with the same numerology, the default DC location would be the same between the two interpretations. However, if the two edge component carriers are with different numerologies, the resulted default DC location could be different between the two interpretations. In this contribution, we propose RAN4 to further clarify the definition for the “edge” of the component carrier and communicate to RAN2 via a separate LS to ensure there is no ambiguity between UE and gNB when calculating the default UL DC location.

# Discussion

Figure 2-1 conceptually illustrates the two possible interpretations for the “edge” of the component carriers where Case (a) is defined as the edge sub-carrier frequency and Case (b) is defined as the edge sub-carrier boundary frequency.

Figure 2-1 Two possible interpretations for the “edge” of the component carriers

Figure 2-2 shows that when the two edge component carriers are with the same numerology, the default DC location would be the same between the Case (a) and Case (b) interpretations. However, when the two edge component carriers are with different numerologies, the resulted default DC location before rounding to the sub-carrier index would be different between the two interpretations, as shown in Figure 2-3. In some cases the default DC location after rounding could be offset by one sub-carrier between the two interpretations.

Figure 2-2 UL CA with two edge component carriers of the same numerology

Figure 2-3 UL CA with two edge component carriers of different numerologies

Observation 1: In UL CA when the two edge component carriers are with the same numerology, the default DC location would be the same between the two “edge” frequency interpretations.

Observation 2: In UL CA when the two edge component carriers are with different numerologies, the default DC location would be different before rounding to the sub-carrier index between the two “edge” frequency interpretations. In some cases the default DC location after rounding could be offset by one sub-carrier.

To ensure that there is no ambiguity between UE and gNB when calculating the default UL DC location, it is proposed RAN4 to further clarify the definition for the “edge” frequency of the edge component carriers and communicate to RAN2 on RAN4’s determination.

Proposal 1: RAN4 to clarify which of the following definitions is intended for “edge” frequency of the edge component carriers:

Definition 1: Edge sub-carrier frequency

Definition 2: Edge sub-carrier boundary frequency

Proposal 2: Send an LS to RAN2 to clarify the definition of the “edge” frequency of the edge component carriers for default UL DC location calculation.

# 3	Conclusion

In this contribution, we propose RAN4 to further clarify the definition for the “edge” of the component carrier and communicate to RAN2 via a separate LS to ensure there is no ambiguity between UE and gNB when calculating the default UL DC location.

Observation 1: In UL CA when the two edge component carriers are with the same numerology, the default DC location would be the same between the two “edge” frequency interpretations.

Observation 2: In UL CA when the two edge component carriers are with different numerologies, the default DC location would be different before rounding to the sub-carrier index between the two “edge” frequency interpretations. In some cases the default DC location after rounding could be offset by one sub-carrier.

Proposal 1: RAN4 to clarify which of the following definitions is intended for “edge” frequency of the edge component carriers:

Definition 1: Edge sub-carrier frequency

Definition 2: Edge sub-carrier boundary frequency

Proposal 2: Send an LS to RAN2 to clarify the definition of the “edge” frequency of the edge component carriers for default UL DC location calculation.

# 4	References

R4-2119964 “WF on DC-Location”, Vivo, 3GPP TSG-RAN WG4 Meeting #101e, online, November 1st – 12th, 2021

R4-2119965 “LS on DC location for >2CC”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #101e, online, November 1st – 12th, 2021

# 5	Appendix

Title:	[draft] LS on intra-band UL CA DC default location clarification

Response to:

Release:	Release 17

Work Item:	NR_RF_FR2_req_enh2-Core

Source:	Apple

To:	RAN2

Cc:

Contact Person:

- Name:	James Wang

- E-mail Address:	 fucheng_wang@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

None

1. Overall Description:

The Rel-17 intra-band UL CA DC reporting framework has mostly been concluded in RAN4 #101e meeting and communicated to RAN2 via the LS R4-2119965. Unlike the Rel-15/Rel-16 DC reporting mechanisms where UE would directly signal its UL DC location to the network, the Rel-17 method is composed of a default UL DC location in conjunction with a frequency offset relative to the default DC location chosen by the UE which can be communicated to the network.

The default UL DC location is defined as the “mathematical center of the UE bandwidth rounded to the subcarrier grid defined for the component carrier on which the DC is located”. If the mathematical center of the UE bandwidth lands on a frequency where there is no subcarrier grid defined, the subcarrier grid of the nearest lower frequency component carrier shall be extended to cover the frequency of the mathematical DC location. On the other hand, the UE bandwidth has been defined as the “frequencies between lower edge of lowest frequency component and upper edge of highest frequency component”. However, it remains unclear on the definition of “edge” of the component carrier which could be interpreted as the frequency at the edge sub-carrier, or the frequency at the edge sub-carrier boundary, as illustrated in Case (a) and Case (b) respectively in the figure below.

As the two interpretations for the “edge” of the component carrier may result in different default DC location when the two edge component carriers are with different numerologies, it is necessary to unify the definition to ensure that there is no ambiguity between UE and gNB when calculating the default UL DC location.

RAN4 has discussed and concluded that the “edge” of the component carrier shall be defined as the “edge sub-carrier frequency” which is the Case (a) in the figure above.

2. Actions:

To: 3GPP TSG RAN WG2

RAN4 respectfully asks RAN2 to take RAN4’s clarification on default DC location into consideration when developing the Rel-17 intra-band UL CA DC location signaling.

3. Date of Next TSG-RAN WG4 Meetings:

- 3GPP RAN4 #104bis-e 					Oct 10th – 19th, 2022		        		   Online

- 3GPP RAN4 #105							Nov 14th – 18th, 2022					   Canada