3GPP RAN WG4 Meeting #104-e		R4-2212370

# Online, , 15th Aug 2022 - 26th Aug 2022

Agenda item:	11.7.2

Source:	Apple

Title:	Discussion on minimum UE EIRP for UL 256QAM

Release:	Rel-18

Work Item:	NR_RF_FR2_req_Ph3-Core

Document for:	Decision

# 1  Introduction

A new work item for Rel-18 was agreed during RAN#96 [1] focusing on RF enhancements for FR2. One of the enhancement topics is the introduction of UL 256QAM. The focus lies on PC1, PC2 and PC5 with PC3 being second priority. In the following the minimum UE EIRP for EVM is discussed and proposals are made for 256QAM.

# 2  Discussion

The specification TS 38.101-2 defines the EVM budget and the minimum UE EIRP in clause 6.4.2.1. The modulations have specifically defined UE EIRP thresholds which are dependent on the UE power class. The UE EIRP threshold for QPSK, 16QAM and 64QAM feature a -1dB/dB relation which is being used in the following to derive the minimum UE EIRP for 256QAM. This contribution provides values for an EVM budget of 3.5% (in case the previous approach of reusing FR1 EVM budget is followed) and for 4.0% (as proposed in [2]).

The Figures 1 to 3 display the EVM to EIRP relation for the different modulations with respect to power classes. The minimum EIRP for 256QAM is calculated by applying the -1dB/dB relationship and using the UE EIRP value for 64QAM as reference. With this setup the following values are obtained:

- UE EIRP for PC1: 18.2dBm

- UE EIRP for PC2, PC3, PC4: 1.2dBm

- UE EIRP for PC5: 8.2dBm

| Figure 1: Minimum UE EIRP for PC1 | Figure 2: Minimum UE EIRP for PC2, PC3 and PC4 |
| --- | --- |
| Figure 3: Minimum UE EIRP for PC5 |  |

The applied linear analysis is a simple method allowing to derive the minimum UE EIRP requirements for different modulations when there are no further limitations involved. However, for 256QAM there exists a considerable thermal noise and phase noise contribution which limits the minimum UE EIRP. According to our analysis for PC3 devices there needs to be a correction factor of 1.2dB to account for thermal and phase noise. The proposal assumes that the correction factor can be translated one to one for other device classes. The values are rounded to the next higher 0.5dB and we propose to set the values as listed in the proposal below.

Proposal 1: We propose to set the minimum UE EIRP for 256QAM as listed below. The values are derived for an EVM budget of 3.5% for 256QAM.

- UE EIRP for PC1: 19.5dBm

- UE EIRP for PC2, PC3, PC4: 2.5dBm

- UE EIRP for PC5: 9.5dBm

Proposal 2: In case the proposal made on UE EVM budget in [2] is accepted and the UE EVM budget is set to 4.0% the minimum UE EIRP for 256QAM would be proposed as listed below:

- UE EIRP for PC1: 18.5dBm

- UE EIRP for PC2, PC3, PC4: 1.5dBm

- UE EIRP for PC5: 8.5dBm

It is proposed to use a similar scaling strategy as used for FR2-2 where the minimum UE EIRP is scaled with bandwidth. Thermal noise contribution provides a more severe issue for high order modulations such as 256QAM due to the small EVM budget. With to the tight budget and thermal noise it was determined that the scaling should start with 200MHz and be applied for higher bandwidth accordingly. The base value is from Proposal 1. In case of UE EVM budget in [2] is accepted the base value should be from Proposal 2. Additionally, PTRS should be configured.

Table 6.4.2.1-3x: Parameters for Error Vector Magnitude for power class 3 in FR2-1

|  |  | Level |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Parameter | Unit | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| UE EIRP for UL 256 QAM | dBm | 2.5 | 2.5 | 5.5 | 8.5 |
| Operating conditions | Normal Conditions |  |  |  |  |
| NOTE 1: PTRS is configured for 256 QAM |  |  |  |  |  |

Proposal 3: Introduce minimum UE EIRP scaling for 256QAM according to Table 6.4.2.1-3x since thermal noise provides a stronger issue for high order modulations such as 256QAM due to the small EVM budget. The base value is from Proposal 1. In case of UE EVM budget in [2] is accepted the base value should be taken from Proposal 2.

# 3  Conclusions

###### This contribution focuses on relaxation of minimum output power for 256QAM and makes the following proposal:

Proposal 1: We propose to set the minimum UE EIRP for 256QAM as listed below. The values are derived for an EVM budget of 3.5% for 256QAM.

- UE EIRP for PC1: 19.5dBm

- UE EIRP for PC2, PC3, PC4: 2.5dBm

- UE EIRP for PC5: 9.5dBm

Proposal 2: In case the proposal made on UE EVM budget in [2] is accepted and the UE EVM budget is set to 4.0% the minimum UE EIRP for 256QAM would be proposed as listed below:

- UE EIRP for PC1: 18.5dBm

- UE EIRP for PC2, PC3, PC4: 1.5dBm

- UE EIRP for PC5: 8.5dBm

Proposal 3: Introduce minimum UE EIRP scaling for 256QAM according to Table 6.4.2.1-3x since thermal noise provides a stronger issue for high order modulations such as 256QAM due to the small EVM budget. The base value is from Proposal 1. In case of UE EVM budget in R4-2212371 is accepted the base value should be taken from Proposal 2.

Table 6.4.2.1-3x: Parameters for Error Vector Magnitude for power class 3 in FR2-1

|  |  | Level |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Parameter | Unit | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| UE EIRP for UL 256 QAM | dBm | 2.5 | 2.5 | 5.5 | 8.5 |
| Operating conditions | Normal Conditions |  |  |  |  |
| NOTE 1: PTRS is configured for 256 QAM |  |  |  |  |  |

# 4  References

- RP-221862, “NR RF requirements enhancement for frequency range 2 (FR2), Phase 3”, Nokia, Xiaomi, 3GPP TSG RAN Meeting #96

- R4-2212371, “Discussion on phase noise and EVM budget for 256QAM”, Apple, 3GPP RAN WG4 Meeting #104-e