3GPP RAN WG4 Meeting #104-e		R4-2212374

Electronic Meeting, August 15-26, 2022

Agenda item:	9.2.2.2

Source:	Apple

Title:	Remaining TRP and TRS EN-DC measurement aspects

Work Item:	NR_FR1_TRP_TRS-Core

Document for:	Discussion

# 1	Introduction

A new work item to define the OTA test methodology and TRP/TRS requirements for UEs operating in NR FR1 stand-alone and EN-DC within FR1 configurations was approved during the RAN #91 [1] and further revised during RAN #92 [2].  During the RAN4 #103 meeting, the following agreements related to EN-DC measurement aspects were achieved:

Sub-topic 2-1 EN-DC band combination Issue 2-1-2: How to treat UE not support example band Agreements: The EN-DC TRP/TRS test procedure shall include a procedure to allow the OEM to declare which configuration it shall use for the test (under the condition that the declared configuration matches the EN-DC band combination principle). Decision tree should be defined to ensure that the declaration procedure is clear/ transparent to generate a sole configuration and avoid the different targeted EN-DC combinations for one NR band in different test labs. Sub-topic 2-2 Test and requirement applicability Agreements: Fast SAR scan-based method can be considered as an alternative to verify TAS OFF on UE. This verification procedure is an optional action for test labs but not mandatary. The judgment criteria is FFS. RAN4 should further study the detailed test procedure for fast SAR-scan based method.

This contribution provides our views on remaining issues with the EN-DC configuration for TRP/TRS testing.

# 2	Discussion

As several companies commented during the discussion of the EN-DC configuration topic [5], it is not guaranteed that all UEs, which support one of the priority NR bands (n28, n41, n78, n79) and also support EN-DC, shall also support the corresponding example EN-DC band combinations.  On the other hand, the general principle on EN-DC band combination selection still needs to be followed.

Observation 1:	Only the OEM maintains the complete list of all supported EN-DC band combinations by the DUT, which are then signaled as UE capabilities to the network. In the context of TRP/TRS testing in EN-DC mode, the test procedure should accommodate an approach based on OEM declaration of the EN-DC configuration to be tested.

During the RAN4 #103 meeting the group agreed to define a decision tree to ensure a clear and transparent declaration process.  We would like to propose the following approach:

Figure 1: Proposed decision tree to select the EN-DC band combination for TRP/TRS testing

Since the decision tree refers to the table of example EN-DC band combinations in TS38.161, it will be necessary to continue to add an example EN-DC band combination for each new band that is added to the scope of TRP/TRS requirements.  Currently we have 4 example combinations corresponding to the four bands being treated as first priority in the work item (n28, n41, n78, n79).

Proposal 1:	It is proposed to define the decision tree to select the EN-DC band combination for TRP/TRS testing as outlined in Figure 1.

Proposal 2:	As a working principle in future TRP/TRS efforts, it will be necessary to continue to add an example EN-DC band combination for each new band that is added to the scope of TRP/TRS requirements.

# 3	Conclusions

This contribution provides our views on the topic of EN-DC test configurations for the TRP/TRS test and makes the following observation and proposals:

Observation 1:	Only the OEM maintains the complete list of all supported EN-DC band combinations by the DUT, which are then signaled as UE capabilities to the network. In the context of TRP/TRS testing in EN-DC mode, the test procedure should accommodate an approach based on OEM declaration of the EN-DC configuration to be tested.

Proposal 1:	It is proposed to define the decision tree to select the EN-DC band combination for TRP/TRS testing as outlined in Figure 1.

Proposal 2:	As a working principle in future TRP/TRS efforts, it will be necessary to continue to add an example EN-DC band combination for each new band that is added to the scope of TRP/TRS requirements.

# 4	References

RP-210807, "Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC)," vivo, OPPO, CMCC, CAICT, Rohde & Schwarz, 3GPP RAN #91, March 2021

RP-211158, "Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC)," vivo, OPPO, CMCC, 3GPP RAN #92, June 2021

R4-2120689, "WF on FR1 TRP TRS," vivo, 3GPP RAN4 #101, November 2021

R4-2203076, "WF on FR1 TRP TRS," vivo, 3GPP RAN4 #101bis, January 2022

R4-2203097, "Email discussion summary for [101-bis-e][325] FR1_TRP_TRS_Part1," Moderator (vivo), 3GPP RAN4 #101bis

R4-2207326, "WF on FR1 TRP TRS," vivo, 3GPP RAN4 #102, February 2022

R4-2210677, “WF on FR1 TRP TRS,” vivo, 3GPP RAN4 #103, May 2022