3GPP TSG RAN WG4 Meeting #104-e	                          R4-2212375

Electronic Meeting, 15th  - 26th August  2022

Agenda item:	9.2.2.3

Source:	Apple Hungary Kft.

Title:                         Remaining issues with multiple antenna test methodologies

Document for:	Discussion and Decision

# 1	Introduction

As previously defined  on [1], there are competing method proposals to address TxD, and a call for further clarifications on test methodology for TAS ON:

Issue 1-1-2: Discussion on Test Method A, test mode v.s. TPMI

Agreement:

- Focus on Method B to develop TxD TRP test method, and come back to test mode discussion only if it is the single solution for TxD TRP testing.

Issue 1-1-3/4: Views and Proposals on Test Method B

Agreement:

- Further discuss the solution which can stably trigger TxD on UE.

- Study the feasibility of UE configured with one antenna port for the TRP testing.

Issue 1-1-5: Test methodology for TxD with four transmit antennas

Agreement:

- Focus on the case of 2 antenna transmitting simultaneously.

Issue 1-1-6: Other considerations on the test methodology

Way forward:

Encourage TE vender’s feedback on whether TxD mode can be maintained by equipment during testing

Sub-topic 1-2: Test methodology for TAS ON

Issue 1-2-1: Factor 1 for Tx antenna switch ON

Way forward:

- Encourage interested companies to verify the sensibility of the proposed test solution.

Issue 1-2-2: Factor 2 for Tx antenna switch ON

Agreement:

- The near body sensor working mechanism on transmit antenna switch can be verified using hand phantom or hand & head phantom during the TRP measurement. And these test scenarios can be combined with the test solution of Factor 1, i.e. downlink Rx signal.

# 2	Discussion

2.a  Discussion on TxD antenna systems

Considering the phase relationship on multiple antenna systems transmitting simultaneously in simple example of two dipoles placed apart /2, Figure 1a the dipoles are in phase =0, Figure 1b they are out of phase =.

- (b)

Figure 1, Top view of radiated performance of two dipoles placed at /2, (a) the dipoles are in phase =0, (b) the dipoles are out of phase =.

In this simple controlled scenario with antennas place at /2 apart. Quantitatively, the amount of energy that the field  carries, which is proportional to the square of electric field average in time. Varies with the phase applied to each antenna element. When antennas are in phase, both radiators contribute equally to double the electric field in the left and right direction. Thus the radiation performance is four times in comparison with a single radiator.

However, in the up/down direction since the in phase radiators are /2 apart, the resulting electric field net is equal to zero.

The opposite effect happens on Figure 1 (b), considering the same distance between antennas (/2 apart) however with antennas out of phase by =.

With this simple example is possible to consider the following implications in real applications on TxD antenna systems:

- Different UEs will have different spacing between antennas

- Different UEs can have any arbitrarily combination of signal phase applied to each antenna

- Since the overall effect of constructed or destructive interference is also related to , even controlling the phase of the signal feeding each antenna, the effect will vary over frequency of operation

- The effect of constructed or destructed interference varies over far-field 3D region, with static antenna placement and phase signal

2b. Discussion on TAS ON antenna systems

Devices equipped with TAS will likely to rely on multiple sensor imputes and sophisticated algorithms to optimise the antenna system radiated performance in real environment conditions. The antenna switching can also results in a configuration of multiple antennas as an array.

In real conditions the antenna system optimization is done to reduce the path loss over the air between handset and base station, therefore improving call quality, minimizing drop calls and improving talk time since it will transmit less power over the up-link.

In a controlled test environment i.e.: anechoic chamber, such real environment radiated performance optimization efforts might not be achieved due the test environment limitations.

Considering an UE that has the TAS algorithm based on down-link signal event to trigger a decision, such UE will produce different OTA measurement results depending on the test setup being utilized.

Since there are no strict guidelines across system vendors regarding the placement on the link antenna on their setups, most likely such chamber design feature won’t be reproduced among test system competitors. The Figure 1 exemplifies three (out of many) options for link antenna placement on the system.

Figure 1, Simplified CATR system

The effect of different link antenna positions in different systems can be even amplified with phantoms. Such uncertainty can result in different OTA measurement results with same UE in different labs, this measurement uncertainty can’t be determine and might exceed the defined MU for such systems.

# 3	Observations

Observation 1:

Considering the option of performing multiple antenna test methodologies in open loop, without gNB feedback to the UE. Without prior knowledge of the UE architecture and independent Tx systems implementation, one can’t predict the radiated performance of a non-signalling Tx system with simultaneous operation when evaluating in a lab environment. The multiple antenna system can be designed to optimize radiated performance characteristics e.g.: real environment with anisotropic channel conditions and realistic user cases. An arbitrarily lab test condition might result in improper or unknown phase relationship between transmitters.

Observation 2:

Considering a simple application where the UE samples the down-link SNR or RSSI to predict the best multiple antenna configuration. Measurement results taken on a controlled environment such as anechoic chamber will vary from lab to lab, due the fact that there’s no consensus where the system link antenna is placed. Some system have one or more link antennas placed on the chamber floor, some systems might have it placed on the turn-table, etc. Therefore, depending on the test environment different multiple antenna scheme will be selected.

Observation 3:

The Transmit Precoding Matrix Index (TPMI) besides Rank Indicator (RI), and correspondent Mutual Information (MI) are required to schedule PUSCH transmission. Is unlikely that an UE designed to have optimized radiated performance on real network environment will reproduce such performance on open loop test environment, i.e.: anechoic chamber. Adopting TPMI as an option to control the Tx system, might require a adaptation from system vendors, e.g.: an optimal TPMI search for each device orientation within the test environment.

Observation 4:

An attempt to test TxD/TAS with all transmitters On shall be preceded by a test campaign, stressing the differences on TxD/TAS system implementations, correlating with manufacturers expectations of radiated performance in these conditions. Test environment limitations also need to be determined.

Observation 5:

Implementing TxD/TAS in a system where a multiplicity of antennas (more than 2) can be selected, might have dedicated switching mechanisms that goes beyond signalling or simple test commands that can be reproduced by test labs.

# 4	Proposals

This paper contributes to the Tx diversity and TAS ON discussion and makes the following observations and proposals:

Proposal 1:

While studies are conducted to guarantee the optimal UE radiated performance at the test environment are done. Evaluate multiple antenna system measuring TRP per antenna under test mode separately and TAS Off.

Proposal 2:

RAN4 to avoid elaborating a test where dedicated  TxD/TAS features algorithms and triggers needs to be declared to labs in order to proper evaluate UE radiated performance.

Proposal 3:

In case of still pursuing the test with multiple Tx antennas, RAN4 shall propose a comprehensive test campaign to stress different TxD/TAS system implementation correlating results with manufacturers real environment radiated performance expectations and test vendors limitations, such as link antenna placement variations and an optimal TPMI search for each device orientation within the test environment.

# 5	References

- [1]	R4-2210679, WF on FR1 TRP TRS for UE with multi-antenna