3GPP TSG-RAN WG4 Meeting # 104-e															R4-2212399

Electronic Meeting, 15– 26 August 2022

Agenda item:			9.11.6.1

Source:	MediaTek Inc.

Title:	Discussion on RRM test list for NR NTN

Document for:	Discussion

# Introduction

In this contribution, our view on overall RRM tests list is provided.

# Dsicussion on RRM Test Cases

First, the terminology of GSO and GEO, and NGSO and LEO are widely been used in the TS 38.133, thus it would need to clarify the GSO tests is appliable for both GSO and GEO.

Proposal 1: Clarify the test cases for GSO are also applicable for GEO.

Proposal 2: Clarify the test cases for NGSO are applicable for LEO and MEO but not GEO.

Because NGSO/LEO is unique scenario comparing to TN systems, thus at least one TC under each requirement group should be tested under NGSP/LEO.

Proposal 3: At least one TC under each requirement group is defined for NGSO/LEO.

For cell reselection, since the time-based/location based cell re-selection is optional, thus at least one TC for LEO for cell reselection would be also needed.

Proposal 4: For cell reselection to FR1 intra/inter-frequency NR cell, TCs are defined for both GSO and NGSO.

For RLM, total 16 TCs could be too many, it suggests to pick either non-DRX or DRX of each sub-test to test.

Proposal 5: For RLM/BFD, select either non-DRX or DRX of each sub-test to test.

For intra-frequency measurement in NGSO, since the two SMTCs and Doppler shift are the major differentiation of LEO, thus they should be considered in the TC. Two Doppler shifts can be set, one for the serving cell and one for the neighbouring cell, wherein the Doppler shifts are fixed values. RAN4 to further discuss whether cell’s timing drifting, corresponding to the fixed Doppler, can be modelled in the tests. Besides, two SMTCs can be configured as overlapped to test the corresponding behaviour.

Proposal 6: For inter-frequency measurement TC, two fixed Doppler shifts are defined, one for the serving cell and one for the neighbouring cell. RAN4 to further discuss whether cell’s timing drifting, corresponding to the fixed Doppler, can be modelled. Two overlapped SMTCs are configured.

For inter-frequency measurement, since the 2 MG is optional UE capability, it will need to introduce TC with one MG.

Proposal 7: For inter-frequency measurement, introduce TC for the case of one MG.

The overall RRM test list is provided below for discussion. The preference and the minimum set of TC is also indicated as “need” in the Table.

| Group of requirements | Test cases | Sub-test | Title of test case | Scenario (Under GSO or NGSO) | Comment (Need or not) |
| --- | --- | --- | --- | --- | --- |
| Cell reselection for satellite access | Intra-frequency | N.A | Cell reselection to FR1 intra-frequency NR cell | GSO, NGSO | Need |
|  |  | Time based | Time-based cell reselection to FR1 intra-frequency NR cell | NGSO | optional |
|  |  |  | Time-based cell reselection to FR1 intra-frequency NR cell for UE configured with [capability for enhanced requirements] | NGSO | optional |
|  |  | Location based | Location-based cell reselection to FR1 intra-frequency NR cell | NGSO | optional |
|  |  |  | Location-based cell reselection to FR1 intra-frequency NR cell for UE configured with [capability for enhanced requirements] | NGSO | optional |
|  | Inter-frequency | N.A | Cell reselection to FR1 inter-frequency NR cell | GSO and NGSO | Need |
|  |  | Time based | Time-based cell reselection to FR1 inter-frequency NR cell | NGSO | optional |
|  |  |  | Time-based cell reselection to FR1 inter-frequency NR cell for UE configured with [capability for enhanced requirements] | NGSO | optional |
|  |  | Location based | Location-based cell reselection to FR1 inter-frequency NR cell | NGSO | optional |
|  |  |  | Location-based cell reselection to FR1 inter-frequency NR cell for UE configured with [capability for enhanced requirements] | NGSO | optional |
| HO for satellite access | NR SAN HO | Intra-frequency HO | Intra-frequency SAN Handover from FR1 to FR1 | GSO and NGSO | Need |
|  |  | Inter-frequency HO | Inter-frequency SAN Handover from FR1 to FR1 | No strong view | Need |
|  | NR SAN CHO | Time based CHO | Intra-frequency SAN time-based Conditional Handover from FR1 to FR1 | NGSO | optional |
|  |  |  | Inter-frequency SAN time-based Conditional Handover from FR1 to FR1 | NGSO | optional |
|  |  | Distance based CHO | Intra-frequency SAN distance-based Conditional Handover from FR1 to FR1 | NGSO | optional |
|  |  |  | Inter-frequency SAN distance-based Conditional Handover from FR1 to FR1 | NGSO | optional |
| RRC Re-establishment for satellite access | Intra-frequency | N.A | Intra-frequency RRC Re-establishment in FR1 |  | Not necessary because no change in core spec. |
|  | Inter-frequency | N.A | Inter-frequency RRC Re-establishment in FR1 |  | Not necessary because no change in core spec. |
| Random access for satellite access | Contention based | N.A | 4-step RA type contention based random access test in FR1 |  | Not necessary because no change in core spec. |
|  | Non-contention based | N.A | 4-step RA type non-contention based random access test in FR1 |  | Not necessary because no change in core spec. |
| RRC Connection Release with Redirection for satellite access | N.A | N.A | Redirection from NR in FR1 to NR in FR1 |  | Not necessary because no change in core spec. |
| UE transmit timing for satellite access | UE transmit timing accuracy | N.A | NTN UE transmit timing test | GSO and NGSO | Need |
|  | UE timing advance adjustment accuracy | N.A | NTN UE timing advance adjustment accuracy test | GSO and NGSO | Need |
| RLM for satellite access | SSB based RLM RS | In-sync | Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode |  | Down-select among RLM tests |
|  |  |  | Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode |  | Down-select among RLM tests |
|  |  | Out-of-sync | Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode |  | Down-select among RLM tests |
|  |  |  | Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode |  | Down-select among RLM tests |
|  | CSI-RS based RLM RS | In-sync | Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM RS in non-DRX mode |  | Down-select among RLM tests |
|  |  |  | Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM RS in DRX mode |  | Down-select among RLM tests |
|  |  | Out-of-sync | Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM RS in non-DRX mode |  | Down-select among RLM tests |
|  |  |  | Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM RS in DRX mode |  | Down-select among RLM tests |
| Beam Failure Detection and Link Recovery for satellite access | SSB-based BFD and LR | N.A | Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode |  | Down-select among BFD tests |
|  |  |  | Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode |  | Down-select among BFD tests |
|  | CSI-RS-based BFD and LR | N.A | Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode |  | Down-select among BFD tests |
|  |  |  | Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode |  | Down-select among BFD tests |
|  | CSI-RS-based BFD and SSB-based LR | N.A | Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode |  | Down-select among BFD tests |
|  |  |  | Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode |  | Down-select among BFD tests |
| Active BWP switch delay for satellite access | DCI-based | N.A | NR FR1 DCI-based DL active BWP switch with non-DRX |  | Not necessary because no change in core spec. |
|  | RRC-based | N.A | NR FR1 RRC-based DL active BWP switch with non-DRX |  | Not necessary because no change in core spec. |
| UE-specific CBW change for satellite access | N.A | N.A | UE specific CBW change on PCell in FR1 in non-DRX |  | Not necessary because no change in core spec. |
| Pathloss reference signal switching delay for satellite access | N.A | N.A | MAC-CE based pathloss reference signal switch delay |  | Not necessary because no change in core spec. |
| Intra-frequency measurement delay for satellite access | Without gap | N.A | Event triggered reporting tests without gap under non-DRX | NGSO | Need |
|  |  | N.A | Event triggered reporting tests without gap under DRX |  | Need |
|  |  | N.A | Event triggered reporting tests without gap under non-DRX with SSB index reading |  | Need |
|  | With gap | FNO case | Event triggered reporting tests with multiple concurrent gaps under non-DRX |  |  |
|  |  |  | Event triggered reporting tests with multiple concurrent gaps under DRX |  |  |
|  |  |  | Event triggered reporting tests with multiple concurrent gaps under non-DRX with SSB index reading |  |  |
|  |  | PPO case | Event triggered reporting tests with multiple concurrent gaps under non-DRX |  |  |
| Inter-frequency measurement delay for satellite access | Without gap | N.A | Event triggered reporting tests for FR1 without gap under non-DRX |  | Optional |
|  |  |  | Event triggered reporting tests for FR1 without gap under DRX |  | Optional |
|  | With gap | FNO case | Event triggered reporting tests for FR1 without SSB time index detection under non-DRX |  | Need |
|  |  |  | Event triggered reporting tests for FR1 without SSB time index detection under DRX |  | Need |
|  |  |  | Event triggered reporting tests for FR1 with SSB time index detection under non-DRX |  | Need |
|  |  |  | Event triggered reporting tests for FR1 with SSB time index detection under DRX |  | Need |
|  |  | PPO case | Event triggered reporting tests for FR1 without SSB time index detection under non-DRX |  | Need |
|  |  | One MG case | Event triggered reporting tests for FR1 without SSB time index detection under non-DRX | NGSO | Need |
| L1-RSRP measurement delay for satellite access | SSB based | N.A | SSB based L1-RSRP measurement when DRX is not used |  | Need |
|  |  |  | SSB based L1-RSRP measurement when DRX is used |  | Need |
|  | CSI-RS based | N.A | CSI-RS based L1-RSRP measurement when DRX is not used |  |  |
|  |  |  | CSI-RS based L1-RSRP measurement when DRX is used |  |  |
| RRM measurement accuracy for satellite access | SS-RSRP | Intra-frequency | Intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell |  | Need |
|  |  | Inter-frequency | Inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell |  | Need |
|  | SS-RSRQ | Intra-frequency | Intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell |  |  |
|  |  | Inter-frequency | Inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell |  |  |
|  | SS-SINR | Intra-frequency | Intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell |  | Optional |
|  |  | Inter-frequency | Inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell |  | Optional |
|  | L1-RSRP | SSB-based | SSB based L1-RSRP measurement |  | Need |
|  |  | CSI-RS based | CSI-RS based L1-RSRP measurement on resource set with repetition off |  |  |

# Summary

In this contribution, the overview on the overall RRM test list is provided and the minimum set of TC is also indicated. And the proposals are summarized below:

Proposal 1: Clarify the test cases for GSO are also applicable for GEO.

Proposal 2: Clarify the test cases for NGSO are applicable for LEO and MEO but not GEO.

Proposal 3: At least one TC under each requirement group is defined for NGSO/LEO.

Proposal 4: For cell reselection to FR1 intra/inter-frequency NR cell, TCs are defined for both GSO and NGSO.

Proposal 5: For RLM/BFD, select either non-DRX or DRX of each sub-test to test.

Proposal 6: For inter-frequency measurement TC, two fixed Doppler shifts are defined, one for the serving cell and one for the neighbouring cell. RAN4 to further discuss whether cell’s timing drifting, corresponding to the fixed Doppler, can be modelled. Two overlapped SMTCs are configured.

Proposal 7: For inter-frequency measurement, introduce TC for the case of one MG.