3GPP TSG-RAN4 Meeting #104-e	R4-2212411

# Electronic Meeting,  15th Aug. - 26th Aug., 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2470 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Maintenance CR for Rel-17 SL on 38.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - There is a typo in section 12.5.2. SD-DRX should be SL-DRX. - According to the agreement in RAN4#101e (WF R4-2110315), interruptions are allowed on SL due to Uu DRX. In our understanding, the agreement is not only applicable when SL is in non DRX but also when SL is in DRX. In addition, interruptions are allowed on WAN due to SL-DRX no matter Uu is in DRX or not. It is reasonable to align these two requirements. But in current spec, interruptions are only allowed on SL when SL is in non DRX due to Uu DRX. - The interruption requirements on the SL due to BWP switch in FDM based intra-band concurrent SL & Uu operation are also applicable to TDM based intra-band concurrent SL & Uu operation. During BWP switch, if the old and new UL BWP are both different from SL BWP, then UE should switch from old UL BWP to SL Tx at first and then switch to new UL BWP. BWP switch operation (e.g. DCI/RRC based BWP switch) would also be impacted due to SL Tx during BWP switch delay for TDM based intra-band concurrent operation. - A wrong clause is cited in section 12.7.7 (Interruptions to WAN due to SyncRef UE detection and/or Sensing during SL DRX off duration). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct SD-DRX to SL-DRX - Delete the sentence “When PCell is DRX and V2X is in SL-DRX, no interruption is allowed.” and do not limit the interruption on SL to the case that SL is in non DRX. - Make it clear that the interruption requirements on the SL due to BWP switch is also applicable to TDM based intra-band concurrent operation. Allow UE to drop SL transmission during BWP switch in TDM based intra-band concurrent operation. - Correct the cited clause to 12.7.4 (Interruptions to WAN at transitions between active and non-active during SL-DRX). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - It will be miss-leading. - The interruption requirement on SL due to Uu DRX is not aligned with agreement and the interruption requirement on WAN due to SL-DRX. - The interruption requirements on the SL due to BWP switch in TDM based intra-band concurrent SL & Uu operation are missing. The impact on BWP switch operation (e.g. DCI/RRC based BWP switch) due to SL Tx is not considered in TDM based intra-band concurrent operation. - The cited clause is not right. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.5.2,12.7.5,12.7.6,12.7.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of change 1>>

12.5.2	SL-RSRP measurements

The UE physical layer shall be capable of performing the L1 SL-RSRP measurements on the carrier operating V2X sidelink communication for determining the subset of resources to be excluded in PSSCH resource selection in sidelink transmission mode 2. The L1 SL-RSRP measurement period corresponds to one slot and the measurement shall meet the L1 SL-RSRP measurement accuracy requirement in Clause 10. After resource (re-)selection procedure, re-evaluation is performed on the reserved resources by L1 SL-RSRP measurements before transmission of SCI with reservation when the conditions specified in TS 38.214[26] are satisfied.

When the pre-emption mechanism is enabled for the resource pool that UE is monitoring and selecting resource from, after UE selects from the resource not excluded based on L1 SL-RSRP measurement procedure, the UE shall be capable of triggering reselection of already signalled resource(s) as a resource reservation when the conditions specified in TS38.214[26] are satisfied.

When partial sensing mechanism is enabled for the resource pool that UE is monitoring and selecting resource from, the UE shall be capable of performing the L1 SL-RSRP measurements on the sensing periods specified in TS38.214[26]. When SL-DRX is enabled, the UE shall be capable of performing the L1 SL-RSRP measurements and select resource during SL-DRX active time as specified in TS38.214[26].

<< End of change 1>>

<< Start of change 2>>

12.7.5	Interruptions to V2X sidelink at transitions between active and non-active during DRX

Interruption on V2X sidelink if configured due to PCell transitions between active and non-active during DRX are allowed with up to 1% probability of missed ACK/NACK when the configured DRX cycle is less than 640 ms, and 0.625% probability of missed ACK/NACK is allowed when the configured DRX cycle is 640 ms or longer. It is only applied when HARQ process on V2X sidelink is supported. Each interruption shall not exceed X slot as defined in table 12.7.5-1.

Table 12.7.5-1: Interruption length X at transition between active and non-active during DRX

|  | NR V2X Slot | Interruption length X (slots) |  |
| --- | --- | --- | --- |
|  | length (ms) | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 1 | 2 |
| 2 | 0.25 | 3 |  |

<< End of change 2>>

<< Start of change 3>>

12.7.6	Interruptions to V2X sidelink due to Active BWP switching Requirement

This clause contains the requirements related to the interruptions on the V2X sidelink due to BWP switch in intra-band concurrent V2X operation.

The requirements in clause 8.2.2.2.5 shall apply. The interrupted X slot is defined in Table 12.7.6-1.

Table 12.7.6-1: Interruption length X

|  | NR V2X Slot | Interruption length X (slots) |
| --- | --- | --- |
|  | length (ms) |  |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |
| 2 | 0.25 | 3 |

Note: UE can drop SL transmission during active BWP switching in TDM based intra-band concurrent opreration.

<< End of change 3>>

<< Start of change 4>>

12.7.7	Interruptions to WAN due to SyncRef UE detection and/or Sensing during SL DRX off duration

This sub-clause contains the requirements related to the interruptions on the PCell/serving cell due to SyncRef UE detection and/or Sensing during SL DRX off duration.

The requirements in clause 12.7.4 shall apply.

<< End of change 4>>